3GPP TSG-RAN WG4 Meeting #102-e                              								 R4-2204736

Electronic Meeting, 21 February– 3 March, 2022

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.717-11-11: Update MSD analysis of DC_(n)3AA

Agenda Item:	9.14.2

Document for:	Approval

# Introduction

Due to small duplex distance in n3, the need to MSD analysis of DC_(n)3AA  combination was raised during RAN WG4 Meeting # 101-e [1].

# Discussion

For DC_(n)3AA only one Tx is enabled at a time (NR or LTE). Two sources of Tx noise could interfere with the n3 Rx band. First source is the Out-of-Band (OOB) emission of the n3 Tx and the second source corresponds the IMD products related to the Tx signal and it image.

Table 1 contains the CBW combinations which were requested for DC_(n)3AA [2]:

Table 1 Bandwidth combination set for DC_(n)3AA

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENDC configuration | Uplink ENDC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | CBW for E-UTRA carrier (MHz) | CBW for NR carrier (MHz) | CBW for E-UTRA carrier (MHz) |  |  |
| DC_(n)3AA | DC_(n)3AA | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30 |  | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 |  |  |

The largest Rx CBW combination happens with a 20MHz LTE Tx/RX at B3 and a 30MHz NR Tx/RX at n3. Table 2 contains the IMD and harmonic products of such band combination with an active NR with an LCRB=50 RBs. IMD3 and IMD5 products may fall on n3 DL band [1805 1880] MHz which are highlighted in red.

Table 2 Harmonics and IMD products

| UE UL carriers | fx_low_image | fx_high_image | fy_low_signal | fy_high_signal |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1755.692 | 1764.692 | 1775.307 | 1784.308 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3511.384 | 3529.384 | 3550.614 | 3568.616 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5267.076 | 5294.076 | 5325.921 | 5352.924 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7022.768 | 7058.768 | 7101.228 | 7137.232 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8778.46 | 8823.46 | 8876.535 | 8921.54 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10.615 | 28.616 | 3530.999 | 3549 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1727.076 | 1754.077 | 1785.922 | 1812.924 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5286.691 | 5313.692 | 5306.306 | 5333.308 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3482.768 | 3518.769 | 3561.229 | 3597.232 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7042.383 | 7078.384 | 7081.613 | 7117.616 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 57.232 | 21.23 | 7061.998 | 7098 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5381.54 | 5336.536 | 5283.461 | 5238.46 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1841.54 | 1796.537 | 1743.462 | 1698.46 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8856.92 | 8901.924 | 8798.075 | 8843.076 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8837.305 | 8882.308 | 8817.69 | 8862.692 |

Figure1 shows the OOB emission regions and IMD regions that could fall on the n3/B3 DL band, however, one can observe that only LTE band is subjected to the IMD5 and the rest of the noises do not fall on the reception bands, hence only IMD5 is considered for MSD analysis. It should be mentioned that if bandwidth combinations larger than the ones requested in Table 1 (for instance, LTE with CBW= 20MHz and NR with CBW=50 MHz) would be asked in future, the OOB and IMD3 may fall on the portion of the n3 DL band which would be occupied by the (NR or LTE) receiver and the MSD analysis should be updated.

Figure 1 Out of band and spurious emissions of a 30MHz NR Tx and IMDs

# MSD Analysis

The UE architecture for DC_(n)3AA is shown in Figure 2. The Tx noise, at band3/n3, could interfere with its Rx band via the following paths:

- Main TRx:

  - Duplexer Path: the PA is connected to the Tx input of the duplexer and the LNA is connected to its Rx input. The noise of the PA could leak, with heavy attenuation (TX/RX isolation), to the LNA input.

  - PCB Path: the PA noise could be received at the input of the LNA via the PCB with heavy attenuation (TRX PCB Isolation)

- Diversity Rx:

  - Antenna Path: the Diversity antenna can capture the Tx noise from the transmitter. This noise reachs the Diversity LNA through the reception chain.

  - Main/Diversity PCB Path: The PA noise could be received at the input of the Diversity LNA via the PCB with heavy attenuation (Main/Diversity PCB Isolation)

Figure 2 UE architecture

A summary of the parameters that are used in this analysis is given below:

- Tx image level = - 28 dBc

- Duplexer n3 Rx rejection at n3 Tx (Duplexer TX/RX isolation): 50 dB

- Antenna isolation = 10 dB

- Diversity SAW filter with n3 Rx rejection at n3 Tx = 50 dB

- TRX PCB Isolation = 60 dB

- Main/Diversity PCB Isolation = 65 dB

- Insertion loss: Diplexer, Antenna switch, Duplexer, SAW Diversity filter: 0.5 dB, 1 dB, 4 dB, 3 dB.

Table 3 contains the MSD analysis with an NR active Tx with a CBW of 30MHz and an inactive LTE Tx with a CBW of 20MHz.

Table 3 MSD Analysis due to IMD5 on LTE receiver on B3 DL.

|  | Main | Div |
| --- | --- | --- |
| LTE Rx BW | 20.000 | 20.000 |
| Receiver NF | 9.000 | 9.000 |
| TX Noise at RX band at the antenna | -89.000 | -99.000 |
| RX Thermal noise at the antenna | -92.447 | -92.447 |
| RX Total noise at the antenna | -87.380 | -91.580 |
| SNR | -1.520 | 2.680 |
| REFSENS | -88.900 | -88.900 |
| MRC RX total noise | -92.979 |  |
| MSD | 2.479 |  |
| SNR | 4.079 |  |
| MSD at SNR=-1 | 0.000 |  |

On the other hand, if the LTE Tx is on and NR Tx is off, the n3/B3 DL band is not impacted at all by the Tx noise (neither OOB nor IMD) as shown in Figure 3.

Figure 3 Out of band and spurious emissions of a 20MHz LTE Tx and IMDs

In case the LTE Tx (20 MHz) is placed the closest to the B3 DL band, as shown in Figure 4 there is no impact on the NR and LTE receivers’ noise level, neither. The Harmonics and IMD products are calculated in Table 4:

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1765 | 1774 | 1776 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3530 | 3548 | 3552 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5295 | 5322 | 5328 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7060 | 7096 | 7104 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8825 | 8870 | 8880 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2 | 20 | 3541 | 3559 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1745 | 1772 | 1778 | 1805 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5306 | 5333 | 5317 | 5344 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3510 | 3546 | 3554 | 3590 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7071 | 7107 | 7093 | 7129 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 40 | 4 | 7082 | 7118 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5375 | 5330 | 5320 | 5275 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1825 | 1780 | 1770 | 1725 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8869 | 8914 | 8836 | 8881 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8858 | 8903 | 8847 | 8892 |

IMD3 and IMD5 could fall on n3/B3 DL band but they are not impacting the LTE and NR receivers.

Hence No MSD is required for DC_(n)3AA with the requested band combinations in [2]

# Conclusion

Based on the above analysis, no MSD is needed for DC_(n)3AA for the requested bandwidth combinations in [2].

# Reference

- R4-2119803 TP for TR 37.717-11-11: DC_(n)3AA.

- RP-212096 Rel.17 1 band LTE 1 band NR WID combination list v1_RAN4_100e.

# Text Proposal

----- Start of TP -----

## 6.3.2	DC_(n)3

#### 6.3.2.5	MSD analysis for DC

Only single switched UL mode is supported and UL-DL separation is not large (20 MHz), however no MSD is needed for the requested channel bandwidths.

----- End of TP -----