# 3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2204972

Electronic Meeting, Feb. 21st – Mar. 3rd, 2022

Source: 	vivo

Title: 	Further discussion on impact of MPE requirements

Agenda Item:	10.19.2.2

Document for:	Discussion

Introduction

In RAN4#101-bis-e, extensive discussions were made regarding MPE requirements. A WF has been agreed in [1]:

WF2: Impact of MPE enhancements

- Discussion summary

  - Most companies support not to make changes to the existing P-MPR related definition/capabilities for the Rel-17 MPE enhancements.

  - On the other hand, some companies share that RAN4 still needs further check if the current definition/capability could accommodate the enhancement.

- Agreements

  - RAN4 will further check if the current definition/capability could implement the enhancement for a following case or not

    - Relation between each of the reported P-MPR values(N≤4) tied with the corresponding respective(M=1) SSBRI(s)/CRI(s) and Pcmax

  - For the next meeting, companies are encouraged to provide views if changes are required considering RAN1 agreements and above

In RAN1#107-e, the following agreements were made regarding MPE issue:

Conclusion

On Rel.17 enhancements to facilitate MPE mitigation, there is no consensus on a specification-based criterion for selecting N from a candidate SSB/CSI-RS resource pool

This basically means that RAN1 would not specify any criterion for beam selection and purely left for implementation.

In RAN1#107e-bis, there is no more discussion on this topic.

In RAN4#101-bis-e, discussions are being documented in [2], however consensus is still not reached and in WF [3] is basically still kept open for impact of MPE ehnahcements:

- Agreements

  - It will be further determined

    - whether to change current PCMAX definition based on the latest RAN1/2 agreement

    - whether to update the P-MPR NOTE based on potential CR proposed to the next meeting.

In this contribution, some views in [4] are resubmitted, and further proposal was provided.

Discussion

As discussed in last meeting, it was already commonly accepted that P-MPR would be reported with one corresponding SSBRI/CRI, and up to 4 values can be reported based on capabilities. In addition, RAN1 did not specify the criterion for selection of SSBRI(s)/CRI(s).

However, there are still different understandings on whether Pcmax definition or P-MPR note need further updated. There are arguments that P-MPR is now beam specific and need to be extended, but no stable or widely supported wordings were proposed. Just as already raised in [4], the P-MPR is already quite flexibly defined in the respective equation, without having a definite need to be extended.

PPowerclass + PIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

Observation 1: The current P-MPR item in the Pcmax related equation is already flexible to cover multiple Beam cases.

Furthermore, even if further extension can be considered for P-MPR, it is quite doubtful whether some more verification can be considered, since RAN1 spec itself is quite flexible and no selection criterion for selection of SSBRI(s)/CRI(s).

Observation 2: More tentative P-MPR extension can be more problematic in verification need or testability, thus may not fruitful.

Based on these two observations, the following proposal is provided:

Proposal: No extension of P-MPR concept or equation is needed.

Conclusion

In this paper, further views of UE RF impact of RAN1 MPE mitigation were provided.

Observation 1: The current P-MPR item in the Pcmax related equation is already flexible to cover multiple Beam cases.

Observation 2: More tentative P-MPR extension can be more problematic in verification need or testability, thus may not fruitful.

Proposal: No extension of P-MPR concept or equation is needed.

- References

[1] R4-2120006, WF on FeMIMO RF, Samsung, RAN4#101-e

[2] R4-2202331, Email discussion summary for [101-bis-e][131] NR_feMIMO, Moderator (Samsung), RAN4#101bis-e

[3] R4-2202370, WF on remaining issues of FeMIMO, Samsung, RAN4#101-bis-e

[4] R4-2200963, Further discussion on impact of MPE requirements, vivo, RAN4#101-bis-e