3GPP TSG-RAN WG4 Meeting #102-e	R4-2204980

Electronic Meeting, 21 Feb - 3 Mar, 2022

Title:	R17 FR2 power control for NR-DC and draft LS

Source:	OPPO

Agenda item:	13.2.1

Document for:	Approval

- Introduction

In RAN4#99e, RAN1 LS [1] is received, which asks the question on whether band combinations in FR2 could be independent power control.

However, RAN1 cannot determine if it is a feasible solution and would like to ask RAN4 inputs about the feasibility, for the following two possible cases: 1) uplink CCs of MCG and uplink CCs of SCG are in different frequency bands in FR2. 2) uplink CCs of MCG and uplink CCs of SCG are in the same frequency band in FR2. In [R1-2007509], RAN4 informed RAN1 that discussion on p-UE-FR2 is postponed to Rel17. RAN1 would like to know if RAN4 expected to discuss p-NR-FR2 also in Rel17. ACTION: RAN1 respectfully requests that RAN4 takes the above into account and provides feedback on the feasibility of independent power control for uplink CCs of MCG in FR2 and uplink CCs of SCG in FR2 for NR-NR Dual Connectivity.

RAN4 has agreed the WF [2] on the understanding of independent power control, i.e. per CG power control and no total power limitation. And it was also agreed that the LS will base on the outcome of total power concept discussion in FR2 inter-band UL CA.

This paper has been discussed in last RAN4 meeting, however, was stopped in 1st round due to not R17 topics. In this meeting, the contents were updated taking the progress in last meeting and also potential conclusions of total power concept into account.

Then in the annex a draft version is given, and it doesn’t mention about total power concept in IBM just saying it is not independent power control to give more room for RAN4 discussion. Hope it can be agreeable.

- Discussion

RAN1 is asking whether UL CCs configured in FR2 within same band or in different bands are independent power control and RAN1 is focus on NR-DC. However, currently in RAN4 FR2 NR-DC hasn’t been introduced and what can be referred is NR-CA. The conclusion for NR CA could be reused also for NR DC.

Observation 1:    FR2 NR-DC hasn’t been discussed in RAN4, but NR CA conclusion can be taken as reference.

In FR2 inter-band UL CA, the discussion will be more complex, and RAN4 has discussed CBM/IBM concepts in Rel-17.

As shown in figure 1, the CBM UE (at least for single chain architecture) shares Tx chain and antennas for band X and band Y, thus it is not independent power control.

Figure 1 CBM and IBM for inter-band UL CA

Observation 2:    Hardware are shared by CBM inter-band UL CA, thus it is not independent power control.

For the IBM UE, it is possible that band X and band Y can do independent power control if no limitation on the total power. However, this means UE can transmit powers in one band without considering the other band. Therefore, to achieve independent between band X and band Y, there must be no total power limitation in any case, no matter hardware limitation, regulation limitation, power consumption control or thermal heating control, etc.

Up to now there is still no conclusion whether total power concept will be considered in RAN4 specification no matter just considered as one relaxation factor or one explicit requirement in RAN4.

In previous meeting, it was agreed that for total power handling the UE power consumption should be addressed and further study how to address it. No matter what kind of solutions defined in the end, it is clear that the two bands will have impact on the power control rather than independent of each other. This is especially true in FR2 that the thermal issue is much severe than the FR1, and some companies even suggest to use PMPR to solve this issue.

From UE implementation perspective, we don’t believe that FR2 inter band UL CA is independent power control for the reasons mentioned above no matter requirement defined in RAN4 or not.

Observation 3:    No matter total power restriction in RAN4 is defined or not, it is clear in implementation that inter-band UL CA cannot be worked as independent power control.

Therefore, with above consideration, the inter-band UL CA in FR2 with no matter CBM or IBM are not independent power control.

Observation 4:    For IBM inter-band UL CA, it is not independent power control.

In summary, no matter for FR2 intra-band UL CA or inter-band UL CA, it should not consider the PCG and SCG as independent power control in RAN1 design. The conclusion also applies to FR2 NR DC.

Proposal 1:         It is proposed to inform RAN1 that FR2 intra-band or inter-band NR DC power control are not independent and not mention about the total power concept to avoid different understandings.

- Conclusion

In this paper we analyzed different cases of UL CA and its power control impacts, and get the following observations and proposals. It should be noticed that this proposal is not depend on the discussion of “total power concept” since no matter it is defined in RAN4 or not UE still has to control its total power for the sake of heating issues, etc. And in the annex a draft LS is given without touching “total power concept” to avoid different understandings.

Observation 1:    FR2 NR-DC hasn’t been discussed in RAN4, but NR CA conclusion can be taken as reference.

Observation 2:    Hardware are shared by CBM inter-band UL CA, thus it is not independent power control.

Observation 3:    No matter total power restriction in RAN4 is defined or not, it is clear in implementation that inter-band UL CA cannot be worked as independent power control.

Observation 4:    For IBM inter-band UL CA, it is not independent power control.

Proposal 1:         It is proposed to inform RAN1 that FR2 intra-band or inter-band NR DC power control are not independent and not mention about the total power concept to avoid different understandings.

- References

[1] R4-2107605, Further Reply LS on power control for NR-DC, RAN1

[2] R4-2114961, WF on FR2 UL CA, Samsung

=========================================================================

Title:	[Draft] Further Reply LS on power control for NR-DC

Response to:	(R1-2104018) Further Reply LS on power control for NR-DC

Release:	Rel-16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for the Further Reply LS on power control for NR-DC.

Regarding the feasibility of independent power control for the mentioned two cases:

1) uplink CCs of MCG and uplink CCs of SCG are in different frequency bands in FR2.

2) uplink CCs of MCG and uplink CCs of SCG are in the same frequency band in FR2.

RAN4 has discussed the definition of independent power control, and the understanding is that it means power control is per CG, and there is no total power limitation.

Up to now RAN4 hasn’t introduce NR-DC band combination for FR2, therefore UL CA is referred here and it is RAN4 understanding that same conclusion can be applied to NR-DC.

For inter-band UL CA, RAN4 has discussed two kinds of UE implementations, one is called CBM (Common Beam Management), and the other is called IBM (Independent Beam Management).

- For the CBM UE, typically Tx hardware are shared between the two bands which makes the Tx power for MCG and SCG are not independent of each other.

- For the IBM UE, typically separate Tx hardware are used for the two bands. RAN4 has agreed that no new requirements than the per-band based requirement package will be defined for power control, however, it is RAN4 understanding that for this kind of UE still the Tx power for MCG and SCG are not independent of each other.

For intra-band UL CA, total max TRP and max EIRP limitation are defined in section 6.2A.4 of TS 38.101-2, thus power sharing is needed when the maximum power limitation was reached. Therefore, it is not independent power control.

In addition, RAN4 do not plan to discuss p-NR-FR2 or p-UE-FR2 in Rel-17.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above information into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #103-e	16  -  27 Fri 2022         E-meeting