3GPP TSG-RAN4 Meeting #102-e	R4-2205053

# Electronic meeting, February 21 – March 3, 2022

Source:		Ericsson

Title:			pCR to TS 38.101-5 – Receiver requirements general

Agenda item:		10.13.4.2

Document for:		Approval

# Background

Based on the various agreements from past RAN4 meetings, the following test is proposed for TS 38.101-5:

- Conducted receiver – general

- Diversity characteristics

# Proposal

It is proposed that the proposed text related to the receiver requirements general here after is included in TS 38.101-5 [1].

# References

- TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

# Text proposal

<Start of the change>

# 7	Conducted receiver characteristics

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective clauses below.

With the exception of clause 7.3, the requirements shall be verified with the network signalling value NS_01 configured (Table 6.2.3-1).

All the parameters in clause 7 are defined using the UL reference measurement channels specified in Annex A.2.2 of TS 38.101-1, the DL reference measurement channels specified in Annex A.3.2 of TS 38.101-1 and using the set-up specified in Annex C.3.1 of TS 38.101-1.

## 7.2	Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands.

For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports.

For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.

The above rules apply for all clauses with the exception of clause 7.9.

<End of the change>