3GPP RAN WG4 Meeting #102-e		R4-2205179

Electronic meeting, February 21 – March 3, 2022

Agenda item:	9.2.5

Source:	Apple

Title:	Text proposal for TR 38.849 (background results for the existing A-MPR values)

WI/SI:	NR_6GHz_unlic_full

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  5GHz is a well-known band for the unlicensed operation, but 6GHz is a relative new band usage of which was approved recently in different regulatory regions. While the 6GHz band for the US is already part of the Rel-16 core functionality, current 3GPP specifications do not support it for other countries, such as South Korea and Canada, which have finalised their regulatory requirements only recently. Thus, RAN#92 meeting approved a new WI aim of which is to enable support of the 6GHz unlicensed band for those countries and regions that have finalised recently the corresponding regulatory requirements.

After the RAN4#101 meeting existing Region 2 and Region 3 regulatory requirements were analysed and RAN WG4 captured simulated A-MPR values for several countries, such as Canada, South Korea, Peru, and Chile. However, while capturing the A-MPR values no background information was included on the simulated scenarios and intermediate simulation results. Thus, this TP adds background results for the existing A-MPR values.

# 2	Text proposal for TR 38.849

---------- TEXT PROPOSAL BEGIN ---------

#### 6.1.1.2	A-MPR for a NS(s) for the full 6GHz NR unlicensed operation

##### 6.1.1.2.1	Canada

Table 6.1.1.2.1-1: Signal setup.

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 60 | 162RB0 | 30 |
|  | 6 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 8 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 9 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 10 | CP-OFDM | 60 | 162RB0 | 30 |
|  | 11 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 12 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 13 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 14 | DFT-s-OFDM | 60 | 1RB0 every 10RBs (30x) | 30 |
|  | 15 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 16 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 17 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 18 | CP-OFDM | 60 | 1RB0 every 10RBs (33x) | 30 |
|  | 19 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

Figure 6.1.1.2.1-1: A-MPR simulation results for PC5 LPI in Canada.

Table 6.1.1.2.1-2: A-MPR for PC5 LPI in Canada

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |
| --- | --- | --- | --- |
|  |  | 20 MHz |  |
|  |  | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |

##### 6.1.1.2.2	South Korea

Table 6.1.1.2.2-1: Signal setup.

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 60 | 162RB0 | 30 |
|  | 6 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 8 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 9 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 10 | CP-OFDM | 60 | 162RB0 | 30 |
|  | 11 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 12 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 13 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 14 | DFT-s-OFDM | 60 | 1RB0 every 10RBs (30x) | 30 |
|  | 15 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 16 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 17 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 18 | CP-OFDM | 60 | 1RB0 every 10RBs (33x) | 30 |
|  | 19 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

Figure 6.1.1.2.2-1: A-MPR simulation results for PC5 LPI in South Korea.

Table 6.1.1.2.2-2: A-MPR for PC5 LPI in South Korea

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |

##### 6.1.1.2.3	Peru and Chile

Table 6.1.1.2.3-1: Signal setup.

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 60 | 162RB0 | 30 |
|  | 6 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 8 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 9 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 10 | CP-OFDM | 60 | 162RB0 | 30 |
|  | 11 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 12 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 13 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 14 | DFT-s-OFDM | 60 | 1RB0 every 10RBs (30x) | 30 |
|  | 15 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 16 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 17 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 18 | CP-OFDM | 60 | 1RB0 every 10RBs (33x) | 30 |
|  | 19 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

##### Figure 6.1.1.2.3-1: A-MPR simulation results for PC5 LPI in Peru and Chile.

Table 6.1.1.2.3-2: A-MPR for PC5 LPI in Peru and Chile

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |

The A-MPR results for PC5 LPI in Peru and Chile are mostly identical to the A-MPR results defined by NS_53 (LPI in US). In some cases, A-MPR results are 0.5-1dB relaxed comparing to NS_53. Thus, accounting for the marginal difference between these A-MPR values and NS_53, the latter can be re-used to support Peru and Chile.

---------- TEXT PROPOSAL END ---------