3GPP TSG-RAN4 Meeting #102-e	R4-2205189

Electronic Meeting, 21st Feb – 3rd March, 2022

Source: 	Huawei, HiSilicon

Title: 	On 60GHz UE Rx RF requirements

Agenda Item:	10.16.3.2

Document for:	Approval

Introduction

For 60GHz UE, there is no agreement on the REFSENS requirements yet. This paper provides some view on the REFSENS requirement for 52.6-71GHz frequency.

Discussion

The EIS performance highly depends on UE’s antenna array design. During RAN4#101bis-e meeting, it was agreed to take 8 elements assumption as a starting point. The estimated EIS budget with 8 antenna elements assumption is provided in below table.

Table 2-1 EIS budget items

| Parameter | units |  |
| --- | --- | --- |
| Frequency range | GHz | 57~71 |
| SNR requirement | dB | -1 |
| Bandwidth | MHz | 100 |
| Thermal noise | dBm/Hz | -174 |
| Noise Figure | dB | 17 |
| Number of antenna elements | / | 8 |
| Element gain (per polarization) | dBi | 4.0 |
| Antenna roll-off loss vs frequency | dB | -3 |
| Realized antenna array gain | dBi | 13 |
| Diversity Gain | dB | 0 |
| Total implementation loss | dB | -12.5 |
| REFSENS | dBm | -75.5 |

Proposal 1: For 60GHz handheld UE, the REFSENS requirement for 52.6~71 GHz is -75.5 dBm/100 MHz based on 8 elements antenna array assumption.

When RF requirements were discussed for band n262, different averaging methods were compared and finally ‘Average (linear) excluding extremes’ was used for EIRP, REFSENS and Spherical coverage requirement [3]. To keep the consistency of RAN4 requirements, it’s proposed to adopt the same method for band n263.

Proposal 2: For 60GHz handheld UE, the averaging is executed in the way of ‘Average (linear) excluding extremes’.

Regarding EIS spherical coverage, the main view is to specify the 50%-tile percentage power level. Considering the Tx/Rx reciprocity of antenna directivity, the gain drop for EIS spherical coverage could be the same as that of EIRP spherical coverage.

Proposal 3: For 60GHz handheld UE, the EIS spherical coverage requirement could be decided by reusing the analysis of EIRP gain drop from 50%-tile to peak EIRP.

Conclusion

In this contribution we discussed on the EIS requirements for 52.6~71GHz, according to the analysis, we have the following proposals:

Proposal 1: For 60GHz handheld UE, the REFSENS requirement for 52.6~71 GHz is -75.5 dBm/100 MHz based on 8 elements antenna array assumption.

Proposal 2: For 60GHz handheld UE, the averaging is executed in the way of ‘Average (linear) excluding extremes’.

Proposal 3: For 60GHz handheld UE, the EIS spherical coverage requirement could be decided by reusing the analysis of EIRP gain drop from 50%-tile to peak EIRP.

- References

[1] R4-2120062, “WF on 60 GHz UE RF requirements”, RAN4#101e

[2] R4-2202328	“Email discussion summary for [101-bis-e][128] NR_ext_to_71GHz_Part_2”, RAN4#101bis-e

[3] R4-2107790, ‘WF on n262 UE EIRP/EIS requirement’, RAN4#99e