3GPP TSG-RAN WG4 Meeting #102e	R4-2205223

Electronic Meeting, 21st Feb - 3rd Mar, 2022

Source:	ZTE Corporation

Title:	Discussion on SRS sharing and antenna switching

Agenda Item:	10.7.3.1

Document for:	Discussion

# Introduction

In RAN4#101bis-e, two WF were approved and there are open issues related to SRS sharing and antenna switching as seen below [1] [2].

Issue 1. Whether it is possible for periodic SRS transmission occasions be shared by two resource sets with different usage? Option 1: Yes Option 2: No; Tentative agreement: No consensus on this issue. Issue 2. Whether we need to ask RAN1 to confirm the first issue? Option 1: Yes Option 2: No Tentative agreement: No consensus on this issue.

Issue 3-1-2: Is SRS power difference for antenna switching dependent on other features than TxD - Way forward: - Is SRS power difference for antenna switching dependent on other features than TxD: - Option 1: Yes - Option 2: No

These issues should be discussed and closed in the forthcoming WG meeting.

# Discussion

Whether it is possible for periodic SRS transmission occasions be shared by two resource sets with different usage?

The meaning of sharing SRS transmission occasions here is that the same SRS resource ID can be indicated in different SRS resource sets (each SRS resource set is associated with only one purpose, e.g., ‘SRS antenna switching’, ‘beam management’ etc.), and from RAN2 signalling perspective, it could be supported, as illustrated below.

SRS-ResourceSet ::= SEQUENCE { srs-ResourceSetId SRS-ResourceSetId, srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId OPTIONAL, -- Cond Setup usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching}, ... } SRS-Resource ::= SEQUENCE { srs-ResourceId SRS-ResourceId, nrofSRS-Ports ENUMERATED {port1, ports2, ports4}, ptrs-PortIndex ENUMERATED {n0, n1 } OPTIONAL, -- Need R resourceMapping SEQUENCE { startPosition INTEGER (0..5), nrofSymbols ENUMERATED {n1, n2, n4}, repetitionFactor ENUMERATED {n1, n2, n4} }, freqDomainPosition INTEGER (0..67), freqDomainShift INTEGER (0..268), freqHopping SEQUENCE { c-SRS INTEGER (0..63), b-SRS INTEGER (0..3), b-hop INTEGER (0..3) }, ... }.

From RAN1 specs, sharing the same SRS resource ID in different SRS resource set is of no issue implying  that the same SRS resource can serve different purposes at the same time. And there is no need to send an LS to RAN1.

Proposal 1: One SRS resource can be indicated by two resource sets with different usages.

Proposal 2: RAN4 does not need to send an LS on the SRS sharing issue.

Is SRS power difference for antenna switching dependent on other features than TxD

Even if SRS virtualization is required under the capability of TxD, the same SRS power difference is still applicable to that when SRS is not virtualized, in other words, other features than TxD plays no impact on the SRS power difference for antenna switching.

Proposal 3: SRS power difference for antenna switching is not dependent on other features than TxD

# Conclusion

In this paper, we have the following proposals for SRS sharing and antenna switching:

Proposal 1: One SRS resource can be indicated by two resource sets with different usages.

Proposal 2: RAN4 does not need to send an LS on the SRS sharing issue.

Proposal 3: SRS power difference for antenna switching is not dependent on other features than TxD

# Reference

R4-2202353 WF on SRS sharing, Vivo

R4-2202354 WF on ULFPTx requirements for fallback and SRS antenna switching, Samsung