3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2205231

# Electronic Meeting, February 21 – March 03, 2022

Source:	LG Electronics

Title:	Discussion on Rx RF requirements in FR2-2

Agenda item:	10.16.3.2

Document for:	Discussion

# Introduction

In this paper, we provide our view on Rx RF requirements in FR2-2.

# Discussion

In last RAN4 meeting, some WFs were made for NR_ext_to_71GHz. We would like to discuss UE transmit power for vehicular UE based on WF [1].

The related agreements in WF are captured.

1 Handheld UE number of antenna element assumption - Note: This is an assumption used to derive requirements, and not a requirement. - Agreement from GTW: Use 8 antenna elements as the assumption for defining minimum requirements - Check if there is any performance problem. If performance problem is identified and agreed, revisit the agreement. - The simulation assumptions for co-existence study can be used as baseline in the second round or future meetings for the performance analysis. 2 Handheld UE min peak EIRP - Agreement: Average handheld UE min peak EIRP in the range of [ 13.2 to 14.1 ] dBm which is the range of values arrived at by computing and average in dB domain and average in power domain. 3 Handheld UE REFSENS - Agreement: Min SENS for n263 400 MHz, based on averaging the proposals in the table is [ -73.0 dBm ] 7 FWA UE number of antenna panels for spatial coverage - Agreement: One panel as an assumption for FWA specification development. All companies that commented agreed with 1 panel. 10 Vehicular UE - Agreed WF: Output power, reference sensitivity and spherical coverage for vehicular UE was discussed but due to limited amount of inputs no agreements could be made. Companies are requested to provide their inputs in the next meeting. Requirements for Vehicular UE should be defined using similar approach as is done for Handheld UE. Based on the agreement in RAN4#101e Vehicular array size is equal to or larger than the handheld.

## FR2-1 background

Table 2.1.1 and Table 2.1.2 summarize the reference sensitivity and the EIS spherical coverage which correspond to channel bandwidth of 400MHz for power class 1, 2, 3, 4 and 5 in FR2-1.

Table 2.1.1: Reference sensitivity for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | Frequency | REFSENS (dBm) / 400MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PC1 | PC2 | PC3 | PC4 | PC5 |
| n257 | 28GHz(26500MHz – 29500MHz) | -88.5 | -83.0 | -79.3 | -88.0 | -83.6 |
| n258 | 24GHz(24250MHz – 27500MHz) | -88.5 | -83.0 | -79.3 | -88.0 | -83.8 |
| n259 | 39GHz(39500MHz – 43500MHz) |  |  | -75.7 |  | -80.7 |
| n260 | 39GHz(37000MHz – 40000MHz) | -85.5 |  | -76.7 | -86.0 |  |
| n261 | 28GHz(27500MHz – 28350MHz) | -88.5 | -83.0 | -79.3 | -88.0 |  |
| n262 | 47GHz(47200MHz – 48200MHz) | -83.5 | -77.8 | -73.8 | -82.0 |  |

Table 2.1.2: EIS spherical coverage for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | EIS at Xth %-tile CCDF (dBm) / 400MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PC1 | PC2 | PC3 | PC4 | PC5 |
|  | X = 85 | X = 60 | X = 50 | X = 20 | X = 85 |
| n257 | -80.5 | -72.0 | -68.4 | -79.0 | -75.6 |
| n258 | -80.5 | -72.0 | -68.4 | -79.0 | -75.8 |
| n259 |  |  | -62.9 |  | -72.7 |
| n260 | -77.5 |  | -64.1 | -74.0 |  |
| n261 | -80.5 | -72.0 | -68.4 | -79.0 |  |
| n262 | -75.3 | -65.9 | -60.7 | -69.9 |  |

Table 2.1.3 summarize the difference between the reference sensitivity and EIS spherical coverage which correspond to each X%-tile for power class.

Table 2.1.3: Difference between the reference sensitivity and EIS spherical coverage for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | Difference between the reference sensitivity and EIS spherical coverage (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PC1 | PC2 | PC3 | PC4 | PC5 |
|  | X = 85 | X = 60 | X = 50 | X = 20 | X = 85 |
| n257 | 8.0 | 11.0 | 10.9 | 9.0 | 8.0 |
| n258 | 8.0 | 11.0 | 10.9 | 9.0 | 8.0 |
| n259 |  |  | 12.9 |  | 8.0 |
| n260 | 8.0 |  | 12.9 | 12.0 |  |
| n261 | 8.0 | 11.0 | 10.9 | 9.0 |  |
| n262 | 8.2 | 11.9 | 13.1 | 12.1 |  |

## Vehicular UE

For vehicular UE, based on the agreement in WF, 16 antenna elements (16Rx) is considered.

For minimum peak EIRP the analysis in Table 2.2.1 shows that level of -80.3dBm/400MHz can be achieved.

Table 2.2.1: Reference sensitivity for vehicular UE in FR2-2

| Parameter | Unit | Value |
| --- | --- | --- |
| Frequency range | GHz | 57~71 |
| Modulation |  | QPSK |
| SNR requirement | dB | -1 |
| Bandwidth | MHz | 400 |
| 10*log10(Max. RX BW), Max.Rx BW = 264RB@120kHz | dB | 85.8 |
| Thermal noise | dBm/Hz | -174 |
| Noise Figure(NF) | dB | 15 |
| # of antennas in an array |  | 16 |
| Array gain | dB | 12.04 |
| Average antenna element gain | dBi | 4.6 |
| Antenna gain roll-off over frequency | dB | -2.5 |
| Realized antenna array gain | dBi | 14.1 |
| Diversity gain | dB | 0 |
| Total Implementation loss(ILs) | dB | 8 |
| REFSENS @ 400MHz | dBm | -80.3 |

Proposal 1: For vehicular UE, reference sensitivity at channel band width of 400MHz is -80.3dBm is proposed (based on 16 antenna elements).

Table 2.2.2: Reference sensitivity for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | Frequency | REFSENS (dBm) / 400MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PC2 |  |  |  |
| n257 | 28GHz(26500MHz – 29500MHz) |  | -83.0 |  |  |  |
| n258 | 24GHz(24250MHz – 27500MHz) |  | -83.0 |  |  |  |
| n259 | 39GHz(39500MHz – 43500MHz) |  |  |  |  |  |
| n260 | 39GHz(37000MHz – 40000MHz) |  |  |  |  |  |
| n261 | 28GHz(27500MHz – 28350MHz) |  | -83.0 |  |  |  |
| n262 | 47GHz(47200MHz – 48200MHz) |  | -77.8 |  |  |  |
| n263 | 60GHz(57000MHz – 71000MHz) |  | -80.3 |  |  |  |

For EIS spherical coverage, the difference between the minimum peak EIRP and EIRP at 60%-tile CDF can be reused as is also done in FR2-1. Analysis for TX spherical coverage is shown in [2] based on 16 antennal elements (2x8 matrix).

Based on analysis above and the agreement to use one panel for defining the spherical coverage requirements for FWA UE following proposal is made.

Proposal 2: For vehicular UE, EIS spherical coverage 60%-tile CDF is 15.1 dB higher than reference sensitivity (based on 16 antenna elements).

Table 2.2.3: Difference between the reference sensitivity and EIS spherical coverage for Power Class 2

| Operating band | Difference between the reference sensitivity and EIS spherical coverage (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PC2 |  |  |  |
|  |  | X = 60 |  |  |  |
| n257 |  | 11.0 |  |  |  |
| n258 |  | 11.0 |  |  |  |
| n259 |  |  |  |  |  |
| n260 |  |  |  |  |  |
| n261 |  | 11.0 |  |  |  |
| n262 |  | 11.9 |  |  |  |
| n263 |  | 15.1 |  |  |  |

# Conclusion

In this contribution, we provided our views on reference sensitivity and EIS spherical coverage for 60GHz vehicular UE.

Proposal 1: For vehicular UE, reference sensitivity at channel band width of 400MHz is -80.3dBm is proposed (based on 16 antenna elements).

Proposal 2: For vehicular UE, EIS spherical coverage 60%-tile CDF is 15.1 dB higher than reference sensitivity (based on 16 antenna elements).

Draft CR that captures the proposal can be found in R4-2205229 [3].

# Reference

[1] R4-2202366, “	WF on 60GHz UE RF”, Qualcomm

[2] R4-2205227, “Discussion on Tx RF requirements in FR2-2”, LG Electronics

[3] R4-2205229, “draft CR on vehicular UE Rx RF requirements  in FR2-2”, LG Electronics