3GPP TSG-RAN WG4 Meeting # 102-e                           R4-2205280

Electronic Meeting, 21 February– 3 March, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on simplifying extended MSD table

Agenda Item:	9.27.2

Document for:	Approval

Introduction

In last RAN4 meeting, a WF [1] on improvements to MSD table in Rel-17 was approved. There are some proposals listed below for the way forward.

P1: The combos introduced in R17 are allowed to be discussed for improvements to MSD table.

FFS the impact on ENDC combos for R17.

FFS how to implement these changes.

P2: Reduce or restrict the number of test points for a given MSD type and a given band combination

FFS the number of test points.

P3: Option 1a: The specific necessity of all these types (“>ACLR2”, “ACLR1/ACLR2” and “C-IM3/C-IM5”) can be discussed on a case-by-case basis.

Note: These types are defined in WF R4-2119878.

P4: For MSD test configurations due to cross band isolation, the proposed guidelines can be way forward.

Guideline A: The UL/DL carrier frequencies shall be carefully selected so as to minimize the frequency distance that separates the UL aggressor carrier from the DL victim’s carrier.

P5: How to specify the MSD due to cross band isolation (“>ACLR2”) for Rel-17 band combinations.

Option 1:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1720 | 20 | 15 | 100 (RBstart=0) | 1515.5 | 5 | 2.6 | >ACLR2 |
| n18 | n28 | 817.5 | 5 | 15 | 18 (RBstart=0) | 800.5 | 5 | 4.5 | >ACLR2 |
| n34 | n3 | 2012.5 | 5 | 15 | 25 (RBstart=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n46 | n78 | 5190 | 100 | 30 | 216 (RBstart=0) | 3795 | 10 | 10.4 | >ACLR2 |
| n46 | n78 | 5190 | 100 | 30 | 216 (RBstart=0) | 3750 | 100 | 5.1 | >ACLR2 |
| n77 | n41 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n41 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| NOTE X: When the victim DL bands are FDD bands, the UL RB allocation of victim FDD bands shouldn’t be configured |  |  |  |  |  |  |  |  |  |

P6: FFS on whether Tx / Rx harmonic mixing tables can be merged in a common table

P7: PC2 and PC3 MSD should be merged in the common table.

P8: candidate options for the format of MSD table due to harmonics

Option 1:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| nX | nY | TBD | 5 | 15 | 25 (RBstart=0) | TBD | 5 | 23.5 | UL2/DL1 |

Option 2:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| nX | nY | 5 | 15 | 25 (RBstart=0) | 10 | 23.5 | Note1 | UL2/DL1 |
| Note 1: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

In this paper, we’d like to provide our views to address the open issues.

Discussion on applicability and scope

For the impacts on ENDC combos for R17, we can find the following R17 NR ENDC band combinations which have MSD due to harmonic interference and cross band isolation after comparing the TS 38.101-1-ga0 with TS 38.101-1-h40.

Table 1 R17 NR ENDC band combinations which have MSD due to harmonic interference and cross band isolation

| The type of MSD | NR ENDC band combination | The release in which NR ENDC band combinations are introduced. |
| --- | --- | --- |
| MSD due to harmonic interference for NR ENDC | DC_2_n77 | R17 |
|  | DC_42_n3 | R17 |
|  | DC_5_n77 | R17 |
|  | DC_7_n79 | R17 |
|  | DC_8_n7 | R17 |
|  | DC_12_n77 | R17 |
|  | DC_28_n66 | R17 |
|  | DC_21_n28 | R17 |
|  | DC_25_n77 | R17 |
|  | DC_25_n78 | R17 |
|  | DC_48_n25 | R17 |
|  | DC_28_n1 | R17 |
|  | DC_4_n28 | R17 |
|  | DC_66_n28 | R17 |
|  | DC_71_n2 | R17 |
|  | DC_71_n41 | R17 |
|  | DC_66_n77 | R17 |
| MSD due to cross band isolation for PC3 NR CA | DC_41_n1 | R17 |
|  | DC_18_n28 | R17 |
|  | DC_38_n1 | R17 |
|  | DC_41_n1 | R17 |
|  | DC_12_n71 | R17 |
| MSD due to cross band isolation for PC2 NR CA | DC_7_n78 | R17 |
|  | DC_2_n41 | R17 |
|  | DC_66_n41 | R17 |

The impact on ENDC combos for R17 is limited based on the summary above, so the WF guideline can be applicable to ENDC band combinations from R17.

Observation 1: the WF guideline can be applicable to ENDC band combinations from R17, since the impact on ENDC combos for R17 is limited based on the summary above.

For the number of test points for a given MSD type and a given band combination, we try to provide the following proposal for a compromise.

Proposal 1: More than one MSD test point can be introduced for a given band combination based on the reasonable judgement, but the maximum number of MSD test point can be five and it’s allowed to test only one configuration.

Discussion on cross band isolation MSD

Based on the approved WF [1] in last meeting, the following MSD test points due to cross band isolation (“>ACLR2”) for Rel-17 band combinations were approved.

Option 1:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1720 | 20 | 15 | 100 (RBstart=0) | 1515.5 | 5 | 2.6 | >ACLR2 |
| n34 | n3 | 2012.5 | 5 | 15 | 25 (RBstart=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n46 | n78 | 5190 | 100 | 30 | 216 (RBstart=0) | 3795 | 10 | 10.4 | >ACLR2 |
| n46 | n78 | 5190 | 100 | 30 | 216 (RBstart=0) | 3750 | 100 | 5.1 | >ACLR2 |
| n77 | n41 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n41 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| NOTE X: When the victim DL bands are FDD bands, the UL RB allocation of victim FDD bands shouldn’t be configured |  |  |  |  |  |  |  |  |  |

For CA_n18-n28, company commented that 15MHz UL BW should be used. Thus, the ACLR1 of UL band n18 may have an impact on Rx band of band n28. However, after checking TS 36.101, there is a restriction on the frequency range of band 28 for CA_18-28, i.e. 758~788MHz. It’s supposed that the similar restriction should be applied for CA_n18-n28. The MSD test point for CA_n18-n28 is proposed as below.

Proposal 2: The MSD test point for CA_n18-n28 is proposed as below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n18 | n28 | 822.5 | 15 | 15 | 18 (RBstart=0) | 785.5 | 5 | 2.6 | >ACLR2 |

Besides, we need to specify the MSD test point for CA_n1-n3 and CA_n1-n40 due to the “ACLR1/ACLR2” interference. Based on the analysis [2], the MSD test point for CA_n1-n3 and CA_n1-n40 due to the “ACLR1/ACLR2” interference is proposed as below.

Proposal 3: the MSD test point for CA_n1-n3 and CA_n1-n40 due to the “ACLR1/ACLR2” interference is proposed as below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n1 | n3 | 1945 | 50 | 15 | 270 (RBstart=0) | 1877.5 | 5 | 22.5 | ACLR1/2 |
| n40 | n1 | 2340 | 80 | 30 | 270 (RBstart=0) | 2167.5 | 5 | TBD | ACLR1/2 |

Discussion on harmonic MSD

Based on the approved WF, there are some open issues to be discussed for the MSD due to harmonic interference. Since RAN4 can’t reach an agreement on whether Tx / Rx harmonic mixing tables can be merged in a common table, one proposal for compromise is to use the same format for both Tx harmonic interference table and Rx harmonic mixing table, no matter whether these two tables can be merged or not.

Proposal 4: To use the same format for both Tx harmonic interference table and Rx harmonic mixing table.

For the candidate format of MSD table due to harmonics. There are two options as below.

Option 1:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| nX | nY | TBD | 5 | 15 | 25 (RBstart=0) | TBD | 5 | 23.5 | UL2/DL1 |

Option 2:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| nX | nY | 5 | 15 | 25 (RBstart=0) | 10 | 23.5 | Note1 | UL2/DL1 |
| Note 1: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

From core perspective, it’s better to specify the requirements by using general method. Thus, it’s proposed to take option2 as the format of MSD table due to harmonics.

Proposal 5: Option 2 can be used as the format of MSD table due to harmonics.

Conclusion

Based on the analysis and discussion, the proposal and observations are shown below.

Observation 1: the WF guideline can be applicable to ENDC band combinations from R17, since the impact on ENDC combos for R17 is limited based on the summary above.

Proposal 1: More than one MSD test point can be introduced for a given band combination based on the reasonable judgement, but the maximum number of MSD test point can be five and it’s allowed to test only one configuration.

Proposal 2: The MSD test point for CA_n18-n28 is proposed as below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n18 | n28 | 822.5 | 15 | 15 | 18 (RBstart=0) | 785.5 | 5 | 2.6 | >ACLR2 |

Proposal 3: the MSD test point for CA_n1-n3 and CA_n1-n40 due to the “ACLR1/ACLR2” interference is proposed as below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n1 | n3 | 1945 | 50 | 15 | 270 (RBstart=0) | 1877.5 | 5 | 22.5 | ACLR1/2 |
| n40 | n1 | 2340 | 80 | 30 | 270 (RBstart=0) | 2167.5 | 5 | TBD | ACLR1/2 |

Proposal 4: To use the same format for both Tx harmonic interference table and Rx harmonic mixing table.

Proposal 5: Option 2 can be used as the format of MSD table due to harmonics.

Option 2:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| nX | nY | 5 | 15 | 25 (RBstart=0) | 10 | 23.5 | Note1 | UL2/DL1 |
| Note 1: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

- References

[1]	R4-2202287	WF on improvements to MSD table, Huawei, HiSilicon

[2]	R4-2119591 BCS4 MSD improvements, Skyworks Solutions, Inc.