3GPP TSG-RAN4 Meeting #102-e	R4-2205356

# Electronic Meeting, 21 February – 3 March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on accuracy requirements for positioning measurement R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2022-03-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The margin for group delay calibration error and frequency drift are missing for RSTD accuracy. - The maximum time offset (in the context of frequency drift margin) between the two PRS resources instances for RSTD requirement is TBD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the overall margin for group delay calibration error and frequency drift to RSTD accuracy requirements, - Define the maximum time offset between the two PRS resources instances for RSTD measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE positioning accuracy requirements are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.23.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.1.23.2	Measurement Accuracy Requirements

The accuracy requirements for RSTD measurement shall be within ±(X+Y) Tc.

X is defined in Table 10.1.23.2-1 for AWGN channel and Table 10.1.23.2-3 for fading channel for FR1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

X is defined in Table 10.1.23.2-2 for AWGN channel and Table 10.1.23.2-4 for fading channel for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

Note: The requriements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

When UE measures RSTD on PRS resources belonging to different PFLs, then the RSTD accuracy is defined as the accuracy corresponding to the largest accuracy value among different PFLs.

When UE measures RSTD on PRS resources belonging to same PFL, Y=32 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 160 ms.

When UE measures RSTD on PRS resources belonging different PFLs, Y=[256] Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than [1280] ms.

[Editor notes: The margins for measurements on different PFLs shall be considered in the group delay margin]

Table 10.1.23.2-1: RSTD absolute accuracy in FR1 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| [132+TBD] +ΔNote 7 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| [98+TBD] +Δ |  |  | ≥ [52] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [78] +Δ |  |  | ≥ [104] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [75+TBD] +Δ |  | 30 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -117 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 | -50 |
| [84] +Δ |  |  | ≥ [48] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [40] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [86] +Δ |  | 60 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -114 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 | -50 |
| [40] +Δ |  |  | ≥ [64] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [22] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Δ=TBD. |  |  |  |  |  |  |  |

Table 10.1.23.2-2: RSTD absolute accuracy in FR2 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| [35+TBD] +ΔNote 6 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ [24] | ≥ [1] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| [56] +Δ |  |  | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [27] +Δ |  |  | ≥ [132] | ≥ [1] | Note 5 | Note 5 |
| [56] +Δ |  | 120 | ≥ [32] | ≥ [1] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| [29] +Δ |  |  | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [18] +Δ |  |  | ≥ [128] | ≥ [1] | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Δ=TBD. |  |  |  |  |  |  |

Table 10.1.23.2-3: RSTD absolute accuracy in FR1 for fading channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| [247+TBD] +ΔNote 7 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| [140+TBD] +Δ |  |  | ≥ [52] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [122] +Δ |  |  | ≥ [104] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [118+TBD] +Δ |  | 30 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -117 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 | -50 |
| [145] +Δ |  |  | ≥ [48] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [44] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [183] +Δ |  | 60 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -114 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 | -50 |
| [43] +Δ |  |  | ≥ [64] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| [33] +Δ |  |  | ≥ [132] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Δ=TBD. |  |  |  |  |  |  |  |

Table 10.1.23.2-4:  RSTD absolute accuracy in FR2 for fading channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| [83+TBD] +ΔNote 6 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ [24] | ≥ [4] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| [96] +Δ |  |  | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [62] +Δ |  |  | ≥ [132] | ≥ [1] | Note 5 | Note 5 |
| [80] +Δ |  | 120 | ≥ [32] | ≥ [4] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| [70] +Δ |  |  | ≥ [64] | ≥ [1] | Note 5 | Note 5 |
| [48] +Δ |  |  | ≥ [128] | ≥ [1] | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Δ=TBD. |  |  |  |  |  |  |

<End of Change 1>