3GPP TSG-RAN4 Meeting #102-e	R4-2205366

# Electronic Meeting, 21 February – 3 March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to introduce EMR TC#5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In R4-2105841, it was agreed to introduce 5 TCs for EMR, but TC#5 has not been introduced yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce EMR TC#5 as agreed in R4-2105841. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC for EMR is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8.2.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.8.2.2.2	E-UTRA – NR Early Measruement Reporting for NR in FR2

A.8.2.2.2.1	Test Purpose and Environment

This test is to verify the requirement for the E-UTRAN to NR inter-RAT Idle mode DC measurement requirements specified in clause 4.9.2.4 in TS 36.133 [15]. This test is also to verify the accuracy requirement for the E-UTRAN to NR inter-RAT Idle mode DC measurement requirements specified in clause 9.11.1A and 9.11.2A in TS 36.133 [15]. Supported test configurations are shown in Table A.8.2.2.2.1-1.

Table A.8.2.2.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

The test scenario comprises of 1 E-UTRA cell (Cell 1) and 1 NR cell (Cell 2). The the test parameters and applicability for the E-UTRAN cell are defined in Table A.8.2.2.2.1-4. The general test parameters and the cell specific test parameters for the NR cell are speficied in Table A.8.2.2.2.1-2 and Table A.8.2.2.2.1-3, respectively.

The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Prior to the start of the time duration T1, the UE shall be connected to Cell 1. During T1, Cell 2 shall be powered off. At the end of T1, the RRC connection to Cell 1 is released and UE is configured Idle mode DC measurement on the carrier frequency of Cell 2. Time duration T2 starts when the RRC connection is released, and during the T2 UE is in Idle mode. Cell 2 shall be powered on from the beginning of T2. At beginning of T3 the UE is paged for connection setup and requested by the network to send idle mode measurements.

Table A.8.2.2.2.1-2: General test parameters

| Parameter | Unit | Test | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  | configuration |  |  |
| Active cell |  | 1, 2 | E-UTRAN Cell 1 |  |
| Neighbour cell |  | 1, 2 | NR Cell 2 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 2: Cell 2 |  |
| DRX cycle length | s | 1, 2 | 1.28 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  |
| T1 | s | 1, 2 | 0.5 |  |
| T2 | s | 1, 2 | 128 |  |
| T3 | s | 1, 2 | 2 |  |
| T331 | s | 1, 2 | 300 |  |

Table A.8.2.2.2.1-3: Cell specific test parameters for NR cell 2

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | TDDConf.3.1 |  |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.3.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.3.1 TDD |  |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.3.1 TDD |  |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| SSB configuration |  | 1, 4 | SSB.1 FR2 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  |
|  | dB | 1, 4 | -infinity | 4 | 4 |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |
|  | dB | 1, 4 | -infinity | -4 | -4 |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -infinity | -102 | -102 |
| SS-RSRQ Note3 | dB | 1, 4 | -infinity | -16.25 | -16.25 |
| Io | dBm/95.04MHz | 1, 4 | -69.01 | -67.56 | -67.56 |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.2.2.2.1-4: Cell specific test parameters for E-UTRA cell 1

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -84 |
| RSRQ Note 3 | dB | -10.96 | -10.96 | -10.96 |
|  | dB | 14 | 14 | 14 |
|  | dB | 14 | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | N/A |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| beamMeasConfigIdle |  | False |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

A.8.2.2.2.2	Test Requirements

At the beginning of the time-period T2 the connection is released, and UE enters idle mode. During the time period T2 the UE is in Idle mode and Cell 2 is active. The UE shall not perform reselection. The UE shall perform Idle Mode DC measurement according to clause 4.9.2.4 in TS 36.133 [15]. UE shall be able to detect, acqure the SSB index and measure the SS-RSRP and SS-RSRQ from Cell 2 for Idle mode DC measurement during T2.

NOTE:	The Idle mode DC measurement period for the test setup can be expressed as: Tdetect, NR.

Where:

Tdetect, NR	See Table 4.2.2.5.6-1 in clause 4.2.2.5.6 in TS 36.133 [15]

This gives a total of 128 s, allow 128 s for the T2.

At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in Section 9.1.2B in TS 36.133 [15] and Section 9.1.3B, respectively and test ends.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 1>