3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2205421

Electronic Meeting, Feb-21 – March 3, 2022

Source:	Ericsson

Title:	Reply LS to RAN1: LS on NTN UL time and frequency synchronization requirements (Timing)

Agenda item:	10.13.5.4

Document for:	Approval

# 1	Introduction

An LS was sent from RAN1 to RAN4 regarding NTN UL time synchronization requirements and NTN UL frequency synchronization requirements (R1-2102263). In this contribution we discuss question 1, the NTN UL time synchronization requirements.

1. Overall Description: As part of the WI NR-NTN-solution, under agenda item 8.4.2, RAN1 discussed the NTN UL synchronization requirements in terms of time alignment and frequency error for: • Initial access (i.e. PRACH transmission) • UL transmissions in RRC CONNECTED State. RAN1 identified the following questions that need clarification from RAN4: Question 1: What are the NTN UL time synchronization requirements? - For initial access (i.e. PRACH transmission) - For UL transmissions in RRC Connected State Question 2: What are the NTN UL frequency synchronization requirements? - For initial access (i.e. PRACH transmission) - For UL transmissions in RRC Connected State Note-1: The questions above are related to the requirements for signal reception at gNB side. Note-2: From RAN1 side we would like to have the requirements such that they are including the ”total accumulated error” in both time and frequency domain. 2. Actions: To TSG RAN WG4 ACTION 1: RAN1 respectfully asks RAN4 to provide feedback on question 1 and any additional information that may help RAN1 understand the NTN requirement of UL time synchronization. ACTION 2: RAN1 respectfully asks RAN4 to provide feedback on question 2 and any additional information that may help RAN1 understand the NTN requirement of UL frequency synchronization 3. Date of Next TSG-RAN WG1 Meetings: TSG-RAN WG1 Meeting #104-bis-e 12 Apr. – 20 Apr. 2021 Electronic Meeting TSG-RAN WG1 Meeting #105-e 19 – 27 May 2021 Electronic Meeting

# 1	Draft reply LS

Title:	Reply LS to RAN4 on NTN UL time and frequency synchronization requirements (Timing)

Response to:

Release:	Rel-17

Work Item:	Solutions for NR to support non-terrestrial networks

Source:	RAN4

To:	RAN1

Cc:

Contact Person:
          Name:	Magnus Larsson
          E-mail Address:	magnus.k.larsson (at) ericsson (dot) com

RAN4 would like to thank RAN1 for the LS on NTN UL time and frequency synchronization requirement for initial access (i.e. PRACH transmission) and UL transmissions in RRC_CONNECTED state (R1-2102263). Based on the last reply LS (R4-2115347, R4-2120311), RAN4 would like to provide the further information on UE initial transmit timing error requirement as follows:

Question 1: What are the NTN UL time synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

[RAN4]: The UL timing requirements for NTN UE will be specified in RAN4 are summarized as follows:

For UL transmissions in RRC_CONNECTED state

#### Gradual timing adjustment for NTN:

Requirements in this section shall apply regardless of whether the reference cell is on a carrier frequency subject to CCA or not.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing, for NTA+NTA,offset shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1.

Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

All adjustments made to the UE uplink timing, for NTA,UE-specific shall follow this rule:

1)	The UE GNSS position accuracy is ±50 meters from true position.

TA adjustment accuracy requirement:

UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |

# 2. Actions:

To TSG RAN WG1

ACTION: 	RAN4 respectfully ask RAN1 to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #103-e 	16-27 May 2022

TSG-RAN WG4 Meeting #104 	22-16 August 2022