3GPP TSG-RAN WG4 Meeting #102-e	R4-2205504

Electronic Meeting, Feb. 21- Mar. 3, 2022

Agenda Item:	10.12.2.1.2

Source:	Ericsson

Title:	Remaining issues on CSI reporting requirements for inter-cell interference

Document for:	Discussion

# 1	Introduction

In last meeting, RAN4 had discussed UE demodulation requirements of MMSE-IRC receiver for suppressing inter-cell interference. The WF for CSI reporting requirements for inter-cell interference MMSE-IRC is agreed in [1]. In this contribution, we will continue to discuss the remaining open issues on CSI reporting requirements with the MMSE-IRC receiver with inter-cell interference scenario.

# 2	Test Metric

The most important issue for CQI reporting test is to define the related test metric. In last meeting, the agreements are attached as follow.

Issue 2-2-2: SINR and T-put gain for 2 Rx The final value shall be deduced based on the simulation. Issue 2-2-3: BLER for 2 Rx - Option 1: 0.02 - Option 2: 0.01 Note: Other options are not precluded. Issue 2-2-3: SINR and T-put gain for 4 Rx The final value shall be deduced based on the simulation. Issue 2-2-4: BLER for 4 Rx - Option 1: 0.02 - Option 2: 0.01 Note: Other options are not precluded.

As mentioned in the agreements, the relative throughput ratio γ for LTE CQI reporting in ICI will be re-used which is defined as follow.

γ= Tfollow with interfTfollow with AWGN

Based on our simulation results[2], the relative throughput ratio is around 1 for MRC/MRC as expected since there is no gain when only MRC receiver is applied. The gain is achieved by the IRC/IRC from the UE demodulation part which can be seen in our simulation results. In legacy LTE CQI reporting, the relative throughput ratio γ is defined as 1.8 in SINR =-2dB to differentiate the different UE implementations. Based on our observations for different simulation results, more relative throughput ratio difference happens in a low SINR. Therefore, we suggest reusing the same side conditions as legacy LTE for SINR=-2dB. Meanwhile, from our simulation, RAN4 to define γ equaling 2 will introduce enough gain from IRC demod and CQI receiver compared with MRC receiver.

Proposal 1: RAN4 to define CQI reporting test metric with SINR=-2dB for both 2Rx and 4Rx.

Proposal 2: RAN4 to define CQI reporting test metric with γ=2 for both 2Rx and 4Rx.

MRC/IRC combination may result in a BLER close to 0 since MRC CSI reporting will give a much pessimistic reporting which causes a much lower CQI and results in a zero BLER. Therefore, it is also necessary to introduce an additional requirement on the minimum BLER to prevent under-reporting. Based on our simulation, it is proposed to define the BLER metric shall be greater than or equal to 2%.

Proposal 3: RAN4 to introduce the BLER metric such as BLER > 2% for both 2Rx and 4Rx.

# 4	Summary

In this contribution, we further discuss the remaining issues on CSI reporting of MMSE-IRC receiver.

Proposal 1: RAN4 to define CQI reporting test metric with SINR=-2dB for both 2Rx and 4Rx.

Proposal 2: RAN4 to define CQI reporting test metric with γ=2 for both 2Rx and 4Rx.

Proposal 3: RAN4 to introduce the BLER metric such as BLER > 2% for both 2Rx and 4Rx.

# References

- R4-2203009, “WF on CSI requirements for inter-cell interference MMSE-IRC”, Ericsson

- R4-2201218, “Simulation results on CSI reporting for inter-cell interference”, Ericsson