3GPP TSG-RAN4 Meeting # 102-e	R4-2205534

# Electronic Meeting,  21st Feb  – 3rd March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | <CR#> | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on DMRS bundling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-2-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE RF requirmeent on DMRS bundling is introduced in Rel-17 in NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inroduce the annex F.8 on the DMRS measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No UE RF requirmeent on DMRS bundling in specificaitons. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.8, F.8.1, F8.2, F.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < start of changes >

# F.8	Phase offset measurement for DMRS bundling

The measurement point for phase offset measurement is defined in Figure F.8-1.

Figure F.8-1: measurement point for phase offset for DMRS bundling

## F.8.1	Modified test signal

The post-FFT modulated signal before the equalization is modified according to:

Z''(t,f)=FFTz(v-Δt)⋅e-j2πΔfv.ej2πfΔt

where

z(v) is the time domain samples of the signal under test within the bundled time slots.

To minimize the error, the signal under test should be modified with respect to a set of parameters following the procedure explained below.

Notation:

Δt is the sample timing difference between the FFT processing window in relation to nominal timing of the ideal signal.

Δf is the RF frequency offset.

To minimize the error, the EVM window estimation is reused for the phase difference measurement for DMRS bundling.

In the following Δc represents the middle sample of the EVM window of length W or the last sample of the first window half if Wis even.

The test equipment shall

-	detect the start of each slot and estimate Δt and Δf,

-	determine Δc so that the EVM window of length W is centred

-	on the time interval determined by the measured cyclic prefix minus 16κ samples of the considered OFDM symbol for symbol l for subcarrier spacing configuration µ in a subframe, with l = 0 or l = 7*2^µ for normal CP, i.e. the first 16κ samples of the CP should not be taken into account for this step. In the determination of the number of excluded samples, a sampling rate of 1/Tc is assumed. If a different sampling rate is used, the number of excluded samples is scaled linearly.

-	on the measured cyclic prefix of the considered OFDM symbol symbol for all other symbols for normal CP and for symbol 0 to 11 for extended CP.

To determine the other parameters a sample timing offset equal to Δc is corrected from the signal under test. The test equipment shall then

-	correct the RF frequency offset Δffor each time slot within the bundled time slots, and

-	apply an FFT of appropriate size. The chosen FFT size shall ensure that in the case of an ideal signal under test, there is no measured inter-subcarrier interference.

The carrier leakage shall be removed from the evaluated signal; however, the removed relative carrier leakage power also has to satisfy the applicable requirement.

At this stage estimates of Δf, a(t,f), ϕ(t,f) and Δc are available. Δt is one of the extremities of the window W, i.e. Δtcan be Δc+α-W2 or Δc+W2, where α=0 if W is odd and α=1 if Wis even. The test equipment  shall then

-	calculate PhaseOffsetl with Δt set to Δc+α-W2,

-	calculate PhaseOffseth with Δt set to Δc+W2.

## F.8.2	Frequency response of the TX chain

Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal Z''(t,f) and the post-FFT ideal reference signal I(t,f), for each reference signal, over one measurement interval of one time slot. This process creates a set of complex ratios of the channel coefficient:

H(t,f)=α(t,f)⋅ejφ(t,f)=Z'(t,f)I(t,f)

Where the post-FFT ideal signal I(t,f), is DMRS reference signal within one time slot.

- Perform time averaging of the phase φt,f at each reference signal subcarrier of the channel coefficient, the time-averaging length is total number of N DMRS symbols over one time slot.

- φf=tφt,f N=ttan-1Im (Ht,f)Re (Ht,f) N

Re (H(t,f)) is the real part of the complex-valued H (t,f) and Im (H(t,f)) is the imaginary part of the the complex-valued H (t,f).

## F.8.3	Phase offset measurement

For the reference time slot Tr, the phase of complex-valued channel coefficient is calculated as φf in Annex F.9.2 with channel coefficient Hφ(t,f).

For the time slot Ti, the phase of complex-valued channel coefficient is calculated as θf in Annex F.9.2 with channel coefficient Hθ(t,f) .

For the phase offset measurement between this time slot and the reference time slot, the PhaseOffset (Ti) is defined as below:

PhaseOffset Ti= maxf(φf- θf)=maxf(ttan-1Im (Hθ(t,f)∙Hφt,f*)Re(Hθ(t,f)∙Hφt,f*)N)

P

The phase offset measurement of PhaseOffset (Ti) is selected as maximum absolute value over all DMRS subcarriers and averaged over N DMRS symbols in one time slot. Hφt,f* is the complex conjugate of Hφt,f.

The measurement should be done within all bundled time slot except the reference time slot and repeated over [10] bundles, the rms value of phase offset should be calculated with below equation assuming the total number of measurement sample is L.

PhaseOffsetrms= 1LPhaseOffset Ti2L

Finally, the PhaseOffset measurement shall be tested against the maximum of the RMS average at the window W extremities of the measurements:

Thus PhaseOffsetl is calculated using in the expressions above and PhaseOffsethis calculated using .

Thus we get:

PhaseOffset=max(PhaseOffsetl ,PhaseOffseth)

## << Unchanged part is omitted>>

## < end of changes >