3GPP TSG-RAN4 Meeting # 102-e	R4-2205540

# Electronic Meeting,  21st Feb  – 3rd March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RedCap UE FR1-RX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2022-2-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Redcap is introduced in Rel-17 in NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce REFSESN requirments and receiver requirement for RedCap UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No RedCap UE specificaion in specificaitons. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.1I, 7.2, 7.3I |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < start of changes >

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

DRB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

ΔRHD-FDD	Reference sensitivity adjustment due to support for HD-FDD operation

ΔShift	Channel raster offset

DTC	Allowed operating band edge transmission power relaxation

DTC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max( BWGB,Channel(k) )

BWGB,Channel(k)	Minimum guard band defined in clause 5.3A.1 of carrier k

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC,low	The Fc of the lowest carrier, expressed in MHz

FC,high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge , low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF, shift	RF reference frequency for Supplementary Uplink (SUL) bands, the uplink of all FDD bands, and TDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

GBChannel	Minimum guard band defined in clause 5.3.3

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRBj*2μj for carrier 1 to j, where μ is defined in TS 38.211 [6]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211 [6]

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	The nominal UE power (i.e., no tolerance)

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart	The lowest RB index of transmitted resource blocks

RBstart_CA	The lowest RB index of transmitted resource blocks for intra-band contiguous CA

SCSc	SCS for the component carrier c

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

SCSlow	SCS for the lowest assigned component carrier in clause 5.3A.1

SCShigh	SCS for the highest assigned component carrier in clause 5.3A.1

tp		Transient Period value signalled by the UE

tpstart		Start position of transient period relative to the symbol boundary

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in clause 6.2.1

SSREF	SS block reference frequency position

UTRAACLR	UTRA ACLR

## << Unchanged part is omitted>>

## 7.1I	General

For a Redcap UE the requirements in Section 7 shall be verified with the channel bandwidth according to clause 5.2I.

## << Unchanged part is omitted>>

## 7.2	Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination.

For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports.

For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.

The above rules apply for all clauses with the exception of clause 7.9.

For a Redcap UE the requirements in Section 7 assume that the receiver is equipped with a minimum of single Rx antenna port.

## << Unchanged part is omitted>>

## 7.3I	Reference sensitivity for RedCap

### 7.3I.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3I.2	Reference sensitivity power level

For a RedCap UE equipped with 2 Rx antenna ports operating in single band in the bands specified in clause 5.2I, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b.

For a RedCap UE equipped with 1 Rx antenna ports operating in single band in the bands specified in clause 5.2I, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,1R in Table 7.3I.2-1 for the applicable operating bands.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth | ΔR1R (dB) |
| --- | --- | --- |
| TDD band according to clause 5.2I | According to clause 5.3.5I | 2,5 |
| FDD band according to clause 5.2I | 5MHz | 2.5 |
| FDD band according to clause 5.2I | 10MHz, 15MHz and 20MHz | 3 |

For a RedCap UE equipped with 2 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3I.2-3 shall be met with ΔRHD-FDD specified in Table 7.3I.2-2.

Table 7.3I.2-2: HD-FDD RedCap UE reference sensitivity allowance ΔRHD-FDD

| Operating band | Channel bandwidth | ΔRHD-FDD (dB) |
| --- | --- | --- |
| n1, n18, n24, n70 | 5MHz | [0] |
| n30, n65, n66, n74 | 5MHz | -[0.5] |
| n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 | 5MHz | -[0.8] |

Table 7.3I.2-3: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |
| --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) |
| n1 | 15 | 5,10,15, 20 | -100 + ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15, 20 | -100 + ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15, 20 | -100 + ΔRHD-FDD +10log10(4*NRB/25) |
| n2 | 15 | 5,10,15,20 | -98+ ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15,20 | -98+ ΔRHD-FDD + 10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -98+ ΔRHD-FDD + 10log10(4*NRB/25) |
| n3 | 15 | 5,10,15,20 | -97.0+ ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.0+ ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -97.0+ ΔRHD-FDD +10log10(4*NRB/25) |
| n5 | 15 | 5,10,15,20 | -98.0+ ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15,20 | -98.0+ ΔRHD-FDD +10log10(2*NRB/25) |
| n7 | 15 | 5,10,15,20 | -98.0+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -98.0+ ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -98.0+ ΔRHD-FDD +10log10(4*NRB/25) |
| n8 | 15 | 5,10,15,20 | -97.0+ ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.0+ ΔRHD-FDD +10log10(2*NRB/25) |
| n12 | 15 | 5,10,15 | -97.0+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15 | -97.0+ ΔRHD-FDD +10log10(2*NRB/25) |
| n13 | 15 | 5,10 | -97.0+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10 | -97.0+ ΔRHD-FDD +10log10(2*NRB/25) |
| n14 | 15 | 5,10 | -97.0+ ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10 | -97.0+ ΔRHD-FDD +10log10(2*NRB/25) |
| n18 | 15 | 5,10,15 | -100 + ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15 | -100 + ΔRHD-FDD +10log10(2*NRB/25) |
| n20 | 15 | 5,10,15,20 | -97.0+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.0+ ΔRHD-FDD +10log10(2*NRB/25) |
| n24 | 15 | 5,10 | -100 + ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10 | -100 + ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10 | -100 + ΔRHD-FDD +10log10(4*NRB/25) |
| n25 | 15 | 5,10,15,20 | -96.5+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -96.5+ ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -96.5+ ΔRHD-FDD +10log10(4*NRB/25) |
| n26 | 15 | 5,10,15,20 | -97.5+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.5+ ΔRHD-FDD +10log10(2*NRB/25) |
| n28 | 15 | 5,10,15,20 | -98.5+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -98.5+ ΔRHD-FDD +10log10(2*NRB/25) |
| n30 | 15 | 5,10 | -99.0+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10 | -99.0+ ΔRHD-FDD +10log10(2*NRB/25) |
| n65 | 15 | 5,10,15,20 | -99.5+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -99.5+ ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -99.5+ ΔRHD-FDD +10log10(4*NRB/25) |
| n66 | 15 | 5,10,15,20 | -99.5+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -99.5+ ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -99.5+ ΔRHD-FDD +10log10(4*NRB/25) |
| n70 | 15 | 5,10,15,20 | -100 + ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -100 + ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -100 + ΔRHD-FDD +10log10(4*NRB/25) |
| n71 | 15 | 5,10,15,20 | -97.2+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.2+ ΔRHD-FDD +10log10(2*NRB/25) |
| n74 | 15 | 5,10,15,20 | -99.5+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -99.5+ ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -99.5+ ΔRHD-FDD +10log10(4*NRB/25) |
| n85 | 15 | 5,10,15 | -97.0+ ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15 | -97.0+ ΔRHD-FDD +10log10(2*NRB/25) |

For a RedCap UE equipped with 1 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 1Rx antenna ports in Table 7.3I.2-4 shall be met with ΔRHD-FDD specified in Table 7.3I.2-2 and ΔR1R for 5MHz channel specified in Table 7.3I.2-1 for the applicable operating bands.

Table 7.3I.2-4: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |
| --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) |
| n1 | 15 | 5,10,15, 20 | -100 + ΔR1R + ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15, 20 | -100 + ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15, 20 | -100 + ΔR1R +ΔRHD-FDD +10log10(4*NRB/25) |
| n2 | 15 | 5,10,15,20 | -98+ ΔR1R +ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15,20 | -98+ ΔR1R +ΔRHD-FDD + 10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -98+ ΔR1R +ΔRHD-FDD + 10log10(4*NRB/25) |
| n3 | 15 | 5,10,15,20 | -97.0+ ΔR1R +ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(4*NRB/25) |
| n5 | 15 | 5,10,15,20 | -98.0+ ΔR1R +ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15,20 | -98.0+ ΔR1R + ΔRHD-FDD +10log10(2*NRB/25) |
| n7 | 15 | 5,10,15,20 | -98.0+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -98.0+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -98.0+ ΔR1R +ΔRHD-FDD +10log10(4*NRB/25) |
| n8 | 15 | 5,10,15,20 | -97.0+ ΔR1R + ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n12 | 15 | 5,10,15 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n13 | 15 | 5,10 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n14 | 15 | 5,10 | -97.0+ ΔR1R +ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n18 | 15 | 5,10,15 | -100 + ΔR1R +ΔRHD-FDD + 10log10(NRB/25) |
|  | 30 | 10,15 | -100 + ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n20 | 15 | 5,10,15,20 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n24 | 15 | 5,10 | -100 + ΔR1R + ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10 | -100 + ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10 | -100 + ΔR1R +ΔRHD-FDD +10log10(4*NRB/25) |
| n25 | 15 | 5,10,15,20 | -96.5+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -96.5+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -96.5+ ΔR1R +ΔRHD-FDD +10log10(4*NRB/25) |
| n26 | 15 | 5,10,15,20 | -97.5+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.5+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n28 | 15 | 5,10,15,20 | -98.5+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -98.5+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n30 | 15 | 5,10 | -99.0+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10 | -99.0+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n65 | 15 | 5,10,15,20 | -99.5+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -99.5+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -99.5+ ΔR1R +ΔRHD-FDD +10log10(4*NRB/25) |
| n66 | 15 | 5,10,15,20 | -99.5+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -99.5+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -99.5+ ΔR1R +ΔRHD-FDD +10log10(4*NRB/25) |
| n70 | 15 | 5,10,15,20 | -100 + ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -100 + ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -100 + ΔR1R +ΔRHD-FDD +10log10(4*NRB/25) |
| n71 | 15 | 5,10,15,20 | -97.2+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -97.2+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
| n74 | 15 | 5,10,15,20 | -99.5+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15,20 | -99.5+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |
|  | 60 | 10,15,20 | -99.5+ ΔR1R +ΔRHD-FDD +10log10(4*NRB/25) |
| n85 | 15 | 5,10,15 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(NRB/25) |
|  | 30 | 10,15 | -97.0+ ΔR1R +ΔRHD-FDD +10log10(2*NRB/25) |

The reference receive sensitivity (REFSENS) requirement specified for a RedCap UE shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

## < end of changes >