3GPP TSG-RAN WG4 Meeting # 102-e	R4-22xxxxx

# Electronic Meeting, Feb 21st -Mar 3rd ,2022

Title: 	Discussion on UE demodulation requirements for additional enhancements of eMTC

Source: 	Huawei, HiSilicon

Agenda item:       12.9.6.3

Document for:	Discussion

Background

As per [1], we provide our views on open issues for Rel-17 additional enhancements of eMTC

Summary of RAN 1’s feature

Whether to define performance requirements to verify UE supporting 14 HARQ processes

Based on our understanding, HARQ number has big impact on soft buffer size which is related to demodulation performance. It should be mentioned that demodulation requirements for 2 HARQ processes for NB Cat 2 UE has also been defined in Rel-14. Follow the same logic, we think 14 HARQ processes should also been verified.

Whether to define performance requirements to verify UE supporting 1736 bits TBS and corresponding soft buffer

Same as increased HARQ processes number, TBS also has impact on soft buffer which is related to demodulation performance. We support to define one case to verify this feature,

Proposal 1: Define performance requirements to verify UE supporting 14 HARQ processes and UE supporting 1736 bits TBS and corresponding soft buffer

Discussions on UE performance requirements

14 HARQ processes reception test

RAN 1 has defined the scheduling pattern in Figure 1 for 14 HARQ processes scenario assuming there is no invalid BL/CE subframe. In this pattern, scheduling period is 17ms. In every period, 0th to 11th subframes are used for MPDCCH/PDSCH transmission, subframes from 13th to 15th are used for PUCCH transmission and 12th and 16th subframes are used for gaps. For scheduling delay, 2 is defined for PDSCH transmitted in subframes from 0th to 9th and 7 is defined for PDSCHs transmitted in subframe 10th  and 11th . As for HARQ delay, ACK/NACKs related to PDSCHs in subframe from 0th to 3th are transmitted in 13th subframe, ACK/NACKs related to PDSCHs in subframe from 4th to 7th are transmitted in 14th subframe and ACK/NACKs related to PDSCHs in subframe from 8h to 11th are transmitted in 15th subframe.  Based on our understanding, RAN 4 can reuse this pattern for demodulation requirements definition.

Figure 1: Scheduling pattern for 14 HARQ processes scenario

In previous release, TM2, TM6 and TM9 were used. For Rel-17, we propose to select only one transmission mode. e.g. TM2 for requirements definition. For other assumptions, test 2 in Table 8.11.1.1.3.1-1 and 8.11.1.1.3.1-2 can be considered as baseline. Therefore, we propose the following simulation assumptions:

  - HD-FDD

  - Bandwidth: 10MHz

  - MCS: 16QAM 1/2

  - TBS: 744

  - CE mode A

  - No repetition for MPDCCH and PDSCH

  - Propagation condition: EPA5

  - MIMO configuration: 2x1 Low

  - HARQ processes: 14

  - HARQ bundling: enabled

Proposal 2: For 14 HARQ processes test, use following simulations:

  - For scheduling pattern, scheduling period is 17ms.

- In every period, 0th to 11th subframes are used for MPDCCH/PDSCH transmission, subframes from 13th to 15th are used for PUCCH transmission and 12th and 16th subframes are used for gaps.

- For scheduling delay, 2 is defined for PDSCH transmitted in subframes from 0th to 9th and 7 is defined for PDSCHs transmitted in subframe 10th  and 11th

- As for HARQ delay, ACK/NACKs related to PDSCHs in subframe from 0th to 3th are transmitted in 13th subframe, ACK/NACKs related to PDSCHs in subframe from 4th to 7th are transmitted in 14th subframe and ACK/NACKs related to PDSCHs in subframe from 8h to 11th are transmitted in 15th subframe.

  - TM2

  - HD-FDD

  - Bandwidth: 10MHz

  - MCS: 16QAM 1/2

  - TBS: 744

  - CE mode A

  - No repetition for MPDCCH and PDSCH

  - Propagation condition: EPA5

  - MIMO configuration: 2x1 Low

  - HARQ transmissions: 14

  - HARQ bundling: enabled

Maximum TBS and soft buffer test

RAN 1 has defined the maximum TBS equals to 1736 bits and corresponding soft buffer size equals to 43008 bits. Based on our understanding, we should define one case to verify both features.  Therefore, in addition to set TBS to 1736 bits, to verify the maximum of soft buffer size, the channel bits per transmission and HARQ transmissions per TB should be as much as possible, therefore, the test metric we propose is 30% instead of 70% of maximum TP to make more HARQ transmissions per TB, if UE doesn’t have enough HARQ buffer size to store, the performance will degrade severely. For other simulation assumptions, we propose to use following assumptions:

- EPA5

- CE mode A

- TBS: 1736 bits

- 64 QAM

- HD-FDD

- 8 HARQ processes

- NRB=6

- HARQ transmission number: 4

- No repetition for MPDCCH and PDSCH

- 64QAM

- Transmission mode: TM2

Proposal 3: For maximum TBS and soft buffer size test, use 30% instead of 70% of maximum TP to let more HARQ transmissions per TB to verify the soft buffer size.

Proposal 4: For maximum TBS and soft buffer size test, use following assumptions:

- EPA5

- CE mode A

- TBS: 1736 bits

- 64 QAM

- HD-FDD

- 8 HARQ processes

- NRB=6

- HARQ transmission number: 4

- No repetition for MPDCCH and PDSCH

- 64QAM

- Transmission mode: TM2

Conclusion

In this paper, we provide our views on open issues for Rel-17 additional enhancements of eMTC.The proposals are:

Proposal 1: Define performance requirements to verify UE supporting 14 HARQ processes and UE supporting 1736 bits TBS and corresponding soft buffer

Proposal 2: For 14 HARQ processes test, use following simulations:

  - For scheduling pattern, scheduling period is 17ms.

- In every period, 0th to 11th subframes are used for MPDCCH/PDSCH transmission, subframes from 13th to 15th are used for PUCCH transmission and 12th and 16th subframes are used for gaps.

- For scheduling delay, 2 is defined for PDSCH transmitted in subframes from 0th to 9th and 7 is defined for PDSCHs transmitted in subframe 10th  and 11th

- As for HARQ delay, ACK/NACKs related to PDSCHs in subframe from 0th to 3th are transmitted in 13th subframe, ACK/NACKs related to PDSCHs in subframe from 4th to 7th are transmitted in 14th subframe and ACK/NACKs related to PDSCHs in subframe from 8h to 11th are transmitted in 15th subframe.

  - TM2

  - HD-FDD

  - Bandwidth: 10MHz

  - MCS: 16QAM 1/2

  - TBS: 744

  - CE mode A

  - No repetition for MPDCCH and PDSCH

  - Propagation condition: EPA5

  - MIMO configuration: 2x1 Low

  - HARQ transmissions: 4

  - HARQ bundling: enabled

Proposal 3: For maximum TBS and soft buffer size test, use 30% instead of 70% of maximum TP to let more HARQ transmissions per TB to verify the soft buffer size.

Proposal 4: For maximum TBS and soft buffer size test, use following assumptions:

- EPA5

- CE mode A

- TBS: 1736 bits

- 64 QAM

- HD-FDD

- 8 HARQ processes

- NRB=6

- HARQ transmission number: 4

- No repetition for MPDCCH and PDSCH

- 64QAM

- Transmission mode: TM2