3GPP TSG-RAN WG4 Meeting #102-e	R4-2205814

Electronic Meeting, February 21st – March 3rd, 2022

Source:				Rohde & Schwarz, vivo

Title:					TP to TR 38.834 on Measurement distance

Agenda Item:		10.2.2.1

Document for:	Approval

# Introduction

This paper provides changes to the TRP and TRS test setup description in TR 38.834 [1] to define the minimum range length based on the approach agreed for RTS in clause 6.6.2 of TR 38.827 [2].

Proposal 1: Approve the text proposal below.

# Conclusion

The following proposal is made in this contribution:

Proposal 1: Approve the text proposal below.

# References

- 3GPP TR 38.834 v0.3.0 (2022-01)

- 3GPP TR 38.827 v16.5.0 (2022-01)

# Appendix – Text Proposal to TR 38.834 v0.3.0

< Unchanged Text Deleted >

< Beginning of Changes >

## 7.1	General

### 7.1.1	Minimum Range Length

This sub-section specifies the minimum range lengths for NR FR1 TRP-TRS OTA systems. The range length is defined as the distance from the centre of the quiet zone to the aperture of the measurement probes/antennas, as illustrated in Figure 7.1.1-1.

Figure 7.1.1-1: Illustration of range length definition

The minimum range length shall be the maximum of the following three limits

-	The phase uncertainty limit: RQZ+2Drad2/

-	The amplitude uncertainty limit: 3D

-	The reactive Near-Field limit: RQZ+2

where RQZ is defined as the radius of the quiet zone, i.e., RQZ=D/2, and Drad is the diameter of the effective radiating aperture. The minimum range length calculations for D=30cm quiet zone size TRP-TRS OTA test systems shall assume that Drad is 30cm below 1.5GHz and decrease linearly from 30cm to 5cm from 1.5GHz to 7.125GHz, respectively. The last column of Table 7.1.1-1 shall be considered the minimum range length for NR FR1 TRP-TRS OTA systems with 30cm quiet zone size.

Table 7.1.1-1: Minimum Range Length for NR FR1 TRP-TRS OTA systems with 30cm quiet zone size.

| f [GHz] | Drad [m] | RQZ+2Drad²/λ | 3D = 6RQZ | RQZ+2λ | max(RQZ+2λ,3D,RQZ+2D²/λ) |
| --- | --- | --- | --- | --- | --- |
| 0.41 | 0.30 | 0.40 | 0.9 | 1.61 | 1.61 |
| 0.6 | 0.30 | 0.51 | 0.9 | 1.15 | 1.15 |
| 0.7 | 0.30 | 0.57 | 0.9 | 1.01 | 1.01 |
| 0.8 | 0.30 | 0.63 | 0.9 | 0.90 | 0.90 |
| 1 | 0.30 | 0.75 | 0.9 | 0.75 | 0.90 |
| 1.2 | 0.30 | 0.87 | 0.9 | 0.65 | 0.90 |
| 1.4 | 0.30 | 0.99 | 0.9 | 0.58 | 0.99 |
| 1.6 | 0.30 | 1.08 | 0.9 | 0.52 | 1.08 |
| 1.8 | 0.29 | 1.14 | 0.9 | 0.48 | 1.14 |
| 2 | 0.28 | 1.18 | 0.9 | 0.45 | 1.18 |
| 2.2 | 0.27 | 1.21 | 0.9 | 0.42 | 1.21 |
| 2.4 | 0.26 | 1.23 | 0.9 | 0.40 | 1.23 |
| 2.6 | 0.25 | 1.24 | 0.9 | 0.38 | 1.24 |
| 2.8 | 0.24 | 1.25 | 0.9 | 0.36 | 1.25 |
| 3 | 0.23 | 1.24 | 0.9 | 0.35 | 1.24 |
| 4 | 0.19 | 1.10 | 0.9 | 0.30 | 1.10 |
| 5 | 0.14 | 0.85 | 0.9 | 0.27 | 0.90 |
| 6 | 0.10 | 0.55 | 0.9 | 0.25 | 0.90 |
| 7 | 0.06 | 0.29 | 0.9 | 0.24 | 0.90 |
| 7.125 | 0.05 | 0.27 | 0.9 | 0.23 | 0.90 |

< End of Changes >