3GPP TSG RAN WG4 Meeting #102-e	R4-2205837

# E-meeting, 21 Feb. 2022 – 03 Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Interruption requirement to NR serving cell, and impacts to other NR RRM requirement (if applicable) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 introduced requirements for SRS antenna port switching feature in Rel-17 and the corresponding Interrption requirements on NR serving cells are need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of interruption requirements to NR serving cell due to SRS antenna port switching, and impacts to other NR RRM requirement (if applicable) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirements to NR serving cell due to SRS antenna port switching will be missing in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.18, 8.2.2.2.16, 8.2.3.2.16, 8.2.4.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

##### 8.2.1.2.18	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

SRS transmission from UE helps gNB in acquisition of the DL CSI (full channel) at the gNB. When UE has a higher number of RX antenna compared to TX antenna, UE sounds the SRS on all the RX antenna with the help of SRS antenna port switching to help gNB acquire full channel. This feature is mandatory with capability signalling, “SRS-TxPortSwitch”. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas. The requirements in this clause applicable for different SRS antenna switch patterns.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

-     the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD.

-	 the SRS switching is not colliding with E-UTRA measurement.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx.

Interruption requirements on the victim CC are defined for following two scenarios for sync and async cases.

Scenario 1: 1 SRS symbol is transmitted

Scenario 2: 6 symbols (SRS and guard symbols including)

Interruption requirements are defined in symbol level for scenario 1 sync case. For other following scenarios, interruption requirements are defined in slot level

- Scenario 1 async case

- Scenario 2 async case and sync case

Interruption requirement for scenario 1 and sync case:

For scenario 1, number of SRS is 1 OFDM symbol. MRTD assumed for derving the interruption length is 33us. The interruption length for different SCS or aggressor and victim CC are provided in table 8.2.1.2.18-1.

Table 8.2.1.2.18-1: Interruption length in symbols for scenario 1 and sync case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 3 | 3 | 3 |
| 30 | 6 | 5 | 3 |
| 60 | 10 | 8 | 7 |
| 120 | 20 | 14 | 14 |

Interruption requirement for scenario 1 and async case:

For scenario 1 number of SRS is 1 OFDM symbol. UL TA is assumed to introduce one additional slot interruption on victim CC. The interruption length for different SCS of aggressor and victim CC are provided in table 8.2.1.2.18-2.

Table 8.2.1.2.18-2: Interruption length in slots for scenario 1 and async case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Interruption requirement for scenario 2 sync and async cases:

For scenario 2, number of SRS and guard period combined is assumed as 6 OFDM symbols for deriving the requirements. UL TA is assumed to introduce additional 1 slot to interruption requirement. The interruption length for different SCS of aggressor and victim CC are provided in table 8.2.1.2.18-3.

Table 8.2.1.2.18-3: Interruption length in slots for scenario 2 and async case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

<End of Change 1>

<Start of Change 2>

##### 8.2.2.2.16	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

SRS transmission from UE helps gNB in acquisition of the DL CSI (full channel) at the gNB. When UE has a higher number of RX antenna compared to TX antenna, UE sounds the SRS on all the RX antenna with the help of SRS antenna port switching to help gNB acquire full channel. This feature is mandatory with capability signalling, “SRS-TxPortSwitch”. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas. The requirements in this clause applicable for different SRS antenna switch patterns.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

-     the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD.

-	 the SRS switching is not colliding with E-UTRA measurement.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx.

Interruption requirements on the victim CC are defined for following two scenarios for sync and async cases.

Scenario 1: 1 SRS symbol is transmitted

Scenario 2: 6 symbols (SRS and guard symbols including)

Interruption requirements are defined in symbol level for scenario 1 sync case. For other following scenarios, interruption requirements are defined in slot level

- Scenario 1 async case

- Scenario 2 async case and sync case

Interruption requirement for scenario 1 and sync case:

For scenario 1, number of SRS is 1 OFDM symbol. MRTD assumed for derving the interruption length is 33us. The interruption length for different SCS or aggressor and victim CC are provided in table 8.2.2.2.16-1.

Table 8.2.2.2.16-1: Interruption length in symbols for scenario 1 and sync case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 3 | 3 | 3 |
| 30 | 6 | 5 | 3 |
| 60 | 10 | 8 | 7 |
| 120 | 20 | 14 | 14 |

Interruption requirement for scenario 1 and async case:

For scenario 1 number of SRS is 1 OFDM symbol. UL TA is assumed to introduce one additional slot interruption on victim CC. The interruption length for different SCS of aggressor and victim CC are provided in table 8.2.2.2.16-2.

Table 8.2.2.2.16-2: Interruption length in slots for scenario 1 and async case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Interruption requirement for scenario 2 sync and async cases:

For scenario 2, number of SRS and guard period combined is assumed as 6 OFDM symbols for deriving the requirements. UL TA is assumed to introduce additional 1 slot to interruption requirement. The interruption length for different SCS of aggressor and victim CC are provided in table 8.2.2.2.16-3.

Table 8.2.2.2.16-3: Interruption length in slots for scenario 2 and async case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

<End of Change 2>

<Start of Change 3>

##### 8.2.3.2.16	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

SRS transmission from UE helps gNB in acquisition of the DL CSI (full channel) at the gNB. When UE has a higher number of RX antenna compared to TX antenna, UE sounds the SRS on all the RX antenna with the help of SRS antenna port switching to help gNB acquire full channel. This feature is mandatory with capability signalling, “SRS-TxPortSwitch”. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas. The requirements in this clause applicable for different SRS antenna switch patterns.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

-     the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD.

-	 the SRS switching is not colliding with E-UTRA measurement.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx.

Interruption requirements on the victim CC are defined for following two scenarios for sync and async cases.

Scenario 1: 1 SRS symbol is transmitted

Scenario 2: 6 symbols (SRS and guard symbols including)

Interruption requirements are defined in symbol level for scenario 1 sync case. For other following scenarios, interruption requirements are defined in slot level

- Scenario 1 async case

- Scenario 2 async case and sync case

Interruption requirement for scenario 1 and sync case:

For scenario 1, number of SRS is 1 OFDM symbol. MRTD assumed for derving the interruption length is 33us. The interruption length for different SCS or aggressor and victim CC are provided in table 8.2.3.2.16-1.

Table 8.2.3.2.16-1: Interruption length in symbols for scenario 1 and sync case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 3 | 3 | 3 |
| 30 | 6 | 5 | 3 |
| 60 | 10 | 8 | 7 |
| 120 | 20 | 14 | 14 |

Interruption requirement for scenario 1 and async case:

For scenario 1 number of SRS is 1 OFDM symbol. UL TA is assumed to introduce one additional slot interruption on victim CC. The interruption length for different SCS of aggressor and victim CC are provided in table 8.2.3.2.16-2.

Table 8.2.3.2.16-2: Interruption length in slots for scenario 1 and async case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Interruption requirement for scenario 2 sync and async cases:

For scenario 2, number of SRS and guard period combined is assumed as 6 OFDM symbols for deriving the requirements. UL TA is assumed to introduce additional 1 slot to interruption requirement. The interruption length for different SCS of aggressor and victim CC are provided in table 8.2.3.2.16-3.

Table 8.2.3.2.1-3: Interruption length in slots for scenario 2 and async case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

<End of Change 3>

<Start of Change 4>

##### 8.2.4.2.14	 Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1.

The interruption requirement in this clause is defined based on the band combination capability reported by UE, i.e., based on txSwitchImpactToRx or txSwitchWithAnotherBand.

SRS transmission from UE helps gNB in acquisition of the DL CSI (full channel) at the gNB. When UE has a higher number of RX antenna compared to TX antenna, UE sounds the SRS on all the RX antenna with the help of SRS antenna port switching to help gNB acquire full channel. This feature is mandatory with capability signalling, “SRS-TxPortSwitch”. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas. The requirements in this clause applicable for different SRS antenna switch patterns.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

-     the SRS switching is not colliding with any NR measurements (i.e., SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD.

-	 the SRS switching is not colliding with E-UTRA measurement.

When SRS antenna port switching is performed, interruption requirements does not depend on per-FR gap.

For interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability. e.g., an UL interruption is allowed on any of the serving cells indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells indicated by txSwitchImpactToRx.

Interruption requirements on the victim CC are defined for following two scenarios for sync and async cases.

Scenario 1: 1 SRS symbol is transmitted

Scenario 2: 6 symbols (SRS and guard symbols including)

Interruption requirements are defined in symbol level for scenario 1 sync case. For other following scenarios, interruption requirements are defined in slot level

- Scenario 1 async case

- Scenario 2 async case and sync case

Interruption requirement for scenario 1 and sync case:

For scenario 1, number of SRS is 1 OFDM symbol. MRTD assumed for derving the interruption length is 33us. The interruption length for different SCS or aggressor and victim CC are provided in table 8.2.4.2.14-1.

Table 8.2.4.2.14-1: Interruption length in symbols for scenario 1 and sync case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 3 | 3 | 3 |
| 30 | 6 | 5 | 3 |
| 60 | 10 | 8 | 7 |
| 120 | 20 | 14 | 14 |

Interruption requirement for scenario 1 and async case:

For scenario 1 number of SRS is 1 OFDM symbol. UL TA is assumed to introduce one additional slot interruption on victim CC. The interruption length for different SCS of aggressor and victim CC are provided in table 8.2.4.2.14-2.

Table 8.2.4.2.14-2: Interruption length in slots for scenario 1 and async case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 2 | 2 | 2 |
| 120 | 2 | 2 | 2 |

Interruption requirement for scenario 2 sync and async cases:

For scenario 2, number of SRS and guard period combined is assumed as 6 OFDM symbols for deriving the requirements. UL TA is assumed to introduce additional 1 slot to interruption requirement. The interruption length for different SCS of aggressor and victim CC are provided in table 8.2.4.2.14-3.

Table 8.2.4.2.14-3: Interruption length in slots for scenario 2 and async case

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

<End of Change 4>