3GPP TSG RAN WG4 Meeting #102-e	R4-2205839

# E-meeting, 21 Feb. 2022 – 03 Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on HO with PSCell requirements for NE DC to NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of HO with PSCell feature in Rel-17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the HO with PSCell requirements for NE-DC to NE-DC scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HO with PSCell requirements for NE-DC to NE-DC will be missing in Rel-17 specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5.4, 6.1.5.4.1, 6.1.5.4.2, 6.1.5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

#### 6.1.5.4	HO with PSCell from NE-DC to NE-DC

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR PCell to NR PCell on FR1, and the PSCell addition is on LTE.

##### 6.1.5.4.1	Handover delay

When the UE receives a RRC message implying PCell handover with PSCell addition/change, starting point of the HO on PCell is end of the last TTI containing the RRC command and ending point of the PCell HO is time at which UE is ready to transmit PRACH on PCell. For PSCell addition/change, starting point of the PSCell addition/change is end of the last TTI containing the RRC command and ending point of PSCell addition/change is time at which UE is ready to transmit PRACH on PSCell.

The PCell handover delay, DHOwithPSCel_PCell, is equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the PCell interruption time (Tinterrupt) define in clause 6.1.5.4.2.

PSCell addition/change delay, DHOwithPSCel_PSCell is defined in clause 6.1.5.4.3.

##### 6.1.5.4.2	HO with PSCell - PCell Interruption time

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch_PCell  + T∆_PCell + Tmargin_PCell + TIU_PCell + Tprocessing ms

Where:

Tsearch_PCell is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch_PCell = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch_PCell = Trs_PCell ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch_PCell = 3* Trs_PCell ms. Regardless of whether DRX is in use by the UE, Tsearch_PCell shall still be based on non-DRX target cell search times.

T∆_PCell is time for fine time tracking and acquiring full timing information of the target cell. T∆_PCell = Trs_PCell.

Tmargin_PCell is time for SSB post-processing. Tmargin_PCell can be up to 2ms.

TIU_PCell is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU_PCell can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Tprocessing is time for UE processing. Tprocessing is FFS.

Trs_PCell is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise T∆_PCell is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, T∆_PCell is FFS.  If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs_PCell =5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs_PCell follows smtc1 or smtc2 according to the physical cell ID of the target cell.

PCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover and Clause 9.3.4 for inter-frequency handover.

##### 6.1.5.4.3	PSCell addition/change in NE-DC to NE-DC  HO with PSCell

When intra-frequency or inter-frequency handover is commanded, the PSCell addition/change time shall be less than  DHOwithPSCel_PSCell:

DHOwithPSCel_PSCell = TRRC_delay + Tsearch_PSCell  + T∆_PSCell  + TIU_PSCell  + Tprocessing ms

Where:

TRRC_delay is maximum RRC procedure delay to be defined in clause 12 in TS 38.331 [2].

Tsearch_PSCell is the PSCell search delay. If the PSCell is known, then Tsearch_PSCell is 20ms. If the PSCell is unknown, then Tsearch_PSCell is 30ms provided the PSCell can be successfully detected on the first attempt.

T∆_PCell is time for fine time tracking and acquiring full timing information of the target cell. T∆_PCell = 50ms.

TPSCell_ IU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ IU is FFS.

PSCell is known if it has been meeting the following conditions:

During the last 5 seconds before the reception of the PSCell configuration command:

-	the UE has sent a valid measurement report for the PSCell being configured and

-	the PSCell being configured remains detectable according to the cell identification conditions specified in section 8.8,

-	PSCell being configured also remains detectable during the PSCell configuration delay according to the cell identification conditions specified in section 8.8.

Otherwise, it is unknown. The PCell interruption specified in section 7.12 is allowed only during the RRC reconfiguration procedure [2].

<End of Change>