3GPP TSG-RAN WG4 Meeting # 102-e	R4-2205850

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clause title change on big CR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | R16 power saving feature requirement clause titles are (two examples): “Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion” “Measurements for UE fulfilling low mobility criterion” We suggest to following the same phrasing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use the following titles 8.1.2.X Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria 8.1.3.X Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria 8.5.2.X Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria 8.5.3.X Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria And aligned some text to the titles. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clause titles are misaligned to R16 power saving features |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.X, 8.1.3.X, 8.5.2.X, 8.5.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

#### 8.1.2.X	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria

This clause contains minimun requirements of radio link monitoring based on SSB for UE fulfilling relaxed measurement criteria [TBD].

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB_Relax [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB_Relax [ms] evaluation period.

TEvaluate_out_SSB_Relax is defined in Table 8.1.2.X-1 for FR1.

TEvaluate_out_SSB_Relax is defined in Table 8.1.2.X-2 for FR2 with scaling factor N=8.

The value of P is defined in clause 8.1.2.2.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 8.1.2.X-1: Evaluation period TEvaluate_out_SSB_Relax for FR1

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80ms | Max([200], Ceil(15 K1 P) Max(TDRX,TSSB)) |
| 80ms < Max(TDRX,TSSB) ≤160ms | Ceil(15 P) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80ms. NOTE 2: K1 = [TBD]. |  |

Table 8.1.2.X-2: Evaluation period TEvaluate_out_SSB_Relax for FR2

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80ms | Max([200], Ceil(15 K2 P N) Max(TDRX,TSSB)) |
| 80ms < Max(TDRX,TSSB) ≤160ms | Ceil(15 P N) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80ms. NOTE 2: K2 = [TBD]. |  |

<End of Change #1>

<Start of Change #2>

#### 8.1.3.X	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria

This clause contains minimum requirements of radio link monitoring based on CSI-RS for UE fulfilling relaxed measurement criteria [TBD].

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS_Relax [ms] period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS_Relax [ms] evaluation period.

TEvaluate_out_CSI-RS_Relax is defined in Table 8.1.3.X-1 for FR1.

TEvaluate_out_CSI-RS_Relax is defined in Table 8.1.3.X-2 for FR2 with scaling factor N=1.

The value of P is defined in clause 8.1.3.2.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of Mout used in Table 8.1.3.X-1 and Table 8.1.3.X-2 are defined as:

-	Mout = 20, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs.

Table 8.1.3.X-1: Evaluation period TEvaluate_out_CSI-RS_Relax for FR1

| Configuration | TEvaluate_out_CSI-RS_Relax (ms) |
| --- | --- |
| Max(TDRX, TCSI-RS) ≤ 80 ms | Max([200], Ceil(1.5 Mout P K1) Max(TDRX, TCSI-RS)) |
| NOTE1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length and no longer than 80ms. NOTE2: K1 = TBD |  |

Table 8.1.3.X-2: Evaluation period TEvaluate_out_CSI-RS_Relax for FR2

| Configuration | TEvaluate_out_CSI-RS_Relax (ms) |
| --- | --- |
| Max(TDRX, TCSI-RS) ≤ 80 ms | Max([200], Ceil(1.5 Mout P N K2) Max(TDRX, TCSI-RS)) |
| NOTE1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length and no longer than 80ms. NOTE2: K2 = 2 is the relaxation factor |  |

<End of Change #2>

<Start of Change #3>

##### 8.5.2.X	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria

This clause contains minimum requirements of SSB based beam failure detection for UE fulfilling relaxed measurement criteria [TBD].

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB_Relax ms period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB_Relax ms period.

The value of TEvaluate_BFD_SSB_Relax is defined in Table 8.5.2.X-1 for FR1.

The value of TEvaluate_BFD_SSB_Relax is defined in Table 8.5.2.X-2 for FR2 with scaling factor N=8

The value of P is defined in clause 8.5.2.2.

Longer evaluation period would be expected if the combination of BFD-RS resource, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer BFD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 8.5.2.X-1: Evaluation period TEvaluate_BFD_SSB_Relax for FR1

<To be added>

Editors note: The table will be added after RAN4 achieving the consensus on relaxation facor

Table 8.5.2.X-2: Evaluation period TEvaluate_BFD_SSB_Relax for FR2

| Configuration | TEvaluate_BFD_SSB_Relax (ms) |
| --- | --- |
| MAX(TDRX, TSSB) ≤ 80 ms | Max(50, Ceil(7.5 [K3] P N) Max(TDRX,TSSB)) |
| 80ms＜MAX(TDRX, TSSB) ≤ 160 ms | Max(50, Ceil(7.5 P) Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: [K3] = TBD is the relaxation factor applicable for UE fulfilling the BFD relaxation criteria in [TBD]. |  |

<End of Change #3>

<Start of Change #4>

#### 8.5.3.X	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria

This clause contains the minimum requirements of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria [TBD].

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS_Relax ms period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS_Relax ms period.

The value of TEvaluate_BFD_CSI-RS_Relax is defined in Table 8.5.3.X-1 for FR1.

The value of TEvaluate_BFD_CSI-RS_Relax is defined in Table 8.5.3.X-2 for FR2 with N=1.

The values of P, MBFD and PBFD is defined in clause 8.5.3.2.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 8.5.3.X-1: Evaluation period TEvaluate_BFD_CSI-RS_Relax for FR1

| Configuration | TEvaluate_BFD_CSI-RS Relax (ms) |
| --- | --- |
| Max(TDRX, TCSI-RS) ≤ 40 ms | Max([50], Ceil(K1 × 1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| 40 ms < Max(TDRX, TCSI-RS) ≤ 80ms | Max([50], Ceil(K2 × 1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| 80 ms < Max(TDRX, TCSI-RS) ≤ 160 ms | Max([50], Ceil(1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| Note 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K1 = TBD, K2= TBD is the relaxation factor applicable for UE fulfilling the BFD relaxation criteria in [TBD] |  |

Table 8.5.3.X-2: Evaluation period TEvaluate_BFD_CSI-RS_Relax for FR2

| Configuration | TEvaluate_BFD_CSI-RS Relax (ms) |
| --- | --- |
| Max(TDRX, TCSI-RS) ≤ 80ms | Max(100, Ceil(K2 × 1.5 × MBFD P N PBFD) Max(TDRX, TCSI-RS)) |
| 80 ms < Max(TDRX, TCSI-RS) ≤ 160 ms | Max([100], Ceil(1.5 × MBFD P N PBFD) Max(TDRX, TCSI-RS)) |
| Note 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K2 = 2 is the relaxation factor applicable for UE fulfilling the BFD relaxation criteria in [TBD] |  |

<End of Change #4>