3GPP TSG-RAN WG4 Meeting #102-e						                                     R4-2205851

Electronic Meeting, 21st of February – 3rd of March, 2022

Agenda item:			10.13.3.2

Source:	THALES

Title:	Draft text proposal for Clause 7.3.5.6 OTA Out-of-band blocking in TR 38.863

Document for:	Approval

# Introduction

During RAN WG4 Meeting RAN4#102-e, a work split has been proposed to complete Section 7 of TR 38.863. In this document, a TP is proposed to update some of the subsections related to RF requirements.

# Discussion

In this contribution is proposed to add information with respect to Section 7.3.5.6	 OTA Out-of-band blocking of TR 38.863 [1] by including some justifications and recommendations with respect to the TS 38.108 specification elaboration.

Proposal 1: To update the TR with the attached TP

# Conclusions

In this contribution, the following proposal is made:

Proposal 1: To update the TR with the attached TP

# Reference

[1]	TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

# Text proposals for TR 38.863

------------------------------------------------<Start of TP>-----------------------------------------------

## 7.3.5.6	OTA out-of-band blocking

### 7.3.5.6.1	General

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

### 7.3.5.6.2	Minimum requirement for SAN type 1-O

#### 7.3.5.6.2.1	General minimum requirement

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted signal applies to each supported polarization, under the assumption of polarization match. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

The values for the minimum requirements have been obtained with the hypothesis from section 7.3.3.2.4 Out-of-band blocking, considering the maximum EIRP of SpaceOps (Space Operations) TC (TeleCommand) stations of 80 dBW, and the interfering signal mean power of -44 dBm, for a LEO satellite at 600 km of altitude (LEO@600km).

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 7.3.5.6.2.1-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SAN type 1-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band.

Table 7.3.5.6.2.1-1: OTA out-of-band blocking performance requirement

| Wanted signal mean power (dBm) | Interfering signal RMS field-strength (V/m) | Type of interfering Signal |
| --- | --- | --- |
| EISminSENS + 6 dB (Note 1) | 0.091 | CW carrier |
| NOTE 1: EISminSENS depends on the channel bandwidth. NOTE 2: The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m. |  |  |

#### 7.3.5.6.2.2	Co-location minimum requirement

The requirement is not applicable in Release-17.

------------------------------------------------<End of TP>-------------------------------------------------