3GPP TSG-RAN Working Group 4 (Radio) Meeting #102-E	R4-2205853

# Electronic Meeting, 21th February – 3rd March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.113 | CR | 0067 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 25.113: Corrections in Clause 9 Immunity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | These corrections were discussed and approved for the draft specifications TS 38.114 NR Repeater during last meeting (R4-2117585). This CR proposes similar changes in TS 25.113 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce use of reverberation chamber for RF electromagnetic field testing. - Remove the note claiming that the EUT should be fully discharged between each ESD exposure. It is very difficult to ensure that EUT is “fully discharged”, especially if there are plastic parts on it. - Remove a statement on application of transients on an impedance that shall be 50 ohm - Remove a note about “intrusive method” as this term is not defined and in fact the IEC spec mention “clamp injection” instead. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reverberation chamber not considered for RF electromagnetic field testing. Confusing statements in the specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.3.2, 9.4.2, 9.5.2, 9.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------- start of changes----------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 25.104: "UTRA (BS) FDD; Radio transmission and reception".

[2]	3GPP TS 25.105: "UTRA (BS) TDD; Radio transmission and reception".

[3]	3GPP TS 25.141: "UTRA (BS) FDD; Base station conformance testing (FDD)".

[4]	3GPP TS 25.142: "UTRA (BS) TDD; Base station conformance testing (TDD)".

[5]	IEC 61000-6-1: 2005; "Electromagnetic compatibility (EMC) - Part 6: Generic standards - Section 1: Immunity for residential, commercial and light-industrial environments".

[6]	IEC 61000-6-3: 2006/AMD1:2010: "Electromagnetic compatibility (EMC) - Part 6: Generic standards - Section 3: Emission standard for residential, commercial and light industrial environments".

[7]	IEC 60050(161): "International Electrotechnical Vocabulary - Chapter 161: Electromagnetic compatibility".

[8]	3GPP TS 25.101: "UTRA (UE) FDD; UE Radio transmission and reception (FDD)".

[9]	3GPP TS 25.102: "UTRA (UE) TDD: UE Radio transmission and reception (TDD)".

[10]	3GPP TS 25.106: "UTRA Repeater; Radio Transmission and Reception".

[11]	3GPP TS 25.143: "UTRA Repeater conformance testing".

[12]	ITU-R Rec. SM.329: "Unwanted emissions in the spurious domain".

[13]	Void

[14]	CISPR 16-1-1: "Specification for radio disturbance and immunity measuring apparatus and methods - Measuring apparatus".

[15]	IEC 61000-3-2 (2004): "Electromagnetic compatibility (EMC) - Part 3: Limits - Section 2: Limits for harmonic current emissions (equipment input current ≤ 16 A)".

[16]	IEC 61000-3-3 (2002): "Electromagnetic compatibility (EMC) - Part 3: Limits - Section 3: Limitation of voltage fluctuations and flicker in low-voltage supply systems for equipment with rated current ≤ 16 A".

[17]	IEC 61000-4-2: "Electromagnetic compatibility (EMC) - Part 4: Testing and measurement techniques - Section 2: Electrostatic discharge immunity test".

[18]	IEC 61000-4-3: "Electromagnetic compatibility (EMC) - Part 4: Testing and measurement techniques - Section 3: Radiated, radio-frequency electromagnetic field immunity test".

[19]	IEC 61000-4-4: "Electromagnetic compatibility (EMC) - Part 4: Testing and measurement techniques - Section 4: Electrical fast transient/burst immunity test".

[20]	IEC 61000-4-5: "Electromagnetic compatibility (EMC) - Part 4: Testing and measurement techniques - Section 5: Surge immunity test".

[21]	IEC 61000-4-6: "Electromagnetic compatibility (EMC) - Part 4: Testing and measurement techniques - Section 6: Immunity to contacted disturbances, induced by radio frequency fields".

[22]	IEC 61000-4-11: "Electromagnetic compatibility (EMC) - Part 4: Testing and measurement techniques - Section 11: Voltage dips, short interruptions and voltage variations. Immunity tests".

[23]	ITU-R Recommendation SM.1539 (2001): "Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541 and ITU-R SM.329".

[24]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[25]	IEC 61000-3-12 (2005): "Electromagnetic compatibility (EMC) - Part 3-12: Limits- Limits for harmonic current produced by equipment connected to public low-voltage system with  input current >16 A and  ≤ 75 A.

[26]	IEC 61000-3-11 (2000): "Electromagnetic compatibility (EMC) - Part 3-11: Limits –Limitation of voltage fluctuations and flicker in low-voltage supply systems for equipment with rated current ≤  75 A and subject to conditional connections"3	Definitions, symbols and abbreviations

[27]	3GPP TS 37.113: “E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)”.

[28]	CISPR 32: “Electromagnetic compatibility of multimedia equipment - Emission requirements”.

[29]	IEC 61000-4-21: "Electromagnetic Compatibility (EMC) Part 4-21: Testing And Measurement Techniques Reverberation Chamber Test Methods".

# 3	Definitions, symbols and abbreviations

---------- end of changes----------

---------- start of changes----------

## 9.3	RF electromagnetic field (80 MHz - 6000 MHz)

The test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 9.3.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic field disturbance at the enclosure.

### 9.3.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [18], which specifies test methodology based on anechoic chamber. The use of reverberation chamber test method according to IEC 61000-4-21 [29], clause 6.1 and Annex D as alternative method is allowed.

--

T	he following requirements shall apply:

-	the test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	the stepped frequency increments shall be 1 % of the momentary frequency;

-	the test shall be performed over the frequency range 80 MHz - 6000 MHz;

-	responses in stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see subclause 4.3;

-	the frequencies selected during the test shall be recorded in the test report.

### 9.3.3	Performance criteria

Base station:

The performance criteria of subclause 6.1 shall apply.

Ancillary equipment:

The performance criteria of subclause 6.4 shall apply.

Repeater:

The performance criteria of subclause 6.7 shall apply.

## 9.4	Electrostatic discharge

The test shall be performed on a representative configuration of the radio equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 9.4.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the event of an electrostatic discharge.

### 9.4.2	Test method and level

The test method shall be in accordance with IEC 61000-4-2 [17]:

-	for contact discharge, the equipment shall pass at ±4 kV;

-	for air discharge shall pass at ±8 kV;

-	electrostatic discharge shall be applied to all exposed surfaces of the EUT except where the user documentation specially indicates a requirement for appropriate protective measures.

### 9.4.3	Performance criteria

Base station:

The performance criteria of subclause 6.2 shall apply.

Ancillary equipment:

The performance criteria of subclause 6.5 shall apply.

Repeater:

The performance criteria of subclause 6.8 shall apply.

## 9.5	Fast transients common mode

The test shall be performed on AC mains power input ports.

This test shall be performed on signal ports, telecommunication ports, control ports and DC power input/output ports if the cables may be longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares that it is not intended to be used with cables longer than 3 m, a list of ports which were not tested for this reason shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 9.5.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the event of fast transients present on one of the input/output ports.

### 9.5.2	Test method and level

The test method shall be in accordance with IEC 61000-4-4 [19]:

-	the test level for signal ports, telecommunication ports and control ports shall be 0,5 kV open circuit voltage as given in IEC 61000-4-4 [19];

-	the test level for DC power input/output ports shall be 0.5 kV open circuit voltage as given in IEC 61000-4-4 [19];

-	the test level for AC mains power input ports shall be 1 kV open circuit voltage as given in IEC 61000-4-4 [19].

### 9.5.3	Performance criteria

Base station:

The performance criteria of subclause 6.2 shall apply.

Ancillary equipment:

The performance criteria of subclause 6.5 shall apply.

Repeater:

The performance criteria of subclause 6.8 shall apply.

## 9.6	RF common mode (0,15 MHz - 80 MHz)

The test shall be performed on AC mains power input/output ports.

This test shall be performed on signal ports, telecommunication ports, control and DC power input/output ports, which may have cables longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

--

### 9.6.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic disturbance.

### 9.6.2	Test method and level

The test method shall be in accordance with IEC 61000-4-6 [21]:

-	the test signal shall be amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	the stepped frequency increments shall be 50 kHz in the frequency range 150 kHz to 5 MHz and 1%frequency increment of the momentary frequency in the frequency range 5 MHz to 80 MHz.

-	the test level shall be severity level 2 as given in IEC 61000-4-6 [21] corresponding to 3 V rms, at a transfer impedance of 150 Ω;

-	the test shall be performed over the frequency range 150 kHz - 80 MHz;

-	the injection method to be used shall be selected according to the basic standard IEC 61000-4-6 [21];

-	responses of stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see subclause 4.3;

-	the frequencies of the immunity test signal selected and used during the test shall be recorded in the test report.

### 9.6.3	Performance criteria

----------end of changes----------