3GPP TSG-RAN Working Group 4 (Radio) Meeting #102-E	R4-2205861

# Electronic Meeting, 21th February – 3rd March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR | 0045 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 38.113: Corrections in Scope and Clause 9 Immunity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | These corrections were discussed and approved for the draft specifications TS 38.114 NR Repeater during last meeting (R4-2117585). This CR proposes similar changes in TS 38.113 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added a statement in the scope to clarify that technical specifications related to the antenna port are not considered. - Add instead the level of 10 V/m and delete the references to EU EMC regulations. - Remove the note claiming that the EUT should be fully discharged between each ESD exposure. It is very difficult to ensure that EUT is “fully discharged”, especially if there are plastic parts on it. - Remove a statement on application of transients on an impedance that shall be 50 ohm - Remove a note about “intrusive method” as this term is not defined and in fact the IEC spec mention “clamp injection” instead. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing statements in the specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 9.2.2, 9.3.2, 9.4.2, 9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------- start of changes----------

# 1	Scope

The present document covers the assessment of NR Base Station (BS) and ancillary equipment in respect of Electromagnetic Compatibility (EMC).

The present document specifies the applicable requirements, procedures, test conditions, performance assessment and performance criteria for NR base stations and associated ancillary equipment in the following categories:

-	BS equipped with antenna connectors or TAB connectors which are possible to be terminated during EMC testing, meeting the BS type 1-C and BS type 1-H RF requirements of TS 38.104 [2], with conformance demonstrated by compliance to TS 38.141-1 [3].

-	BS not equipped with antenna connectors nor TAB connectors, i.e. with antenna elements radiating during the EMC testing, meeting the BS type 1-O and BS type 2-O RF requirements of TS 38.104 [2], with conformance demonstrated by compliance to TS 38.141-2 [4].

Technical specifications related to the antenna port are not included in the present document.

The environment classification used in the present document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 [7] and IEC 61000-6-3 [8].

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

---------- end of changes----------

---------- start of changes----------

## 9.2	RF electromagnetic field (80 MHz to 6000 MHz)

The test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 9.2.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic field disturbance at the enclosure.

### 9.2.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [18], which specifies test methodology based on anechoic chamber. The use of reverberation chamber test method according to IEC 61000-4-21 [29], clause 6.1 and Annex D as alternative method is allowed.

The following requirements shall apply:

-	The test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 1 % of the momentary frequency;

-	The test shall be performed over the frequency range 80 MHz  - 6000 MHz; with the exception of the exclusion band for receivers (see subclause 4.4);

-	Responses in stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see subclause 4.3;

-	The frequencies selected during the test shall be recorded in the test report.

-	For the test method in accordance with IEC 61000-4-3[18], the spatial exclusion zone can be chosen to protect the base station receiver. For the frequency range above 690 MHz a level of 10 V/m applies on the non-radiating faces of the BS type 1-O or BS type 2-O, as depicted on figure 9.2.2-1.

NOTE:	Depending on the BS implementation, application of the spatial exclusion to all radiating faces of the BS may not allow proper execution of the RI testing. In such cases, to protect the BS type 1-O or BS type 2-O receiver(s), exclusion bands shall be considered, as in table 4.4.2-1.

Figure 9.2.2-1: EMC RF electromagnetic field immunity requirement testing directions for BS type 1-O and BS type 2-O (horizontal plane depicted) with the spatial exclusion zone applied

### 9.2.3	Performance criteria

Base station:

The performance criteria of subclause 6.1 shall apply.

Ancillary equipment:

The performance criteria of subclause 6.3 shall apply.

## 9.3	Electrostatic discharge

The test shall be performed on a representative configuration of the radio equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 9.3.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the event of an electrostatic discharge.

### 9.3.2	Test method and level

The test method shall be in accordance with IEC 61000-4-2 [17]:

-	for contact discharge, the equipment shall pass at  ±4 kV;

-	for air discharge shall pass at ±8 kV;

-	electrostatic discharge shall be applied to all exposed surfaces of the EUT except where the user documentation specially indicates a requirement for appropriate protective measures.

### 9.3.3	Performance criteria

Base station:

The performance criteria of subclause 6.2 shall apply.

Ancillary equipment:

The performance criteria of subclause 6.4 shall apply.

## 9.4	Fast transients common mode

The test shall be performed on AC mains power input ports.

This test shall be performed on signal ports, telecommunication ports, control ports and DC power input/output ports if the cables may be longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares that it is not intended to be used with cables longer than 3 m, a list of ports which were not tested for this reason shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 9.4.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the event of fast transients present on one of the input/output ports.

### 9.4.2	Test method and level

The test method shall be in accordance with IEC 61000-4-4 [19]:

-	The test level for signal ports, telecommunication ports and control ports shall be 0.5 kV open circuit voltage as given in IEC 61000-4-4 [19];

-	The test level for DC power input/output ports shall be 0.5 kV open circuit voltage as given in IEC 61000-4-4 [19];

-	The test level for AC mains power input ports shall be 1 kV open circuit voltage as given in IEC 61000-4-4 [19].

### 9.4.3	Performance criteria

Base station:

The performance criteria of subclause 6.2 shall apply.

Ancillary equipment:

The performance criteria of subclause 6.4 shall apply.

## 9.5	RF common mode (0.15 MHz - 80 MHz)

The test shall be performed on AC mains power input/output ports.

This test shall be performed on signal ports, telecommunication ports, control and DC power input/output ports, which may have cables longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

--

### 9.5.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic disturbance.

----------end of changes----------