3GPP TSG-RAN WG4 Meeting #102-e		R4- 2206003

E-meeting, February 21 – March 03, 2022

Agenda item:	10.13.6.2

Source:	Intel Corporation

Title:	Discussion on Satellite Access Node demodulation requirements for NR NTN

Document for:	Discussion

# Introduction

In this paper we provide our views on the open issues identified during the RAN4 101-bis-e meeting and captured in the WF [1]

# Discussion

General assumptions

Channel model

NTN channel models for link level simulations are introduced in TR38.811: two TDL models, namely NTN-TDL-A and NTN-TDL-B are constructed to represent two different channel profiles for NLOS, while NTN-TDL-C and NTN-TDL-D are constructed for LOS.

During RAN4 101-bis-e meeting it was agreed to use all four cahannel models as the starting point. In Rel-15/16 the PUCCH and PRACH requirements are defined for one channel model and PUSCH requirements are defined for different combinations of TDL channels and FRC. We propose to have the same approach for NR NTN. Different combinations of NTN-TDL channels and FRC can be defined as it is done for Rel-15.

Proposal 1: RAN4 to use one NTN-TDL channel model for PUCCH requirements definition, one NTN-TDL channel model for PRACH requirements definition and consider different combinations of NTN-TDL channels and FRC for PUSCH requirements definition.

Delay spread

Study on the delay spread in NTN systems can be found in [2] and [3]. However, there is no DS values recommended for link-level simulation as in TR38.901. In [2] and [3] the DS distributions for different elevation angles and different scenarios are present. We propose not to consider different scenarios and elevations but define short, nominal and long delay spread instead.

Proposal 2: RAN4 will not consider different scenarios and elevations for delay spread but define short, nominal and long delay spread instead.

Based on the above-mentioned distributions we propose the following values for delay spread:

Table 2-1 Scaling parameters for TDL channel model

| Model | DS |
| --- | --- |
| Short delay spread | 10 ns |
| Nominal delay spread | 50 ns |
| Long delay spread | 150 ns |

Proposal 3: RAN4 to consider the DS values for requirements definition as shown in Table 2-1

Doppler shift

Our understanding is that RAN1 agreement on NTN UE Doppler pre-compensation for UL considers only doppler due to Satellite motion. Thus, Doppler shift due to UE motion still need to be considered. Assuming UE maximum velocity of 120km/h we propose to consider frequency shift of 200Hz as the value which covers both UE motion and pre-compensation residual error.

Proposal 4: RAN4 to consider frequency shift of 200Hz which covers both UE motion and pre-compensation residual error

Tx and Rx assumptions for UL demodulation

Based on the satellite company feedback during the previous meeting, we sgree to focus on 1Tx, 2Rx configuration for UL demodulation

Proposal 5: For UL demodulation requirements definintion RAN4 to focus on 1Tx, 2Rx configuration as a starting point

PUSCH requirements

Scope of PUSCH requirements

The following list of PUSCH requirements to be defined was agreed during the RAN4 101-bis-e meeting

  - Transform precoding disabled

  - Transform precoding enabled

  - UL timing adjustment

  - repetition Type A

We think that the agreed list is sufficient and no additional requirements are needed for NTN SAT

Proposal 6: RAN4 to focus on the list of PUSCH requirements to be defined agreed during the RAN4 101-bis-e meeting. No need to extend the list

SCS/CBW set for PUSCH requirements

In order to align with Rel-15/16 BS requirements, we propose to consider the following SCS/CBW set: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

Proposal 7: RAN 4 to consider the following SCS/CBW set for PUSCH requirements definition: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

PUCCH requirements

SCS/CBW set for PUCCH requirements

Same as for PUSCH, in order to align with Rel-15/16 BS requirements, we propose to consider the following SCS/CBW set: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

Proposal 8: RAN 4 to consider the following SCS/CBW set for PUCCH requirements definition: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

# Conclusion

In this paper we discussed the identified open issues for the SAN performance requirements. The following proposals were made:

Proposal 1: RAN4 to use one NTN-TDL channel model for PUCCH requirements definition, one NTN-TDL channel model for PRACH requirements definition and consider different combinations of NTN-TDL channels and FRC for PUSCH requirements definition.

Proposal 2: RAN4 will not consider different scenarios and elevations for delay spread but define short, nominal and long delay spread instead.

Proposal 3: RAN4 to consider the DS values for requirements definition as shown in Table 2-1

Table 2-1 Scaling parameters for TDL channel model

| Model | DS |
| --- | --- |
| Short delay spread | 10 ns |
| Nominal delay spread | 50 ns |
| Long delay spread | 150 ns |

Proposal 4: RAN4 to consider frequency shift of 200Hz which covers both UE motion and pre-compensation residual error

Proposal 5: For UL demodulation requirements definintion RAN4 to focus on 1Tx, 2Rx configuration as a starting point

Proposal 6: RAN4 to focus on the list of PUSCH requirements to be defined agreed during the RAN4 101-bis-e meeting. No need to extend the list

Proposal 7: RAN 4 to consider the following SCS/CBW set for PUSCH requirements definition: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

Proposal 8: RAN 4 to consider the following SCS/CBW set for PUCCH requirements definition: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

# References

- R4-2203043, WF on NTN SAN demodulation requirements, Huawei, HiSilicon, RAN4 #101-bis-e, January 17th – 25th, 2022

- R1-1807164, NR-NTN Channel Modeling - Flat fading criteria, Thales RAN1 #93 May 21st  –  25th, 2018

- 3GPP TR 38.811