3GPP TSG-RAN WG4 Meeting # 102-e	R4- 2206051

# Electronic meeting, 21 February – 3 March 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On rel-17 UE features

Agenda item:	8

Document for:	Approval

# Introduction

This contributions summarizes Nokia’s proposals for UE features in rel-17. For more detailed technical discussion for the proposals, please see dedicated agenda items for each work item.

# Discussion

  - Proposals for WI: Extending current NR operation to 71GHz

Proposal 1: As each SCS is optional to support, further optionality on maximum channel bandwidth support is not required.

Proposal 2: For optional ON-ON transient time, only one value among 1 us or 2 us is specified

2.2 Proposals for WI: FR2 UE RF enhancements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL gap for Tx power management | 2-1 | Support of UL gap for Tx power management | Capability of performing BPS sensing for Tx power management. The UE indicating this capability shall meet the corresponding enhanced UE requirements defined in Section TBD. |  | yes | no | UE does not support UL gap for Tx power management | Per band | No | FR2 only |  |  | Optional with capability signalling |
| UL gap pattern for Tx power management ? | 2-2 |  |  |  |  |  |  |  |  | FR2 only |  |  |  |
| UL gap for coherent UL MIMO | 2-3 | Support of UL gap for coherent UL MIMO | Capability of performing coherent UL MIMO calibration in UL gap. The UE indicating this capability shall meet the corresponding enhanced UE requirements defined in Section TBD. |  | yes | no | UE does not support UL gap for coherent UL MIMO calibration | FFS per band or per UE | No | FR2 only |  |  | Optional with capability signalling |
| FR2 Interband CA | 2-4 | Support of beam management | Capability of support of specific beam management type. |  | yes | no | UE does not support FR2 interband CA | FFS per band or per UE | No | FR2 only |  | Indicate the supported beam management type for inter-band CA within FR2. Beam management type can be independent beam management (IBM) or common beam management (CBM), or both | Optional with capability signalling |
| DC-location | 2-5 | Support of UL DC location(s) report | Capability of support for the extended DC location reporting (based on indicated default DC location) for at least 2 UL CCs. |  | yes | no | UE does not support the Rel-17 extended UL DC location reporting | Per band per BC | No | No |  |  | Optional with capability signalling |
| New CA BW clases | 2-6 | Support of new CA BW Classes | RAN4 has introduced new CA BW Classes ‘R, S, T, U’ for REL17 |  | yes | no | UE does not support the Rel-17 extended FBG2 bandwidths | per band | No | FR2 only |  |  | Optional with capability signalling |
| [FBG 3+2] | 2-7 | [Support of new CA BW Classes] | RAN4 may introduce new fall back group and or new CA BW classes under FBG3+2 discussion |  | yes | no |  | per band | No | FR2 only |  |  | Optional with capability signalling |

  - Proposals for WI: enhanced IIoT/URLLC

Overall, from RAN4 perspective we think there is no need to introduce additional UE feature group for Rel-17 enhanced IIOT/URLLC.

Proposal 3: RAN4 does not introduce additional UE feature group for Rel-17 enhanced IIOT/URLLC.

# Conclusion

In this contribution we have provided a summary of our proposals for rel-17 UE features. We propose all of them to be approved.

# References