3GPP TSG-RAN WG4 Meeting#102-e	R4-2206070

Online, 21st Feb – 3rd Mar, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on Inter-cell Interference PDSCH Tests

Agenda item:	10.12.2.1.1

Document for:	Discussion

- Introduction

In RAN4#101-bis-e, WF [1] was agreed and it had a few open issues. In this paper, we provide our views on those open issues for inter-cell interference scenarios.

- PDSCH Test Parameters

Number of Interfering Cells

As we go from one interfering cell to two interfering cells, it will distort the interference structure since interference from two cells will get superimposed on top of each other. As the purpose of this test is to verify whether UE can handle the interference structure properly or not, 1 cell interference should be good enough and will not unnecessarily complicate the test setup.

We presented our simulation results in [2] comparing 1 cell (only the stronger cell) vs 2 cell (both strong and weak interfering cell) to illustrate this. Based on these results, we can clearly see that 2nd interfering cell reduces the MMSE-IRC gain compared to MMSE-MRC, as expected. Therefore, we have following observation and proposal.

Observation 1: Gain of MMSE-IRC over MMSE-MRC is higher with 1 interfering cell compared to 2 interfering cells.

Proposal 1: Only consider 1 interfering cell for defining the requirements.

INR Level for 1 cell HetNet

We present the simulation results below for FDD for both options of INR levels for 1 cell HetNet scenario.

Table 1: SNR at 70% of peak throughput for FDD 1cell HetNet interference scenario

| Case | MMSE-MRC | MMSE-IRC | Gain (dB) |
| --- | --- | --- | --- |
| FDD, 2Rx, INR = 4.84dB | 9.86 | 8.32 | 1.54 |
| FDD, 4Rx, INR = 4.84dB | 6.24 | 3.89 | 2.35 |
| FDD, 2Rx, INR = 7.58dB | 11.69 | 9.41 | 2.28 |
| FDD, 4Rx, INR = 7.58dB | 8.05 | 4.72 | 3.33 |

Based on above results, we see > 1.5dB of gain for both INR levels. Therefore, we observe and propose the following.

Observation 2: INR of 4.84dB achieves > 1.5dB gain of IRC over MRC in case of 1 interference cell HetNet deployment.

Proposal 2: Use INR = 4.84dB for defining requirements for 1 interference cell HetNet deployment.

Time Offset for TDD

Based on the RRM requirements, all cells should have time offset of within 3us for sync scenario. Considering time offset of 3us for interfering cell 1 and -1us for interfering cell 2 violates that requirement since time offset between cell1 and cell2 is 4us. Therefore, we observe and propose the following.

Observation 3: Considering time offset of 3us for interfering cell 1 and -1us for interfering cell 2 violates cell phase synchronization accuracy of 3us.

Proposal 3: Time offset from serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (if modeled) for 30kHz SCS.

Interference modelling in PDCCH region

For RLM procedure, UE estimates the SNR based on SSB. Therefore, we prefer to align the interference seen by SSB and PDCCH and propose the following.

Proposal 4: Assume no interference signal in PDCCH region when SSB is non-colliding and PDCCH transmission from interference cells when SSB is colliding.

Requirements for scenario 2

This scenario is not much deployed as most of the current deployments have slot-based transmission. This will require an enhanced receiver which needs to be studied first before agreeing to define the requirements and it will also require a separate UE capability. Given the limited time left in RAN4 for Rel-17, we prefer not to define the requirements for scenario 2 in Rel-17 and propose the following.

Proposal 5: Do not define requirements for scenario 2 in Rel-17.

Simulation Results

Based on simulation assumptions in [1], we provide the alignment SNR at 70% of peak throughput below for single cell interference assuming 100% loading on interfering cell.

Table 2: SNR in dB at 70% of peak throughput for FDD single cell interference scenario with MCS13, Rank1

| Case | MMSE-MRC | MMSE-IRC |
| --- | --- | --- |
| TDLA30-10 ULA Low, 2x2, INR = 4.84dB | 9.86 | 8.32 |
| TDLA30-10 ULA Low, 2x4, INR = 4.84dB | 6.24 | 3.89 |
| TDLC300-100 ULA Low, 2x2, INR = 5.49dB | 10.97 | 9.52 |
| TDLC300-100 ULA Low, 2x4, INR = 5.49dB | 7.60 | 4.78 |

Table 3: SNR in dB at 70% of peak throughput for FDD 2 cell interference scenario with MCS13, Rank1

| Case | MMSE-MRC | MMSE-IRC |
| --- | --- | --- |
| TDLA30-10 ULA Low, 2x2, INR = (11.39, 5.45) dB | 16.29 | 14.19 |
| TDLA30-10 ULA Low, 2x4, INR = (11.39, 5.45) dB | 12.46 | 8.84 |
| TDLC300-100 ULA Low, 2x2, INR = (7.77, 2.29) dB | 14.05 | 12.58 |
| TDLC300-100 ULA Low, 2x4, INR = (7.77, 2.29) dB | 10.50 | 7.71 |

- Conclusions

This paper provides our views on open issues for inter-cell interference scenarios in RAN4. Following has been observed and proposed.

Observation 1: Gain of MMSE-IRC over MMSE-MRC is higher with 1 interfering cell compared to 2 interfering cells.

Proposal 1: Only consider 1 interfering cell for defining the requirements.

Observation 2: INR of 4.84dB achieves > 1.5dB gain of IRC over MRC in case of 1 interference cell HetNet deployment.

Proposal 2: Use INR = 4.84dB for defining requirements for 1 interference cell HetNet deployment.

Observation 3: Considering time offset of 3us for interfering cell 1 and -1us for interfering cell 2 violates cell phase synchronization accuracy of 3us.

Proposal 3: Time offset from serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (if modeled) for 30kHz SCS.

Proposal 4: Assume no interference signal in PDCCH region when SSB is non-colliding and PDCCH transmission from interference cells when SSB is colliding.

Proposal 5: Do not define requirements for scenario 2 in Rel-17.

# References

[1] R4-2203008, “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, 3GPP TSG RAN WG4 Meeting #101-bis-e, Online, Jan 17 - 25, 2022.

[2] R4-2201965, “Views on Inter-cell Interference PDSCH Tests”, 3GPP TSG RAN WG4 Meeting #101-bis-e, Online, Jan 17 - 25, 2022.