3GPP TSG-RAN WG4 Meeting # 102-e	R4-2206120

# Electronic Meeting, 21 Feb – 03 Mar, 2022

Source: 	Huawei

Title: 	Discussion on the remaining BS RF requirements for FR2-2: Rx requirements

Agenda Item:	10.16.4.2

Document for:	Discussion

# Introduction

During previous RAN4#101bis-e meeting, the following was agreed on the remaining RX requirements [2]:

|  | WF form last meeting (R4-2203017) |
| --- | --- |
| Reference sensitivity | For reference sensitivity specify OTA reference sensitivity level as described in Table 2-1. Table 2-1: Reference sensitivity requirement Frequency Range; BS channel Bandwidth (MHz); Sub-carrier spacing (kHz); Reference measurement channel; OTA reference sensitivity level, EISREFSENS (dBm). FR2-1; 50, 100, 200; 60; G-FR2-A1-1; EISREFSENS_50M + ΔFR2_REFSENS. 50; 120; G-FR2-A1-2; EISREFSENS_50M + ΔFR2_REFSENS. 100, 200, 400; 120; G-FR2-A1-3; EISREFSENS_50M + 3 + ΔFR2_REFSENS. FR2-2; 100,400; 120; G-FR2-A1-3; EISREFSENS_50M + 3 + ΔFR2_REFSENS. 400, 800, 1600; 480; G-FR2-A1-6; EISREFSENS_50M + [9] + ΔFR2_REFSENS. 400, 800, 1600, 2000; 960; G-FR2-A1-7; EISREFSENS_50M + [9] + ΔFR2_REFSENS. |

In this contribution, we provide feedback on the remaining BS RF Rx requirements for FR2-2.

# Discussion

Below we briefly summarize our view in Proposal 1.

|  | WF form last meeting (R4-2203017) | Huawei proposals |
| --- | --- | --- |
| Reference sensitivity | For reference sensitivity specify OTA reference sensitivity level as described in Table 2-1. Table 2-1: Reference sensitivity requirement Frequency Range; BS channel Bandwidth (MHz); Sub-carrier spacing (kHz); Reference measurement channel; OTA reference sensitivity level, EISREFSENS (dBm). FR2-1; 50, 100, 200; 60; G-FR2-A1-1; EISREFSENS_50M + ΔFR2_REFSENS. 50; 120; G-FR2-A1-2; EISREFSENS_50M + ΔFR2_REFSENS. 100, 200, 400; 120; G-FR2-A1-3; EISREFSENS_50M + 3 + ΔFR2_REFSENS. FR2-2; 100,400; 120; G-FR2-A1-3; EISREFSENS_50M + 3 + ΔFR2_REFSENS. 400, 800, 1600; 480; G-FR2-A1-6; EISREFSENS_50M + [9] + ΔFR2_REFSENS. 400, 800, 1600, 2000; 960; G-FR2-A1-7; EISREFSENS_50M + [9] + ΔFR2_REFSENS. | Proposal 1: confirm the 9dB scaling due to wider CHBW. |

# Conclusions

Consider the above Proposal 1 for the final decisions on the remaining BS RF Rx requirements for FR2-2.

# References

[1]		R4-2203016 		WF on BS RF Tx requirements, RAN4#101bis-e

[2]		R4-2203017		WF on BS RF Rx requirements, RAN4#101bis-e