3GPP TSG-RAN WG4 Meeting#102-e	R4-2206365

# Electronic meeting, 21 February – 3 March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unwanted emissions requirements for Band n102 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_6GHz_unlic_EU-Core |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce unwanted emissions requirements for n102 starting with the requirements applicable in the EU for LPI (Low Power Indoor) devices in the range 5945-6425 MHz. The applicable requirements are specified in the EC Decision 2021/1067 and the (draft) EN 303 687. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.5F.3.3.x (new): unwanted emissions requirement are specified consistent with the EN 303 687 for operations with LPI devices in Europe. A tentative NS value “NS_57” is used for indicating these additional requirements. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unwanted emissions for operations in the range 5945.6425 MHz in the EU is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5F.3.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes made against the running CR in R4-2202422 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: NS_58 is used for n102 (LPI operation), the additional requirements clarified and the complete table for applicable spurious emissions requirements inserted. |  |  |  |  |  |  |  |  |

< start of changes >

##### 6.5F.3.3.5	Requirements for network signalling value "NS_53" or "NS_54"

When "NS_53" or "NS_54" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5F.3.3.5-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

##### 6.5F.3.3.x			Requirement for network signalling value "NS_58"

When "NS_58" is indicated in the cell, the power of any UE emission for channels assigned within 5945-6425 MHz shall not exceed the levels specified in Table 6.5F.3.3.x-1 and Table 6.5F.3.3.x-2. The requirements in Table 6.5F.3.3.x-1 apply for frequency ranges that are greater than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth, whereas the requirement in Table 6.5F.3.3.x-2 applies for frequency ranges that are less than the said FOOB (MHz) from the edge of the channel bandwidth.

Table 6.5F.3.3.x-1: Additional requirements in the spurious domain

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 30 ≤ f < 87.5 | −36 | 100 kHz |
| 87.5 ≤ f ≤ 118 | −54 | 100 kHz |
| 118 < f < 174 | −36 | 100 kHz |
| 174 ≤ f ≤ 230 | −54 | 100 kHz |
| 230 < f < 470 | −36 | 100 kHz |
| 470 ≤ f ≤ 694 | −54 | 100 kHz |
| 694 < f ≤ 1000 | −36 | 100 kHz |
| 1000 < f ≤ 26000 | −30 | 1 MHz |

Table 6.5F.3.3.x-2: Additional requirement in the out-of-band domain

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 5945 – FOOB ≤ f ≤ 5935 | -22 | 1 MHz |

### 6.5F.4	Transmit intermodulation

< end of changes >