3GPP TSG-RAN WG4 Meeting #102-e 	R4-2206397

# Online, 21st Feb – 3rd March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-1 on MPR requirements for intra-band contiguous ULCA for shared spectrum access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Skyworks Solutions Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR-U contiguous ULCA |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduce MPR requirements for intra-band contiguous ULCA for shared spectrum access |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce dedicated section and tentative MPR requirements for intra-band contiguous ULCA for shared spectrum access. Requirements and applicable configurations are introduced in [] to allow time to interested companies to further check. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for intra-band contiguous ULCA for shared spectrum access are included in 38.101-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

~~~~~~~~~~~~~~~~ Start of change

#### 6.2F.2A.2	UE maximum output power reduction for intra-band contiguous CA

For intra-band contiguous carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.1-1 with contiguous RB allocation is specified in Table 6.2F.2A.2-1 and Table 6.2F.2A.2-2 for UE power class 5 CA bandwidth classes B and C. For UE maximum output power reduction, the general requirements of clause 6.2.2 do not apply but instead the UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations for power class 5 according to Table 6.2F.2A.2-1 and Table 6.2F.2A.2-2.

Table 6.2F.2A.2-1 Maximum power reduction (MPR) for power class 5 shared spectrum access intra-band contiguous CA for bandwidth class B and class C.

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for Full5 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ [3.0] |  | ≤ [4.0] |
|  | QPSK | ≤ [3.0] |  | ≤ [4.0] |
|  | 16 QAM | ≤ [3.5] |  | ≤ [4.5] |
|  | 64 QAM | ≤ [4.5] |  | ≤ [5.0] |
|  | 256 QAM | ≤ [6.0] |  | ≤ [6.0] |
| CP-OFDM | QPSK | ≤ [4.0] |  | ≤ [5.5] |
|  | 16 QAM | ≤ [4.5] |  | ≤ [6.0] |
|  | 64 QAM | ≤ [6.0] |  | ≤ [6.5] |
|  | 256 QAM | ≤ [7.0] |  | ≤ [7.0] |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration [B] in Table 6.2F.2A.2-2. |  |  |  |  |

- Table 6.2F.2A.2-2 MPR mapping for intra-band CA wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration [CC1-CC2] |  |
| --- | --- | --- |
|  | A | B |
| 20+20 | 1-1 | None |
| 20+40 | 1-11, 1-10 |  |
| 20+60 | 1-111, 1-10, 1-100 |  |
| 20+80 | 1-1111, 1-1110, 1-1100, 1-1000 |  |
| 40+20 | 11-1, 01-1 |  |
| 40+40 | 11-11, 11-10, 01-11, 01-10 |  |
| 40+60 | 11-111, 11-110, 11-100, 01-111, 01-110, 01-100 |  |
| 40+80 | 11-1111, 11-1110, 11-1100, 11-1000, 01-1111, 01-1110, 01-1100, 01-1000 |  |
| 60+20 | 111-1, 011-1, 001-1 |  |
| 60+40 | 111-11, 111-10, 011-11, 011-10, 001-11, 001-10, |  |
| 60+60 | 111-111, 111-110, 011-111, 011-110, 011-100, , 001-110, 001-100 | [111-100], [001-111] |
| 60+80 | 111-1111, 111-1110, 111-1100, 111-1000, 011-1111, 011-1110, 011-1100, 011-1000, 001-1111, 001-1110, 001-1100, 01-1000, | None |
| 80+20 | 1111-1, 0111-1, 0011-1, 0001-1 |  |
| 80+40 | 1111-11, 1111-10, 0111-11, 0111-10, 0011-11, 0011-10, 0001-11, 0001-10 |  |
| 80+60 | 1111-111, 1111-110, 1111-100, 0111-111, 0111-110, 0111-100, 0011-111, 0011-110, 0011-100, 0001-111, 0001-110, 0001-100 |  |
| 80+80 | 1111-1111, 1111-1110, 1111-1100, 0111-1111, 0111-1110, 0111-1100, 0111-1000, 0011-1111, 0011-1110, 0011-1100, 0011-1000, 0001-1110, 0001-1100, 0001-1000 | 1111-1000, 0001-1111 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |

~~~~~~~~~~~~~~~~ End of change