# 3GPP TSG-RAN WG4 Meeting # 102-e                                                           R4-2206399

# Electronic Meeting, February 21 – March 3, 2022

# Source: 	ZTE Corporation, Qualcomm

# Title: 	WF on MSD for CA_n28A-n40A-n41A

# Agenda Item:	9.11.2

# Document for:	Approval

- Background

In this meeting, a TP on CA_n28A-n40A-n41A was provided in [1], where in terms of the co-existing analysis, IMD4 MSD caused by 2UL CA_n40A-n41A may fall into band n28 own DL Rx range should be defined.

In terms of the 1st round email discussion, IMD4 MSD values are needed to be further check.

In this way forward, the RF architecture assumptions, IMD4 MSD test points, etc. are captured.

- Way forward

2.1  RF architecture assumptions

The RF architecture assumption for CA n28+n40+n41 is shown as follow:

Where no simultaneous RX/TX for n40 and n41, which is consistency with 2DL CA. Architectural adjustments to accommodate 4RX operation should be studied.

Accordingly,  the assumption for some of the RF parameters for the IMD4 MSD are listed as table 1 and table 2

Table 1:	General RF component assumption

| Component | IP4 (dBm) |
| --- | --- |
| Switch | 56 |
| PA forward mixing | [28] |
| PA reverse mixing | 32 |
| LNA | -6 |

Table 2:	Some RF components assumption

| Component | Values (dB) |
| --- | --- |
| Antenna Isolation | 10 |
| PCB isolation | 60 |
| N-plexer isolation | 15 |
| n40 filter attenuation @n28 Rx | 35 |
| n41 filter attenuation @n28 Rx | 35 |
| LNA to Ant port | 4 |

2.2 IMD4 MSD

Using the following test points to estimate the MSD value:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n40-n41 | n28 | 710 | 5 | 25 | 765 | FFS | FDD | IMD4 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |

References

[1] R4-2204754, TP for TR38.717-03-02_CA_n28A-n40A-n41A, ZTE Corporation

[2] R4-22xxxxx, Issues arising from basket WIs but not subject to block approval, Moderator (Dominique Brunel (Skyworks Solutions Inc.)