3GPP TSG-RAN WG4 Meeting #102-e	R4-2206935

Electronic Meeting, Feb 21 – Mar 3, 2022

Title: 	WF on other RRM requirements for FeMIMO

Source: 	Huawei, HiSilicon

Agenda item:	10.19.3

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM in RAN4 #102-e meeting.

Way-forward

- Issue: Requirement for TRP specific Beam Failure Recovery

- Agreements

  - RRM requirements for TRP-specific BFR should be specified for FR1 in R17 and do not introduce sharing factor PTRP in FR1;

  - Introduce PTRP = 2 in FR2 for overlapping resources for equal sharing between BFD/ CBD resources (SSB and CSI-RS) between the 2 TRPs.

    - FFS the conditions on applying PTRP = 2 in FR2

- Issue: QCL definition

- Agreements

  - In R17, UL TCIs are only applicable to UL signals/channels, and UL RSs cannot be used as source RSs of DL TCIs or joint TCIs.

  - In Rel-17, DL TCI chain and UL TCI chain need to be defined for deriving the QCL information of PDCCH/PDSCH and PUCCH/PUSCH respectively.