3GPP TSG-RAN WG4 Meeting #102-e                     	R4-2206972

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RedCap UE in clause 9.11A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of 1 Rx RedCap requirements for autonomous measurement gaps due to CGI reading are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Rx RedCap UE preliminary requirements for autonomous measurement gaps due to CGI reading are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarity which RRM requirements apply to 1 Rx RedCap UE. Possible misinterpretation that legacy RRM requirements are applicable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2205938. |  |  |  |  |  |  |  |  |

--------  Start of First Change  --------

## 9.11A	NR measurements with autonomous gaps for RedCap UE

### 9.11A.1	Introduction

The requirements in this clause are applicable for CGI identification of an intra frequency and inter frequency NR target cell for 1 Rx RedCap UE in FR1. For 2 Rx RedCap UE, the requirements in clause 9.11 apply.

The requirements in this clause are specified for CGI identification of an NR target cell and are applicable for 1 Rx RedCap UE in FR1:

-	in RRC_CONNECTED state, and

-	configured with SA operation mode and no SCell(s).

The overall CGI reporting delay is defined in clause 9.11A.3.

### 9.11A.2	CGI identification of an NR cell with autonomous gaps

Editor’s Note: The scheduling restriction requirement of CGI reading will be captured in this section later.

The RedCap UE shall identify and report the CGI of a known NR target cell when requested by the network for the purpose of reportCGI. Only one cell is provided to the RedCap UE with cellForWhichToReportCGI for identifying the CGI.The RedCap UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that a RedCap UE is not required to use autonomous gap if useAutonomousGaps is set to false. If autonomous gaps are used for measurement with the purpose of reportCGI, regardless of whether DRX is used or not, the RedCap UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI, RedCap = (TMIB, RedCap + T SIB1,RedCap) ms

Where:

TMIB,RedCap is the time period used to acquire MIB message. TMIB,RedCap = [TBD] * TSMTC ms for target cell carrier frequency on FR1 and TMIB,RedCap = [TBD] * TSMTC ms for target cell carrier frequency on FR2.

TSIB1,RedCap is the time period used to acquire SIB1 message. TSIB1,RedCap = [TBD] * TRMSI-scheduling ms.

Where TSMTC is the SMTC periodicity configured for the target cell measurement, and TRMSI-scheduling is

-	the periodicity with which the SIB1 is actually transmitted by the NR target cell when SSB and RMSI CORESET multiplexing pattern is 1

-	the maximum between the periodicity with which the SIB1 is actually transmitted by the NR target cell and TSMTC when SSB and RMSI CORESET multiplexing pattern is 2 or 3.

The requirement for identifying the CGI of an NR cell within Tidentify_CGI,RedCap is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.

Within the time Tidentify_CGI,RedCap, over which the RedCap UE identifies the CGI of an NR cell, the RedCap UE shall fulfil interruption requirements specified in clause 8.2A.

In the requirement a cell is known if,

-	During the last 5 seconds for FR1 or 3 seconds for FR2 before the reception of the report CGI command:

-	The RedCap UE has sent a valid L3-RSRP measurement report with SSB index for the target cell and

-	During MIB decoding at least reported SSBs remains detectable according to the cell identification conditions specified in clauses 9.2B or 9.3B of TS 38.133, and

-	During SIB1 decoding the SSB used for MIB decoding remains detectable according to the cell identification conditions specified in clauses 9.2B or 9.3B of TS 38.133, and

-	During MIB decoding, the SSB for MIB decoding remains detectable with SNR ≥[-3]dB

-	During SIB1 decoding, the PDSCH for SIB1 decoding remains detectable with SNR ≥[-3]dB

### 9.11A.3	CGI reporting delay

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the RedCap UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for RedCap UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI,RedCap defined in clause 9.11A.2 plus RRC procedure delay defined in clause 12 in TS 38.331 [2], and additional 20ms margin if target cell is on FR2.

--------  End of First Change  --------