3GPP TSG-RAN WG4 Meeting # 102-bis-e 												R4-2207209

Electronic Meeting, 21th Feb– 3th March, 2022

Agenda item:			10.19.4

Source:	Samsung

Title:	WF on CSI requirement for Rel-17 FeMIMO

Document for:	Approval

Background

- R4-2203090, “WF on general and CSI requirement for Rel-17 FeMIMO”, Samsung. RAN4#101-bis-e meeting

- R4-2203091, “WF on demodulation requirement for Enhancement on HST-SFN deployment”, Samsung, RAN4#101-bis-e meeting

- R4-2203092, “WF on demodulation requirement for Enhancement on Multi-TRP”, Huawei, HiSilicon, RAN4#101-bis-e meeting

CSI reporting requirement for multi-TRP

Issue 3-1-1: Test cases for CSI reporting enhancement for m-TRP transmission

- Define PMI reporting requirement for single-DCI based Multi-TRP scheme with full overlapped resource allocation (SDM) only in FR1

- FFS on  additional CSI reporting requirement for single/multi-DCI based Multi-TRP scheme

  - Option 1

    - Option 1a: Define new CSI reporting requirement for CQI reporting for Multi-DCI based Multi TRP scheme

    - Option 1b : Define RI, CQI reporting requirement for single-DCI based Multi-TRP, and define CQI reporting requirement for multi-DCI

  - Option 2: Not define RI, CQI reporting requirement for single-DCI, Not define CQI reporting requirement for multi-DCI

  - Option 3: if time allows define CQI reporting requirement for multi-DCI

Issue 3-2-1: Common simulation assumption

- Channel and correlation models:  TDLA30-10 with XP High with statistically independent for each TRP

- Pc setting: Same Pc ratios for each TRP in defining requirement

- SNR setting: The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP

Issue 3-2-2: General test set-up for CSI reporting

- 2 TPs configured with fully overlapping resource allocation

- One CSI-RS resource with Ks = 2

  - TP1 associated with NZP-CSI-RS resource 1

  - TP2 associated with NZP CSI-RS resource 2

- CSI reporting: One CSI associated with multi-TRP measurement hypothesis and X=0 CSI associated with single-TRP measurement hypothesis (CSI reporting mode 1 with X=0)

  - CMR group 1 {CMR a} corresponding to NZP CSI-RS resource 1, K1=1

  - CMR group 2 {CMR b} corresponding to NZP CSI-RS resource 2, K2=1

  - CMR pair (N=1) : CMR {a,b} for M-TRP measurement hypothesis

- No time/frequency offset between two TPs

- WB PMI reporting for mode 1 with X=0

- RAN4 apply the above test setup as baseline for CSI reporting requirement definition

Issue 3-2-3: CSI resource configuration

- Configure two resources in a resource pair in the same slot for CSI reporting requirements for mTRP.

Issue 3-2-4: Number of CSI-RS Ports

- Option 1

  - 8   for each TRP

- Option 2

  - 4  for each TRP

- Companies are encouraged to provide the simulation result in the next meeting with both option 1 and option 2 base on agreed simulation assumption to check the performance gain with the test metric, i.e. throughput ratio with a given SNR test. Make the decision about number of CSI-RS ports in next meeting.

Issue 3-2-5: Number of layers

- Number of layers: 2 (1MIMO layer per TRP)

Issue 3-2-6: Test metric for PMI reporting

- Apply test metric of TP ratio follow PMI and random PMI with m-TRP reporting. The layer for random PMI per TRP should be orthogonal

Issue 3-2-7: Performance evaluation

- Interested companies can provide the performance evaluation of PMI reporting with enhanced CSI reporting against single PMI reporting for multi-TRP transmission. No impact on the PMI reporting requirement definition for single-DCI based Multi-TRP.

Issue 3-2-8: Codebook Structure

- Reusing the existing Rel-15 PMI requirement setup: i.e, type I single panel for PMI reporting requirement for single-DCI based Multi-TRP

PMI reporting requirement for Rel-17 enhanced ype II PS codebook

Issue 4-1-1: Whether to define PMI requirement for Rel-17 FeTye II PS codebook

- Option 1 : Yes

  - Option 1a: Define PMI reporting requirement for Rel-17 FeTypeII port selection codebook based on evaluation on the performance gain over eTypeII codebook.

  - Option 1b: Consider defining PMI requirement for Rel-17 eType II port selection only if RAN4 can reach an agreement on a simplified way of testing with SU-MIMO test set-up, otherwise not to define requirement.

- Option 2 : No

- Companies are encouraged to provide the detail test setup and test metric , also including gNB implementation model in the next meeting

Issue 4-2-1: General Test seup of PMI reporting requirement

- Option 1 : Both SU-MIMO and MU-MIMO

- Option 2 :  SU-MIMO

Issue 4-2-2: Modelling BF CSI-RS Port:

- Option 1

  - Option 1a: MIMO fading channel as Rel-13 LTE Class B K=1 PMI test cases

  - Option 1b: Power scaling method similar as Rel-13 LTE Class B K>1 CRI test case

- Other options are not precluded

- Apply option 1 as a starting point for initial evaluation in next meeting

Other

Issue 5-1-1: whether to define PMI reporting requirement for inter-cell interference scenario in Rel-17 FeMIMO

- FFS to define PMI reporting requirement with inter-cell interference in Rel-17 FeMIMO WI

- FFS on where to handle the PMI reporting requirement with inter-cell interference

  - Option 1: Rel-17 TEI

  - Option 2: Rel-18 timeframe

  - Option 3 : Rel-17 FeMIMO WI

Issue 5-1-2: PMI reporting with inter-cell interference evaluation assumption

- Option 1: For further evaluation of PMI reporting in ICI use the following simulation assumptions as baseline

  - Antenna config: 8x2 XP High

  - Prop. Channel model: TDLA30-5; ensure that channel from target and interference cell are statistically independent and have different beam direction (to ensure PMI are different)

  - NZP CSI-RS for interference:

    - Overlapping with serving cell

    - Non-overlapping with serving cell

  - CSI-IM for interference: non overlapping with CSI-IM for serving cell

  - Loading for interference cell: PDSCH transmission is enabled in all slots for interference cell

  - Evaluate performance based on TP ratio with and with ICI for (1) overlapping NZP CSI-RS (2) non-overlapping NZP CSI-RS

- Other options are not precluded

Issue 5-1-3: Test metric of PMI reporting with inter-cell interference

- Option 1

  - TP ratio with following PMI with inter-cell interference and follow PMI without interference

- Other options are not precluded

Reference

- R4-2207160, Email discussion summary fo [102-e][320] NR_HST_FR2_Demod, Samsung, RAN4#102-e meeting