3GPP TSG-RAN WG4 Meeting # 102-e              									     R4-2207346

Electronic Meeting, 21st of February – 3rd of March, 2022

Agenda item:			10.13.1, 10.13.1.1, 10.13.1.2, 10.13.1.3, 10.13.1.4

Source:	Moderator (THALES)

Title:	Way Forward on NTN_solutions_Part1

Document for:	Agreement

# Introduction

This proposed way forward is based on the outcomes of “Email discussion summary for [102-e][308] NTN_Solutions_Part1”, see R4-2207438 (revision of R4-2207165). Moreover, as suggested by “RAN4#102-e E-meeting Arrangements and Guidelines”, this Way Forward uses WORD document rather than POWERPOINT (.PPTX) in order to facilitate others to comment and easily track the changes.

As described in R4-2207438, Email discussion summary for [102-e][308] NTN_Solutions_Part1 identified the following topics and issues:

- Topic #1: NTN Satellite System Parameters/Features

  - Issue 1-1-1: UE feature for NTN – PDSCH

  - Issue 1-1-2: UE feature for NTN - PUSCH

- Topic #2: NTN SAN Class

  - Issue 2-1-1: Satellite Access Node Class (SAN Class)

  - Issue 2-1-2: SAN number of classes

  - Issue 2-1-3: SAN class differentiation

- Topic #3: TS (writing) assumptions

  - Issue 3-1-1: General approach to write NTN TS (i.e. TS 38.108 and TS 38.101-5)

  - Issue 3-1-2: General approach to write NTN TS (i.e. TS 38.108 and TS 38.101-5)

- Topic #4: pCRs/CRs

  - Issue 4-1-1: pCR to TR 38.863 – see R4-2205232 (HUGHES Network Systems Ltd)

  - Issue 4-1-2: pCR to TR 38.863 – see R4-2205111 (Ligado Networks)

  - Issue 4-1-3: pCR to TR 38.863 – see R4-2205314 (HUGHES Network Systems Ltd)

  - Issue 4-1-4: pCR to TR 38.863 – see R4-2205555 (Nokia, Nokia Shanghai Bell)

  - Issue 4-1-5: pCR to TS 38.863 – see R4-2205921 (THALES)

  - Issue 4-2-1: pCR to TS 38.108 – see R4-2203953 (CATT)

  - Issue 4-2-2: pCR to TS 38.108 – see R4-2205673 (THALES)

  - Issue 4-2-3: pCR to TS 38.108 – see R4-2205730 (THALES)

  - Issue 4-2-4: pCR to TS 38.108 – see R4-2205667 (THALES)

  - Issue 4-2-5: pCR to TS 38.108 – see R4-2203952 (CATT)

  - Issue 4-2-6: pCR to TS 38.108 – see R4-2205437 (HUGHES Network Systems Ltd)

  - Issue 4-2-7: pCR to TS 38.108 – see R4-2205476 (ZTE Corporation)

  - Issue 4-3-1: pCR to TS 38.101-5 – see R4-2204507 (Qualcomm Incorporated)

  - Issue 4-3-2: pCR to TS 38.101-5 – see R4-2205671 (THALES)

  - Issue 4-3-3: pCR to TS 38.101-5 – see R4-2205672 (THALES)

  - Issue 4-3-4: pCR to TS 38.101-5 – see R4-2205472 (ZTE Corporation)

  - Issue 4-4-1: CR to TS 38.104 – see R4-2204195 (Softbank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat)

- Topic #5: HAPS Generalities

  - Issue 5-1-1: NR operating band for HAPS

  - Issue 5-1-2: Description of HAPS operating band to TS 38.104

  - Issue 5-2-1: HAPS UE

# Agreements

- Topic #1: NTN Satellite System Parameters/Features

  - Issue 1-1-1: UE feature for NTN – PDSCH

  - Issue 1-1-2: UE feature for NTN – PUSCH

GTW Agreement (28/02/2022) for 1-1-1, 1-1-2:

Supporting UL/DL 64QAM for NTN operation as optional with per band

- Topic #2: NTN SAN Class

  - Issue 2-1-1: Satellite Access Node Class (SAN Class)

  - Issue 2-1-2: SAN number of classes

  - Issue 2-1-3: SAN class differentiation

GTW Agreement (23/02/2022) for 2-1-1, 2-1-2, 2-1-3:

- Introduce SAN classes in Rel-17: LEO and GEO

- Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission]

- Note: Introducing additional classes in future release not precluded.

- Topic #3: TS (writing) assumptions

  - Issue 3-1-1: General approach to write NTN TS (i.e. TS 38.108 and TS 38.101-5)

Proposal 3-1-1-1: Capture the following note in TR 38.863 (and not in TS 38.108 and TS 38.101-5):

  - Note: Satellite operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band.

  - Issue 3-1-2: General approach to write NTN TS (i.e. TS 38.108 and TS 38.101-5)

Proposal 3-1-2-1: Capture the following note in the scope sub-clause of TR 38.863 (and not in TS 38.108 and TS 38.101-1):

  - Note: Satellite operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band.

- Topic #4: pCRs/CRs

- See decisions captured in R4-2207438, Email discussion summary for [102-e][308] NTN_Solutions_Part1

- Topic #5: HAPS Generalities

  - Issue 5-1-1: NR operating band for HAPS

Proposal 5-1-1-1: Specify that HAPS can be deployed in band n1.

  - Issue 5-1-2: Description of HAPS operating band to TS 38.104

## 5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n41, n65, n66, n70, n71, n74, n90 which are defined in Table 5.2-1.

Proposal 5-1-2-1: Add the description of HAPS operating band to TS 38.104 clause 5.2 as below.

  - Issue 5-2-1: HAPS UE

Proposal 5-2-1-1: NR UEs as defined by current TS 38.101-1 can support HAPS deployments with no additional changes needed in TS 38.101-1

# Appendix: Submitted documents for [102-e][308] NTN_Solutions_Part1

A total of 19+3 TDocs have been identified for discussion in [102-e][308] NTN_Solutions_Part1 (please also see the Appendix for the details, with all the observations/proposals):

| TDoc Number | TDoc Type | Title | Company | Status | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- | --- |
| R4-2203964 | discussion | UE feature for NTN | CATT | available | Discussion | 10.13.1 |
| R4-2205232 | pCR | TP TR 38.863 7.4.1 NTN UE Requirement (General) | HUGHES Network Systems Ltd | available | Approval | 10.13.1 |
| R4-2205554 | discussion | On NTN System parameters | Nokia, Nokia Shanghai Bell | available | Approval | 10.13.1.1 |
| R4-2203953 | pCR | TP for 38.108: clause 5.3&5.4 on system parameters | CATT | available | Approval | 10.13.1.1 |
| R4-2204507 | pCR | TP on TS 38.101-5 for UE channel bandwidth and channel arrangement | Qualcomm Incorporated | available | Approval | 10.13.1.1 |
| R4-2203956 | pCR Moved to [310] | TP for 38.108: clause 9.3 OTA Satellite Access Node output power | CATT | Available Moved to [310] | Approval | 10.13.1.2 |
| R4-2205673 | pCR | Draft text proposal for Clause 4.3 Requirement reference points - TS 38.108 | THALES | available | Approval | 10.13.1.2 |
| R4-2205730 | pCR | Draft text proposal for Clause 4.4 Satellite Access Node classes - TS 38.108 | THALES | available | Approval | 10.13.1.2 |
| R4-2205048 | other | NTN - SAN class | Ericsson | available | Approval | 10.13.1.2 |
| R4-2205111 | pCR | TP for TR 38.863: Regulatory aspects for NTN satellite access nodes and UEs operating in UL1626.5-1660.5 MHz and DL 1525-1559 MHz frequencies ranges | Ligado Networks | available | Approval | 10.13.1.3 |
| R4-2205314 | pCR | TP to TR 38.863 on Section 5.2 NTN Satellite band | HUGHES Network Systems Ltd | available | Approval | 10.13.1.3 |
| R4-2205733 | pCR withdrawn | Draft text proposal for Clause 4.5 Regional requirements - TS 38.108 | THALES | withdrawn | Approval | 10.13.1.3 |
| R4-2205555 | pCR | TP to TR 38.863 Regulatory aspects for HAPS | Nokia, Nokia Shanghai Bell | available | Approval | 10.13.1.3 |
| R4-2205671 | pCR | Draft text proposal for Clause 3 - TS 38.101-5 | THALES | available | Approval | 10.13.1.4 |
| R4-2205672 | pCR | Draft text proposal for Clause 4 - TS 38.101-5 | THALES | available | Approval | 10.13.1.4 |
| R4-2205667 | pCR | Draft text proposal for Annex B - TS 38.108 | THALES | available | Approval | 10.13.1.4 |
| R4-2205921 | pCR | Draft text proposal for Clauses 7, 7.1, 7.2, 7.3 in TR 38.863 | THALES | available | Approval | 10.13.1.4 |
| R4-2205051 | other | NTN - General aspect related to TS assumptions | Ericsson | available | Approval | 10.13.1.4 |
| R4-2204195 | CR | CR for TS 38.104: capturing HAPS requirements | Softbank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat | available | Agreement | 10.13.1.4 |
| R4-2203952 | pCR | TP for 38.108: clause 4.3 requirement reference point | CATT | available | Approval | 10.13.1.4 |
| R4-2203536 | discussion | Considerations on HAPS operating band(s) | SoftBank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat | available | Decision | 10.13.1.4 |
| R4-2205437 | pCR | TP to TR 38.108 on 4.5 Regional Requirement | HUGHES Network Systems Ltd | available | Approval | 10.13.3 (added for discussion in [308] as asked by the chairman) |
| R4-2205476 | Other -> pCR | TP for TS 38.108: Genera(5.1) and Operating Band(5.2) | ZTE Corporation | available | Approval | 10.13.3 (added for discussion in [308] as asked by the chairman) |
| R4-2205472 | Other -> pCR | TP for TS 38.101-5: Genera(5.1) and Operating Band(5.2) | ZTE Corporation | available | Approval | 10.13.4.1 (added for discussion in [308] as asked by the chairman) |

Moderator note1: There are 5 pCRs to TR 38.863, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note2: There are 5+2 pCRs to TR 38.108, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note3: There are 3+1 pCRs to TR 38.101-5, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note4: There is 1 CR to TR 38.104, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note5: TDoc R4-2203956 has been moved from [102-e][308] NTN_Solutions_Part1 discussion list in [102-e][310] NTN_Solutions_Part3.

Moderator note6: TDocs R4-2205437 and R4-2205476 have been moved from [102-e][310] NTN_Solutions_Part3 discussion list in [102-e][308] NTN_Solutions_Part1.

Moderator note7: TDoc R4-2205472 has been moved from [102-e][311] NTN_Solutions_Part4 discussion list in [102-e][308] NTN_Solutions_Part1.