3GPP TSG-RAN WG4 Meeting # 102-e	R4-2207382

Electronic Meeting, 21 Feb – 03 Mar, 2022

Source: 	Huawei

Title: 	TP to TS 38.108: section 7.6 (Rx spur) and section 7.7 (Rx IMD)

Agenda Item:	10.13.3.4

Document for:	Approval

# Introduction

Based on the worksplit agreed in [1] (Issue 3-3-2), in this contribution a TP to TS 38.108 section 7.6 (Rx spur) and section 7.7 (Rx IMD) is provided for approval.

TS 38.108 skeleton in [2] was used as the starting point.

As the Rx spurious emissions requirements was subject to the emission scaling in the NR specification, please refer to the related discussion paper in [3]. As multiple open issues were identified, part of the proposed TP text is captured in [] until the technical conclusion is reached.

# Conclusions

Proposal 1: Approve the attached TP to TS 38.108.

# References

[1]		R4-2203080	Way Forward on NTN_solutions_Part1, RAN4#101bis-e

[2]	R4-2203087	Skeleton for TS 38.108 NR Satellite Access Node radio transmission and reception v0.0.1, RAN4#101bis-e

[3]	R4-2205978	Discussion on the AAS architecture and consideration of the emissions scaling

# Annex A: TP to TS 38.108

------------------------------ Modified sections ------------------------------

### 7.6.1	General

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at the TAB connector of the SAN type 1-H. The requirements apply to all SAN with separate RX and TX TAB connectors.

NOTE:	In this case for FDD operation the test is performed when both TX and RX are ON, with the TX TAB connectors terminated.

### 7.6.2	Minimum requirement for SAN type 1-H

The receiver spurious emissions basic limits are provided in table 7.6.2-1.

Table 7.6.2-1: General SAN receiver spurious emissions limits

| Spurious frequency range | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz | -47 dBm | 1 MHz | Note 1, Note 2 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: The frequency range from ΔfOBUE below the lowest frequency of the SAN transmitter operating band to ΔfOBUE above the highest frequency of the SAN transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.6.1. |  |  |  |

## 7.7	Receiver intermodulation

The requirement is not applicable in  this version of the specification.

------------------------------ End of modified section ------------------------------