3GPP TSG-RAN WG4 Meeting #102-e    				     R4-2207388

Electronic Meeting, 21st of February - 3rd of March, 2022

Agenda item:	     10.13.1.2

Source:	THALES

Title:	Operating band unwanted emissions (Draft text proposal for OBUE - TS 38.108 and TR 38.863)

Document for:	Approval

# Introduction

Draft TS skeleton for Non-terrestrial networks (NTN) related to NR; Satellite Access Node radio transmission and reception (TS 38.108) has been approved in R4-2203087 [1] and the work split has been summarized in R4-2203080 Way Forward on NTN_solutions_Part1 [2] approved in RAN4 101-bis-e meeting. For further information, please also follow the discussions in R4-2203111 [3].

In this document, following changes have been proposed as a TP to update TS 38.108:

- Section x.x: SAN Operating Band Unwanted Emissions (OBUE)

In this contribution, we follow methodology provided in R4-2205468 (“Further discussion on conducted Tx requirements of satellite access node”, ZTE) and adapted it by taking into account the following RAN4 102-e GTW decisions (23/02/2022):

Agreement GTW discussion [102-e][308]:

Introduce SAN classes in Rel-17: LEO and GEO

Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission]

Note: Introducing additional classes in future release not precluded.

Agreement GTW discussion [102-e][310]:

Measurement bandwidth: 4kHz

General principle: Follow ITU recommendation 15-41 (SM.1541) at least for the frequency offset range within first two break points

Define two set requirements: one for LEO SAN class and one for GEO SAN class

As a reminder, R4-2205468 provided the following information:

- Table 1. GEO UEM limit values

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz /4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | 51dBm-[24dBc/GEO ACLR]-10*log10(5*10)+1dB margin=[11dBm] | 100 kHz /4kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | [10dBm/-13dBm with 4kHz] | 100 kHz /4kHz |

- Table 2. LEO-1200 OBUE requirements

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | 53dBm-24dBc-10*log10(5*10)+1dB margin=13dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | 12 | 100 kHz |

- Table 3. LEO-600 OBUE requirements

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | 47dBm-24dBc-10*log10(5*10)+1dB margin=7dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | 6 | 100 kHz |

For the reasons mentioned above, we propose new OBUE limit values for SAN, taking into account SAN class definition and new measurement bandwidth values.

# References

[1] R4-2203087 Skeleton for TS 38.108 NR Satellite Access Node radio transmission and reception v0.0.1, THALES.

[2] R4-2203080 Way Forward on NTN_solutions_Part1, THALES.

[3] R4-2203111 Email discussion summary for [101-bis-e][306] NTN_Solutions_Part1, THALES.

# OBUE SAN Proposal

Based on 2 class SAN definition, 4 kHz measurement bandwidth (as in SM.1541-6), SAN power of 51 dBm in 20 MHz (GEO class) and 53 dBm in 20 MHz (LEO class), we further define the SAN OBUE requirement for GEO and LEO classes as following :

- Table 1. SAN GEO class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 3.0dBm-25foffsetMHz-0.002dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 51dBm-[16dB]-10*log(20000/4) +3dB_margin= 1 dBm with GEO ACLR=[16 dB] TBC | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |

- Table 2. SAN LEO class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 5.0dBm-105foffsetMHz-0.002dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 53dBm-[24dB]-10*log(20000/4) +3dB_margin= -5 dBm with LEO ACLR=[24 dB] TBC | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |

With respect to the last line in each table, is important to mention that for MSS S-Band n256 and MSS L-Band n255 the operational bandwidth is maximum 30 MHz. For this reason, according to the table 6.6.1-1 from TS 38.104, the maximum frequency offset of OBUE outside the DL operating band is limited to 10 MHz, since the satellite operational band B is below 100 MHz bandwidth (if we follow the same methodology as in TS 38.104).

On the other hand, if the last row uses the spurious emission limit beyond 10 MHz, using SM.329-12 for P > 50W (4th row from Table 10), one can obtain:

1) -9 dBm (i.e. 21-30 [dBm] = -9 dBm) spurious emission power over 4 kHz measurement bandwidth for the GEO class, and

2) -7 dBm (i.e. 23-30 [dBm] = -7 dBm) spurious emission power over 4 kHz measurement bandwidth for the LEO class.

Please also note that in SM.329-12 the 43+log(P) attenuation is applicable only if transmitter power < 50W, and attenuation of 60 dBc is applicable for transmitter power > 50W.

# Text proposals for TS 38.108

------------------------------------------------<Start of TP>----------------------------------------------

## x.x		Operating band unwanted emissions for SAN

The SAN OBUE requirements for GEO and LEO classes are defined as described in Table x.x.-1 and Table x.x.-2.

Table x.x-1. SAN GEO Class OBUE limit values

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 3.0dBm-25foffsetMHz-0.002dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 1 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |

Table x.x-2. SAN LEO Class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 5.0dBm-105foffsetMHz-0.002dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -5 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |

------------------------------------------------<End of TP>-----------------------------------------------

# Text proposals for TR 38.863

------------------------------------------------<Start of TP>----------------------------------------------

## x.x		Operating band unwanted emissions for SAN

Based on 2 class definition, 4 kHz measurement bandwidth (as in SM.1541-6), SAN power of 51 dBm in 20 MHz (GEO class) and 53 dBm in 20 MHz (LEO class), we further define the SAN OBUE requirements for GEO and LEO classes as following :

Table x.x-1. SAN GEO Class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 3.0dBm-25foffsetMHz-0.002dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 51dBm-[16dB]-10*log(20000/4) +3dB_margin = 1 dBm with GEO ACLR=[16 dB] TBC | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |

Table x.x-2. LEO Class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 5.0dBm-105foffsetMHz-0.002dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 53dBm-[24dB]-10*log(20000/4) +3dB_margin = -5 dBm with LEO ACLR=[24 dB] TBC | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |

With respect to the last line in each table, is important to mention that for MSS S-Band n256 and MSS L-Band n255 the operational bandwidth is maximum 30 MHz. For this reason, according to the table 6.6.1-1 from TS 38.104, the maximum frequency offset of OBUE outside the DL operating band is limited to 10 MHz, since the satellite operational band B is below 100 MHz bandwidth (if we follow the same methodology as in TS 38.104).

On the other hand, if the last row uses the spurious emission limit beyond 10 MHz, using SM.329-12 for P > 50W (4th row from Table 10), we obtain:

1) -9 dBm (i.e. 21-30 [dBm] = -9 dBm) spurious emission power over 4 kHz measurement bandwidth for the GEO class, and

2) -7 dBm (i.e. 23-30 [dBm] = -7 dBm) spurious emission power over 4 kHz measurement bandwidth for the LEO class.

Please also note that in SM.329-12 the 43+log(P) attenuation is applicable only if transmitter power < 50W, and attenuation of 60 dBc is applicable for transmitter power > 50W.

------------------------------------------------<End of TP>-----------------------------------------------