3GPP TSG-RAN WG4 Meeting # 102-e					                                                                                                                     R4-2207432

Electronic Meeting, February 21 – March 3, 2022

Agenda item:			10.8.3

Source:	Moderator (CMCC)

Title:	Email discussion summary for [102-e][319] NR_HST_FR1_Demod

Document for:	Information

- Introduction

This email discussion focuses on UE demodulation for Rel-17 NR HST, including agenda 10.8.3.1 and 10.8.3.2. The agreed way forward in previous meeting is in R4-2202985.

The targets of email discussion for 1st round and 2nd round are:

- 1st round: discuss the open issues and strive to minimize the open issues

- 2nd round: according to 1st round discussion, discuss left open issues for 2nd round, and strive to minimize the open issues, and strive to approve the WF.

Topic #1 PDSCH requirements for CA scenarios

Agenda  10.8.3.1 and 10.8.3.2

Companies’ contributions summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2205080 | Summary for FR1 HST demodulation results | Ericsson |  |
| R4-2203762 | Discussion on PDSCH CA Requirements in HST | Apple | Proposal #1: Rel-17 FR1 HST PDSCH demod requirements for CA are defined as release independent from Rel-16 if RRM requirements are agreed to be release independent. Observation #1: There was no objective to introduce HST CA requirements for additional CBW of 35, 45MHz in FDD. Observation #2: If HST FR1 enhancements WI is aimed for early completion in March 2022, we don’t see how additional CBW can be introduced. Proposal #2: Do not introduce FDD requirements with additional CBW for HST CA unless there is need to support these CBW in HST deployments. |
| R4-2203763 | Draft CR on HST DPS CA requirements for 4Rx | Apple |  |
| R4-2204253 | Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx | CMCC |  |
| R4-2204259 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Proposal 1: for the UE capability for supporting HST-SFN CA in FR1, the granularity is per band combination. Proposal 2: it is proposed that Rel-17 FR1 HST-SFN CA requirements are release independent from Rel-15. Proposal 3: for FDD 15KHz SCS, specify PDSCH requirements on single carrier of BW of {35, 45} MHz for HST scenario. |
| R4-2204384 | Discussion on HST FR1 CA PDSCH performance requirements | Intel Corporation | Proposal #1: Apply HST-SFN CA requirements only from Rel-17. Proposal #2: Define HST-SFN CA feature with FSPC granularity. Proposal #3: Define HST CA requirements for 35 and 45 CBWs. |
| R4-2204385 | draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx | Intel Corporation |  |
| R4-2204430 | Discussion on PDSCH requirements for FR1 HST CA scenarios | ZTE Corporation | Proposal 1: To confirm that the UE capability granularity is per band combination. Proposal 2: Option 2 and option 3 are preferred for release independence for FR1 HST PDSCH SFN CA demodulation requirements. |
| R4-2205081 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Proposal 1: Rel-17 FR1 HST PDSCH SFN CA demodulation requirements are applicable from the same release as the Enhanced RRM requirements for FR1 CA in HST scenario. If the demodulation part needs to decide the applicable release, Rel-17 FR1 HST PDSCH SFN CA demodulation requirements are applicable from Rel-16. |
| R4-2205082 | draft CR: FRC for CA PDSCH demodulation requirements for HST | Ericsson |  |
| R4-2205752 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei,HiSilicon | Proposal 1: HST-SFN CA requirements are release independent from Rel-16. Proposal 2: Define PDSCH requirements on single carrier of BW of {35, 45} MHz for FDD 15kHz SCS. |
| R4-2205753 | Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4) | Huawei,HiSilicon |  |
| R4-2206090 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated |  |
| R4-2206110 | Draft CR on Applicability Rules for FR1 HST CA requirements | Qualcomm Incorporated |  |

Open issues summary

Issue 1-1: UE capability for HST-SFN CA

- Agreements in RAN4#101bis-e meeting:

  - Introduce a new capability for supporting HST-SFN CA in FR1

      - The granularity: [per band combination]

- Proposals in RAN4#102 meeting:

  - Option 1 (CMCC, ZTE): Per band combination

  - Option 2 (Intel): per FSPC

- Recommended WF

  - Can we agree with per band combination?

Issue 1-2: Release independent

- Agreements in RAN4#101bis-e meeting:

  - HST-DPS CA requirements are release independent from Rel-15

  - For Rel-17 FR1 HST PDSCH SFN CA demodulation requirements

      - Option 1: Applicable from Rel-17

      - Option 2: Aligned with RRM agreement

      - Option 3: Release independent from Rel-15 /16

- Proposals in RAN4#102 meeting:

  - HST-SFN CA requirements are release independent from Rel-15 (CMCC)

  - For Rel-17 FR1 HST PDSCH SFN CA demodulation requirements

      - Option 1: Applicable from Rel-17 (ZTE, Intel)

      - Option 2: Aligned with RRM agreement (ZTE, Apple, Ericsson)

      - Option 3: Release independent from Rel-15 /16 (ZTE)

        - Option 3a: Release independent from Rel-16 (Ericsson, Huawei)

- Recommended WF

  - Can we agree the following:

  - From demodulation perspective, release independent from Rel-16.

Issue 1-3: FDD BW

- Agreements in RAN4#101bis-e meeting:

  - FFS: for FDD 15KHz SCS, specify PDSCH requirements on single carrier of BW of {35, 45} MHz.

- Proposals in RAN4#102-e meeting:

  - Option 1 (Apple, Qualcomm): Do not introduce FDD requirements with additional CBW for HST CA unless there is need to support these CBW in HST deployments.

  - Option 2 (CMCC, Huawei, Qualcomm, Intel): For FDD 15KHz SCS, specify PDSCH requirements on single carrier of BW of {35, 45} MHz for HST scenario.

- Recommended WF

  - More discussion is neededs

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-1: We support the recommended WF, Option 1. We prefer the simple one. Issue 1-2: We support the recommended WF. Issue 1-3: We are fine to add CBW=35/45MHz for FDD SCS=15kHz CC. |
| Apple | Issue 1-1: UE capability for HST-SFN CA We understand Intel’s clarification and agree with the justification. The UE capability indication granularity should be either per UE or per band, per band combination (FS) or per band, per band combination, per CC (FSPC) for baseband related capabilities. Issue 1-2: Release independent We support the recommended WF. But would also like to add that demod reqt can be release independent if RRM are also release independent from Rel-16 or earlier. Issue 1-3: FDD BW We would like to understand if there is plan to support new CBW in HST deployments, otherwise we shouldn’t introduce requirements for additional CBW. |
| Huawei | Issue 1-1: UE capability for HST-SFN CA We are OK for both options. We slightly prefer Option 2 since that it is more reasonable to consider per FSPC UE capability granularity for baseband processing related UE features. Issue 1-2: Release independent Option 3a. Considering that agreement has been achieved in last meeting that new UE capability is defined per band combination, we can compromise to define release independent for HST SFN CA. For the release number, we prefer to consider Rel-16 to balance early-implementation effort from UE vendor and network performance from operation’s demand. Issue 1-3: FDD BW Option 2. Considering that 35/45MHz channel bandwidth has been introduced for FDD 15kHz and the related WI has been completed, it is benefit to define corresponding requirements for HST CA. |
| Intel | Issue 1-1: UE capability for HST-SFN CA It is not technically reasonable to consider per band combination granularity for baseband related capability. UE processing capability depends on the exact data-rate that depends on CBW and other demod related characteristics like modulation order and number of MIMO layers. All these parameters are signaled with FSPC granularity. Therefore, UE has different processing capability per each CC per each band per band combination. HST-SFN CA support for certain band combination with BC granularity will be based on the maximum aggregated CBW for this band combination. Same time, in case there are several CCs on each/one band, using of HST-SFN CA on all possible CC combinations will be precluded if max aggregated CBW is not supported by UE. FSPC does not have such constraints. In addition, based on our understanding FSPC granularity has smaller signaling overhead compared to BC. Support Option 2. Issue 1-2: Release independent We are fine to compromise to the recommended WF. Issue 1-3: FDD BW We do not see a harm to consider additional CBWs. |
| Qualcomm | Issue 1-1: UE capability for HST-SFN CA Ok with recommended WF. Issue 1-2: Release independent Ok with recommended WF. Issue 1-3: FDD BW We don’t have strong preference. |
| CMCC | Issue 1-1: Support option 1 and the recommended WF. For option 2, we are not sure whether we need to design this UE capability so flexible? According to the WID, HST under CA is the target scenario. It is expected that UE works with HST-SFN for all the component of a combination. Since UE is in the HST network, if it only supports HST-SFN for some component, but not supports HST-SFN for other components for a CA combination, we are wondering how to schedule this kind of UE. In our view, this kind of UE can indicate not support of HST-SFN for this certain CA combination. Issue 1-2: Firstly, we would like to clarify that our proposal in contribution is to have FR1 HST-SFN CA requirements release independent from Rel-15, not for HST-DPS which was already agreed in last meeting, the wording in open issue summary (section 1.2) is updated accordingly. For HST-SFN CA, our preference is to be release independent from Rel-15, and we do not see any issues to do so by adopting early implementation method. However, to move forward, we can compromise to Rel-16. Issue 1-3: Support option 2. From our point of view, we do not see technical issues to apply 35/45MHz for HST CA, it is preferred to specify PDSCH requirements for BW of {35, 45} MHz. |
| ZTE | Issue 1-1: We are fine with option 1 and the recommended WF. One of the motivation to introduce CA for HST is to increase the throughput, if the CC within a band is limited to e.g. a smaller BW, the throughput improvement will not be obvious for the band combination with the cost of more complex design. As pointed by CMCC, in this case the UE can report that certain BC is not supported. Issue 1-2: We can compromise to Rel-16 and we are fine with the WF. |

CRs/TPs comments collection

| CR Tdoc number | Title | Responsible company | Comment collection |
| --- | --- | --- | --- |
| R4-2203763 | Draft CR on HST DPS CA requirements for 4Rx | Apple | Apple: Based on section number of 2RX requirements, HST-DPS CA for 4RX can be 5.2A.3.5 Qualcomm: Based on endorsed draft CR R4-2202982 in the last meeting for HST-DPS 2Rx, section number should be 5.2A.3.4. There should be another table for HST DPS Scheme 1b tests. Currently, test 1,2,3 seem to point to Scheme 1a requirements. Also, in table 5.2A.3.X-7 test1 should refer to table X-3, test 2 should refer to X-5 and test 3 should refer to X-3 and X-5. Apple2: Thanks Qualcomm for the careful check. We will revise the CR accordingly. We need to align the section numbers. we believe the endorsed draft CRs in last meeting had requirements in section that already exists. This is our proposal based on single carrier requirements section numbering/order: 5.2A.2.4: HST-SFN CA 2RX 5.2A.2.5: HST-DPS CA 2RX 5.2A.3.4: HST-SFN CA 4RX 5.2A.3.5: HST-DPS CA 4RX |
| R4-2204253 | Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx | CMCC | Apple: Based on section number of 2RX requirements, HST-SFN CA for 4RX can be 5.2A.3.4 Intel: Please add test index as 1,2,3 Qualcomm: Based on endorsed draft CR R4-2202982 in the last meeting for HST-DPS 2Rx, section number should be 5.2A.3.5. |
| R4-2204385 | draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx | Intel | Apple: Section number should be 5.2A.2.4. Fraction of maximum throughput (%) is ‘a’ for 10MHz BW. Intel: Thanks Apple for careful check, we will update section number. By the way, we can ask MCC to update header of section 5.2A.2.3 because in TS 38.101-4 version 17.3 the header style is not correct. We will also correct test index from 1-1 to 1,2,3. Qualcomm: Based on endorsed draft CR R4-2202982 in the last meeting for HST-DPS 2Rx, section number should be 5.2A.2.5. |
| R4-2205082 | draft CR: FRC for CA PDSCH demodulation requirements for HST | Ericsson | Ericsson: This CR added CBW=35/45MHz to R4-2202983 endorsed in RAN4#101-bis-e. We can remove them according to the conclusion on Issue 1-3. |
| R4-2205753 | Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4) | Huawei,HiSilicon | Apple: Section number should be 5.2A.2.5. Qualcomm: Based on endorsed draft CR R4-2202982 in the last meeting for HST-DPS 2Rx, section number is ok. There should be another table for HST DPS Scheme 1b tests. Currently, test 1,2,3 seem to point to Scheme 1a requirements. Also, in table 5.2A.2.X-7 test1 should refer to table X-3, test 2 should refer to X-5 and test 3 should refer to X-3 and X-5. |
| R4-2206110 | Draft CR on Applicability Rules for FR1 HST CA requirements | Qualcomm Incorporated |  |

Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Issue 1-1: UE capability for HST-SFN CA

  - Option 1 (CMCC, Ericsson, ZTE, Huawei, Qualcomm): Per band combination

  - Option 2 (Intel, Huawei, Apple): per FSPC

GTW Agreement:

  - Per band combination granularity (option 1).

Huawei/Intel: we have concern on above agreement.

Issue 1-2: Release independents

  - HST-SFN CA requirements are release independent from Rel-15 (CMCC)

  - For Rel-17 FR1 HST PDSCH SFN CA demodulation requirements

      - Option 1: Applicable from Rel-17

      - Option 2: Aligned with RRM agreement (Apple)

      - Option 3: Release independent from Rel-15 /16

        - Option 3a: Release independent from Rel-16 (Ericsson, Huawei, Intel, Qualcomm, CMCC, ZTE)

GTW Agreement:

  - From demodulation perspective, release independent from Rel-16.

Issue 1-3: FDD BW

  - Option 1 (Apple, Qualcomm): Do not introduce FDD requirements with additional CBW for HST CA unless there is need to support these CBW in HST deployments.

  - Option 2 (CMCC, Huawei, Qualcomm, Intel, Ericsson): For FDD 15KHz SCS, specify PDSCH requirements on single carrier of BW of {35, 45} MHz for HST scenario.

GTW Agreement: Option 2

  - Discussion on 2nd round (if applicable)

    - Open issues summary

No open issues to be discussed in 2nd round. Capture the above agreements in a WF.

Open issues

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

Summary on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FR1 HST demodulation | CMCC |  |
| LS on release independent of FR1 HST demodulation | CMCC |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2203763 | Draft CR on HST DPS CA requirements for 4Rx | Apple | Revised |  |
| R4-2204253 | Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx | CMCC | Revised |  |
| R4-2204385 | draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx | Intel | Revised |  |
| R4-2205753 | Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4) | Huawei,HiSilicon | Revised |  |
| R4-2205082 | draft CR: FRC for CA PDSCH demodulation requirements for HST | Ericsson | Revised |  |
| R4-2206110 | Draft CR on Applicability Rules for FR1 HST CA requirements | Qualcomm Incorporated | Revised |  |

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2203762 | Discussion on PDSCH CA Requirements in HST | Apple | Noted |  |
| R4-2204259 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Noted |  |
| R4-2204384 | Discussion on HST FR1 CA PDSCH performance requirements | Intel Corporation | Noted |  |
| R4-2204430 | Discussion on PDSCH requirements for FR1 HST CA scenarios | ZTE Corporation | Noted |  |
| R4-2205081 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Noted |  |
| R4-2205752 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei,HiSilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-2207194 | WF on FR1 HST demodulation | CMCC | Agreeable |
| R4-2207195 | LS on release independent of FR1 HST demodulation | CMCC | Agreeable |

| Tdoc number | Title | Source | Decision | Comments |
| --- | --- | --- | --- | --- |
| R4-2207190 (revision of R4-2203763) | Draft CR on HST DPS CA requirements for 4Rx | Apple | Agreeable |  |
| R4-2207191 (revision of R4-2204253) | Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx | CMCC | Agreeable |  |
| R4-2207192 (revision of R4-2204385) | draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx | Intel | Agreeable |  |
| R4-2207193 (revision of R4-2205753) | Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4) | Huawei,HiSilicon | Agreeable |  |
| R4-2207233 (revision of R4-2205082) | draft CR: FRC for CA PDSCH demodulation requirements for HST | Ericsson | Agreeable |  |
| R4-2207234 (revision of R4-2206110) | Draft CR on Applicability Rules for FR1 HST CA requirements | Qualcomm Incorporated | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |
| Intel | Artyom Putilin | artyom.putilin@intel.com |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)