3GPP TSG-RAN WG4 Meeting #102-e	R4-2203509

Online Meeting, 21 February – 03 March 2022

3GPP TSG-RAN WG2 Meeting #116bis-e	Tdoc R2- 2201759

Electronic Meeting, Jan 17th - 25th, 2022

Title:	Reply LS on the use of NCD-SSB or CSI-RS in DL BWPs for RedCap UEs

Response to:	LS on the use of NCD-SSB or CSI-RS in DL BWPs for RedCap UEs (R1-2112802)

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	RAN2

To:	RAN1

Cc:	RAN4

Contact Person:

#### Name:	Emre A. Yavuz

E-mail Address:	emre dot yavuz at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 would like to thank RAN1 for the LS on the use of NCD-SSB or CSI-RS in DL BWPs for RedCap UEs and provide the following feedback on whether the highlighted working assumptions in the LS from RAN1 are acceptable from RAN2 perspective.

Regarding the following working assumption for FR1 and FR2 related to a separate initial DL BWP: “If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective”

- RAN2 agreed that “A RedCap UE in idle/inactive mode monitors paging only in an initial BWP (default or RedCap specific) associated with CD-SSB and performs cell (re-)selection and measurements on the CD-SSB.” Therefore, using an NCD-SSB for such purposes is not considered further.

Regarding the following working assumption for FR1 and FR2 related to an RRC-configured active DL BWP in connected mode: “A RedCap UE can in addition optionally support relevant operation based on CSI-RS”

- The use of CSI-RS for cell/beam RLM and measurements is supported from RAN2 signalling standpoint as indicated earlier. RAN4 has informed RAN2 and RAN1 that CSI-RS cannot be used as a standalone mechanism for RRM measurements and existing requirements rely on the presence of SSB signals, in their reply LS provided in R4-2120327. RAN2 does not intend to introduce a new mechanism that would enable a RedCap UE to perform CSI-RS based RRM measurements and think that it is up to RAN4 to decide whether RAN1 working assumption regarding the use of CSI-RS in connected mode is acceptable based on the information provided above.

2. Actions:

To RAN1

ACTION: 	RAN2 kindly asks RAN1 to take the information above into consideration.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #117-e                       21st Feb - 3rd Mar 2022		e-Meeting

TSG-RAN WG2 Meeting #118-e                  	16th - 27th May 2022 		e-Meeting