3GPP TSG-RAN WG4 Meeting #103-e	R4-2208326

Electronic Meeting, 9th – 20th May 2022

Agenda Item:	9.15.10.2.2

Source:	Ericsson

Title:	On the PDCCH and PBCH requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on PDCCH and PBCH demodulation requirements for FR2-2. Furthermore, in RAN4 meeting #102-e, the channel model and the corresponding delay spread have been agreed in [1], such as

The channel models have been decided to be TDLA and TDLD, where both using the following delay spread RMS: - SCS = 120 KHz, 10 ns. - SCS = 480 KHz, 10 ns / 5 ns. - SCS = 960 KHz, 10 ns / 5 ns.

However, several needed related parameters have not been agreed yet, such as

Issue 1-3-3: Max Doppler frequency Consider 3 km/h UE speed (200 Hz). FFS on higher UE speed. - Option 1: 10 km/h (650 Hz) - Option 2: 30 km/h (2000 Hz) Issue 3-1-1: SCS for DL requirements definition Consider the following SCS for DL requirements definition: - 120, 480 kHz - FFS 960 kHz Issue 3-1-2: CBW for DL requirements definition 120 kHz: - 100, 400 MHz 480 MHz: - 400 MHz - FFS on 1600 MHz 960 kHz: - 400 MHz - FFS on 2000 MHz

We follow-up on these issues in a separate paper on PDSCH requirements [2].

# 2	Remaining issues for PDCCH requirements

Several issues need to be discussed in this meeting. Then, we will provide our proposals to address them.

Issue 3-4-1: Performance requirements for multi-slot PDCCH monitoring Option 1: Define performance requirements for multi-slot PDCCH monitoring for 480 and 960 kHz SCS Option 2: Do not define performance requirements for multi-slot PDCCH monitoring for 480 and 960 kHz SCS Issue 3-4-2: PDCCH simulation assumptions Use following assumptions for PDCCH performance test. - Antenna configuration: 1T2R and 1T4R - Aggregation level: 2 and 4 for 1T2R; 8 and 16 for 2T2R

We propose to define PDCCH with all the possible aggregation levels 2, 4, 8, and 16 by reusing the existing test case specified in TS38.101-4 Section 7.3.2.2 for SCS 120 KHz in FR2-2 (see Appendix A1).

Observation 1: TDD UL-DL pattern should be defined for tests.

Consequently, we propose to define PDCCH requirements in FR2-2 for both SCS 120 KHz and SCS 480 KHz.

Observation 2: While RAN1 defined multi-slot PDCCH monitoring for high SCS, 480 KHz and 960 KHz, we do not believe that it is necessary from a demodulation point of view, since each Slot will be decoded independently.

Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |

FR2-2 TDD, SCS 120 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 1 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 240 | 1 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 240 | 1 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 240 | 1 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 2 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 240 | 2 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 240 | 2 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 240 | 2 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |

FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 2 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 60 | 2 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 60 | 2 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |

FR2-2 TDD, SCS 480 KHz/CBW 1600 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 1 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 240 | 1 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 240 | 1 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 240 | 1 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |

# 3	Remaining issues for PBCH requirements

It has been stated in TR 38.808 that it is recommended to strive for maximum commonality for the system design for licensed and unlicensed operation for FR2-2 band and maximize the re-use of the existing NR design.

PBCH simulation assumptions (if introduced pending on outcome of issues Issue 1-2-2) SSB Index: - Option 1: Only with not known SSB index - Option 2: Both known and not known SSB index - Option 3: Only with known SSB index - Option 4: Not known SSB index for 120 kHz and known SSB index for 480/960 kHz.

RAN4 defined the PBCH demodulation requirements (i.e., MIB decoding performance) in Rel-15 and we propose to extend the configurations in FR2-2. Since we have new propagation environments, we should then define new requirements for PBCH considering whether the SS/PBCH block index is known or unknown.

Proposal 2: Define PBCH demodulation requirements with the following test setup

FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz / 100 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |

FR2-2 TDD, SCS 120 KHz/CBW 400 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz / 400 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |

FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz / 400 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |

FR2-2 TDD, SCS 480 KHz/CBW 1600 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz / 1600 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |

# 5	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.3 | PDCCH demodulation requirements | Updates in Table 7.3-1 |
| 7.3.1 | 1RX requirements | Void |
| 7.3.2 | 2RX requirements |  |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.1 | 1 Tx Antenna performances | Updates on FR2-2 |
| 7.3.2.2.2 | 2 Tx Antenna performances | Updates on FR2-2 |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.1 | 1RX requirements |  |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Updates on FR2-2 UEs |

# 6	Summary

In this paper, we tried to contribute with the following proposals for further discussions:

Observation 1: TDD UL-DL pattern should be defined for tests.

Observation 2: While RAN1 defined multi-slot PDCCH monitoring for high SCS, 480 KHz and 960 KHz, we do not believe that it is necessary from a demodulation point of view, since each Slot will be decoded independently.

Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |

FR2-2 TDD, SCS 120 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 1 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 240 | 1 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 240 | 1 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 240 | 1 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 2 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 240 | 2 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 240 | 2 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 240 | 2 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 2x2 ULA Low | New |

FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 2 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 60 | 2 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 60 | 2 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |

FR2-2 TDD, SCS 480 KHz/CBW 1600 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 1 | 2 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-2 | 2 | 6 | 240 | 1 | 4 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 1x2 ULA Low | New |
| Test 1-3 | 3 | 2 | 240 | 1 | 8 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |
| Test 1-4 | 3 | 2 | 240 | 1 | 16 | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 TDLA5-200 TDLD5-200 | 2x2 ULA Low | New |

Proposal 2: Define PBCH demodulation requirements with the following test setup

FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz / 100 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |

FR2-2 TDD, SCS 120 KHz/CBW 400 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz / 400 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |

FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz / 400 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |

FR2-2 TDD, SCS 480 KHz/CBW 1600 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz / 1600 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLD10-200 | New |  |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLD10-650 | New |  |

# References

- R4-2207223, “WF on demodulation performance requirements definition for 52.6 - 71 GHz”, Intel.

- R4-2208324, “The remaining issues of the PDSCH requirements in 52.6 – 71 GHz band”, Ericsson.

# Appendix	Simulation assumption

## A.1	PBCH

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | TBD |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

## A.2	PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern |  | TBD |
| Note 1: as specified in clause 4.1 of TS 38.213 Note 2: as specified in clause 11.1 of TS 38.213 |  |  |

See 38.101-4 Table 7.4.2.2-1