3GPP TSG-RAN WG4 #103-e	R4-2210166

e-Meeting, May 9 - 20, 2022

Agenda item:	9.15.3.1

Source: 	Qualcomm Incorporated

Title: 	60GHz UE TX

Document for:	Approval

# Introduction

In this paper we analyze and propose various requirements for the UE in n263

# Transmission bandwidth configuration

IN the last RAN4 meeting the following table was agreed as a working assumption in RAN4.

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33 | 66 | 132 | [156] |

Proposal: Use the NRB table values for FR2-2 transmission bandwidth requirements, with [] removed from 156.

# Transmit OFF power

Proposal: Use the table in 38.101-2 to specify the FR2-2 TX OFF power. Note the value is -35 dBm in the measurement bandwidth.

Table 6.3.2-1: Transmit OFF power

| Operating band | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257, n258, n259, n260, n261, n262 | -35 | -35 | -35 | -35 | N/A | N/A | N/A |
|  | 47.58MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz | N/A | N/A | N/A |
| n263 | N/A | -35 | N/A | -35 | -35 | -35 | -35 |
|  | N/A | 95.16 MHz | N/A | 380.28 MHz | 760.32 MHz | 1520.64 MHz | 1797.12 MHz |

# PRACH Time Mask

RAN1 has agreed that PRACH may use 120, 480, or 960 kHz SCS. We can specify the measurement time period in the same table for FR2-1 and -2.

Proposal: PRACH ON power measurement period table should be updated for 480 and 960 SCS as shown.

Table 6.3.3.4-1: PRACH ON power measurement period

| Format | SCS | Measurement period | Note |
| --- | --- | --- | --- |
| A1 | 60 kHz | 0.035677 ms |  |
|  | 120 kHz | 0.017839 ms |  |
|  | 480 kHz | 0.004460 ms |  |
|  | 960 kHz | 0.002230 ms |  |
| A2 | 60 kHz | 0.071354 ms |  |
|  | 120 kHz | 0.035677 ms |  |
|  | 480 kHz | 0.008919 ms |  |
|  | 960 kHz | 0.004460 ms |  |
| A3 | 60 kHz | 0.107031 ms |  |
|  | 120 kHz | 0.053516 ms |  |
|  | 480 kHz | 0.013379 ms |  |
|  | 960 kHz | 0.006690 ms |  |
| B1 | 60 kHz | 0.035091 ms |  |
|  | 120 kHz | 0.0175455 ms |  |
|  | 480 kHz | 0.004386 ms |  |
|  | 960 kHz | 0.002193 ms |  |
| B4 | 60 kHz | 0.207617 ms |  |
|  | 120 kHz | 0.103809 ms |  |
|  | 480 kHz | 0.025952 ms |  |
|  | 960 kHz | 0.012976 ms |  |
| A1/B1 | 60 kHz | 0.035677 ms for front X1 occasion 0.035091 ms for last occasion | X1 = [2,5] |
|  | 120 kHz | 0.017839 ms for front X1occasion 0.017546 ms for last occasion | X1 = [2,5] |
|  | 480 kHz | 0.004460 ms for front X1 occasion 0.004387 ms for last occasion | X1 = [2,5] |
|  | 960 kHz | 0.017839 ms for front X1occasion 0.017546 ms for last occasion | X1 = [2,5] |
| A2/B2 | 60 kHz | 0.071354 ms for front X2 occasion 0.069596 ms for last occasion | X2 = [1,2] |
|  | 120 kHz | 0.035677 ms for front X2 occasion 0.034798 ms for last occasion | X2 = [1,2] |
|  | 480 kHz | 0.008919 ms for front X2 occasion 0.008700 ms for last occasion | X2 = [1,2] |
|  | 960 kHz | 0.004460 ms for front X2 occasion 0.004350 ms for last occasion | X2 = [1,2] |
| A3/B3 | 60 kHz | 0.107031 ms for first occasion 0.104101 ms for second occasion |  |
|  | 120 kHz | 0.053515 ms for first occasion 0.052050 ms for second occasion |  |
|  | 480 kHz | 0.013379 ms for first occasion 0.013013 ms for second occasion |  |
|  | 960 kHz | 0.006689 ms for first occasion 0.006506 ms for second occasion |  |
| C0 | 60 kHz | 0.026758 ms |  |
|  | 120 kHz | 0.013379 ms |  |
|  | 480 kHz | 0.003345 ms |  |
|  | 960 kHz | 0.001672 ms |  |
| C2 | 60 kHz | 0.083333 ms |  |
|  | 120 kHz | 0.0416667 ms |  |
|  | 480 kHz | 0.010417 ms |  |
|  | 960 kHz | 0.005208 ms |  |
| NOTE: For PRACH on PRACH occasion start from begin of 0ms or 0.5 ms boundary, the measurement period will be increased by 0.032552 μs |  |  |  |  |

# Absolute, Relative, and Aggregate Power Tolerance

Power tolerance is a function of the complexity and variance in the UE transmitter chain. For FR2-2 it may be necessary to use one more upconversion in the TX chain, as compared to FR2-1. This can lead to more variability across power settings. To specify FR2-2 power tolerance one approach is to use the requirements for FR2-1 as a working assumption, and allow for more exceptions due to increased TX path variability.

The FR2-1 sections are shown below for convenience.

The minimum requirements specified in Table 6.3.4.2-1 apply in the power range bounded by the minimum output power as specified in sub-clause 6.3.1 ('Pmin') and the maximum output power as specified in sub-clause 6.2.1 as minimum peak EIRP ('Pmax').

Table 6.3.4.2-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

Table 6.3.4.2-2: Intermediate power point

| Power Parameter | Value |
| --- | --- |
| Pint | Pmax – 12.0 dB |

Proposal: Use the same FR2-1 absolute power tolerance and Pint for FR2-2 as a working assumption, with possibly more exceptions.

Relative power tolerance

Are we ok with the FR2-1 formulation?

Table 6.3.4.3-1: Relative power tolerance, Pint ≥ P ≥ Pmin

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | ±5.0 |
| 2 ≤ ΔP < 3 | ±6.0 |
| 3 ≤ ΔP < 4 | ±7.0 |
| 4 ≤ ΔP < 10 | ±8.0 |
| 10 ≤ ΔP < 15 | ±10.0 |
| 15 ≤ ΔP | ±11.0 |
| NOTE: The requirements apply with ue-BeamLockFunction enabled. |  |

Table 6.3.4.3-2: Relative power tolerance, PUMAX ≥ P > Pint

| Power step ∆P (Up or down) (dB) | All combinations of PUSCH and PUCCH, PUSCH/PUCCH and SRS transitions between sub-frames, PRACH (dB) |
| --- | --- |
| ΔP < 2 | ± 3.0 |
| 2 ≤ ΔP < 3 | ± 4.0 |
| 3 ≤ ΔP < 4 | ± 5.0 |
| 4 ≤ ΔP < 10 | ± 6.0 |
| 10 ≤ ΔP < 15 | ± 8.0 |
| 15 ≤ ΔP | ± 9.0 |
| NOTE 1: The requirements apply with ue-BeamLockFunction enabled. NOTE 2: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, guard periods: for a power step ΔP = 1 dB, the relative power tolerance for transmission is ± 1.0 dB. |  |

Proposal: Use the same FR2-1 relative power tolerance and Pint for FR2-2 as a working assumption, with possibly more exceptions.

Aggregate power tolerance

Table 6.3.4.4-1: Aggregate power tolerance, Pint ≥ P ≥ Pmin

| TPC command | UL channel | Aggregate power tolerance within 21 ms |
| --- | --- | --- |
| 0 dB | PUCCH | ± 5.5 dB |
| 0 dB | PUSCH | ± 5.5 dB |

Table 6.3.4.4-2: Aggregate power tolerance, Pmax ≥ P > Pint

| TPC command | UL channel | Aggregate power tolerance within 21 ms |
| --- | --- | --- |
| 0 dB | PUCCH | ± 3.5 dB |
| 0 dB | PUSCH | ± 3.5 dB |

Proposal: Use the same FR2-1 aggregate power tolerance and Pint for FR2-2 as a working assumption, with possibly more exceptions.

# EVM

EVM percentages

It is reasonable to adopt the same EVM %age levels as FR2-1. Those values are shown here for convenience.

Table 6.4.2.1-1: Minimum requirements for error vector magnitude

| Parameter | Unit | Average EVM level | Reference signal EVM level |
| --- | --- | --- | --- |
| Pi/2 BPSK | % | 30.0 | 30.0 |
| QPSK | % | 17.5 | 17.5 |
| 16 QAM | % | 12.5 | 12.5 |
| 64 QAM | % | 8.0 | 8.0 |

Proposal: Use the FR2-1 EVM percentages for n263.

EVM Applicable power ranges

In FR2-2 the EVM range will be reduced as compared to FR2-1. For bandwidths > 400 MHz the noise power will increase and become a more significant factor in the EVM. To reflect this EVM degradation effect the minimum EVM power level should be dB-scaled from 400 MHz.

Proposal: Scale the minimum EVM power level from 400 MHz for noise BW. 3 dB for 800 MHz, 6 dB for 1600 MHz, 7 dB for 2000 MHz.

PTRS processing for EVM

RAN4 agreed that PTRS processing should be part of the EVM requirement for modulation order higher than 16QAM, and possibly for 16QAM and lower modulation order. Based on that:

Proposal: RAN4 to further discuss how to capture PTRS processing in EVM requirement.

# TRP

# Carrier Leakage

Power class 1

Table 6.4.2.2.2-1: Minimum requirements for relative carrier leakage power for power class 1

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 17 dBm | -25 |
| 4 dBm ≤ EIRP ≤ 17 dBm | -20 |

Proposal: Use the values in the table above for n263 PC1 relative carrier leakage requirements.

Power class 2

Table 6.4.2.2.3-1: Minimum requirements for relative carrier leakage power for power class 2

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 6 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 6 dBm | -20 |

Proposal: Use the values in the table above for n263 PC2 relative carrier leakage requirements.

Power class 3

Table 6.4.2.2.4-1: Minimum requirements for relative carrier leakage power for power class 3

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 0 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 0 dBm | -20 |

Proposal: Use the values in the table above for n263 PC1 relative carrier leakage requirements.

# Inband emissions

For inband emissions we propose to start with the FR2-1 requirements for PC1-3 and modify them for use in n263. Specifically:

Inband emissions proposals:

- Use the same general in-band emissions limit as FR2-1

- Modify the carrier leakage power limits, using the limits shown in the previous section tables for PC1-3

- Modify the IQ image values, adding 10 dB to the carrier leakage power limits. This is the same method used in FR2-1.

# Spectrum emission mask

In a previous RAN4 meeting the agreement was to use the FR2-1 spec table method and extend it to FR2-2. The table below does that.

Table 6.5.2.1-1: General NR spectrum emission mask for frequency range 2.

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | -5 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | -5 | -5 | -5 | 1 MHz |
| 40-80 | -13 | -13 | -13 | -13 | -5 | -5 | -5 | 1 MHz |
| 80-100 | -13 | -13 | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| 100-160 |  | -13 | -13 | -13 | -13 | -5 | -5 | 1 MHz |
| 160-200 |  |  | -13 | -13 | -13 | -13 | -5 | 1 MHz |
| 200-400 |  |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | -13 | -13 | -13 | 1 MHz |
| 800-1600 |  |  |  |  | -13 | -13 | -13 | 1 MHz |
| 1600-3200 |  |  |  |  |  | -13 | -13 | 1 MHz |
| 3200-4000 |  |  |  |  |  |  | -13 | 1 MHz |
| NOTE 1: Void |  |  |  |  |  |  |  |  |

Proposal: Use the modified SEM table for the FR2-2 general SEM requirement.

# Spurious Emissions, Coexistence, and Additional Spurious

Spurious domain boundary.

Proposal: For n263 use 2x the CCBW as the spurious domain, as in FR2-1.

Table 6.5.3-1: Boundary between NR out of band and spurious emission domain

| Channel bandwidth | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 100 | 200 | 400 | 800 | 1600 | 3200 | 4000 |

Spurious emission limits for n263:

Proposal: For n263 spurious emissions use the same limits as FR2-1

Table 6.5.3-2: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 1 MHz |

# Beam Correspondence

UEs without beam correspondence were conceived of early in the development of mmWave UE technology. As the technology has been deployed the operation of the UE antenna arrays is well understood. We have found that invariably the UEs can achieve beam correspondence. The advantage of beam correspondence UEs is elimination of UL beam sweeping. RAN4 should consider declaring beam correspondence for all n263.

Proposal: All n263 UEs shall support beamCorrespondenceWithoutUL-BeamSweeping.

# MPR

## Single carrier CP-OFDM trends

We have performed some preliminary simulation studies of backoff for CP-OFDM with EVM and SEM. We wanted to look for trends in the backoff over modulation order, CCBW and SCS. For these studies we used 66 RBs in 100 MHz with QPSK modulation and 120 kHz SCS. The table shows the maximum backoff for all possible RB configuration.

It is still under discussion whether this will be the reference waveform for the requirement, however this reference serves the purpose of comparison.

The EVM limit was assumed to the FR2-1 EVM limit, and the SEM was assumed to be the SEM in this paper.

Note some select plot of simulation results are included in the Annex.

|  | 100M |  | 400 M |  |  | 800 M |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 120k QPSK | 120k 64QAM | 960k QPSK | 960k 64QAM | 480k 64QAM | 960k QPSK | 960k 64QAM |
| PC1 SEM | 0 | 3.2 | 0 | 0 | 0 | 0 | 0 |
|  | PC1 SEM Observation: Only 100 MHz is SEM-limited, with some backoff required for higher modulation orders. |  |  |  |  |  |  |
| PC1 EVM | 0 | 3.5 | 0 | 3.5 | 3.5 | 0 | 3.5 |
|  | PC1 EVM Observation: Backoff for EVM is modulation order dependent, with no sensitivity to SCS or CCBW. |  |  |  |  |  |  |
| PC3 SEM | 0 | 0.8 | 0 | 0 | 0 | 0 | 0 |
|  | PC3 SEM Observation: Only 100 MHz is SEM-limited, with some backoff required for higher modulation orders. This is similar to PC1 SEM. |  |  |  |  |  |  |
| PC3 EVM | 0 | 12 | 2.2 | 12 | 12 | 2.2 | 12 |
|  | PC3 EVM Observation: Backoff for EVM is primarily modulation order dependent. There is also some dependence on CCBW as 400 and 800M QPSK require more backoff than 100M. We can’t determine yet if there is a dependence on SCS for QPSK. |  |  |  |  |  |  |

# ON/ON transient periods

From the standpoint of ON/ON transient period, the FR2-2 UE is the same as FR2-1. In one sense FR2-2 is quite like FR2-1, with FR2-2 much like a new FR2-1 band with some additional SCS. FR2-1 UE designs are commercial and mature. The ON/ON transient time of 5usec is governed by the time required to configure the transmitter and receiver. Those constraints are the same for FR2-2. Further in future as devices support FR2-1 and FR2-2 much hardware will be common. It is natural to specify the transient period the same as FR2-1.

Proposal: The transient period from FR2-1 is based on the capability of the UE to configure the transmitter and receiver. The same capability will exist in FR2-2. Use the same 5usec for FR2-2.

# Beam direction only switching time baseline assumption

One valid approach to the 60 GHz UE design is to base it on the existing FR2 design. In this aspect, 60 GHz is like a new FR2 band. This means the configuration times for beam switching are the same for 60 GHz and FR2.The FR2 switching assumptions are based on the capabilities of the UE to switch, and what is feasible. 60 GHz UE feasibility is the same as FR2. Further the configuration times are similar for any SCS, and this should be part of the assumption.

The FR2 assumption for UE beam switching is 200 nsec, 60 GHz requires the same amount of time to switch. We find no difference between inter-panel and intra-panel switching. Both require the same amount of time to configure the hardware for switching.

On clarification, our understanding is the UE can perform a beam direction change, or a power control change, or both during this 200 nsec time.

Proposal: UE requires 200 nsec for beam direction only switching for all SCS

Observation: Our understanding is the UE can perform a beam direction change, or a power control change, or both during this 200 nsec time.

# Conclusions

Proposal: Use the NRB table values for FR2-2 transmission bandwidth requirements, with [] removed from 156.

Proposal: Use the table in 38.101-2 to specify the FR2-2 TX OFF power. Note the value is -35 dBm in the measurement bandwidth.

Proposal: PRACH ON power measurement period table should be updated for 480 and 960 SCS as shown.

Proposal: Use the same FR2-1 absolute power tolerance and Pint for FR2-2 as a working assumption, with possibly more exceptions.

Proposal: Use the same FR2-1 relative power tolerance and Pint for FR2-2 as a working assumption, with possibly more exceptions.

Proposal: Use the same FR2-1 aggregate power tolerance and Pint for FR2-2 as a working assumption, with possibly more exceptions.

Proposal: Use the FR2-1 EVM percentages for n263.

Proposal: Use the values in the table above for n263 PC1 relative carrier leakage requirements.

Proposal: Use the values in the table above for n263 PC2 relative carrier leakage requirements.

Proposal: Use the values in the table above for n263 PC1 relative carrier leakage requirements.

Inband emissions proposals:

- Use the same general in-band emissions limit as FR2-1

- Modify the carrier leakage power limits, using the limits shown in the previous section tables for PC1-3

- Modify the IQ image values, adding 10 dB to the carrier leakage power limits. This is the same method used in FR2-1.

Proposal: Use the modified SEM table for the FR2-2 general SEM requirement.

Proposal: For n263 use 2x the CCBW as the spurious domain, as in FR2-1.

Proposal: For n263 spurious emissions use the same limits as FR2-1

Proposal: All n263 UEs shall support beamCorrespondenceWithoutUL-BeamSweeping.

PC1 SEM Observation: Only 100 MHz is SEM-limited, with some backoff required for higher modulation orders.

PC1 EVM Observation: Backoff for EVM is modulation order dependent, with no sensitivity to SCS or CCBW.

PC3 SEM Observation: Only 100 MHz is SEM-limited, with some backoff required for higher modulation orders. This is similar to PC1 SEM.

PC3 EVM Observation: Backoff for EVM is primarily modulation order dependent. There is also some dependence on CCBW as 400 and 800M QPSK require more backoff than 100M. We can’t determine yet if there is a dependence on SCS for QPSK.

Proposal: The transient period from FR2-1 is based on the capability of the UE to configure the transmitter and receiver. The same capability will exist in FR2-2. Use the same 5usec for FR2-2.

Proposal: UE requires 200 nsec for beam direction only switching for all SCS

Observation: Our understanding is the UE can perform a beam direction change, or a power control change, or both during this 200 nsec time.

# Annex

Select backoff plots