3GPP TGS-RAN WG4 Meeting #103-e	     R4-2210212
Electronic Meeting, May 9 – 20, 2022

Agenda item:	11.1.2.1

Source:	Intel Corporation

Title:	General aspects of test methods for 52.6~71GHz

Document for:	Discussion

# 1	Introduction

During the RAN #95e plenary meeting, the Study on enhanced test methods for FR2 was declared 90% completed and its target completion date was updated to June 2022 [1]. This paper discusses some open items to finalize the seventh objective of this study (defining test methods for FR2-2).

# 2	Discussion

General aspects – radiating aperture

Agreement:

- Option 1: reusing D = 5cm for PC3

- Additional information can be included into TR for the background if needed

In RAN4 #102e, it was agreed to reuse the current radiating aperture D = 5cm for PC3 in FR2-2 [2]. Given this, we propose adding a column to the minimum range length of DFF table to include 71 GHz.

Proposal 1: Add a column for 71 GHz to the minimum range length of DFF table and include in the TR.

Table 5.2.1.2-1: Minimum Range Length of DFF System for D = 5cm [3]

| f [GHz] QZ [cm] | 24.25 | 30 | 40 | 50 | 52.6 | 71 |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.45 | 0.55 | 0.72 | 0.88 | 0.93 | 1.23 |
| 30 | 0.53 | 0.63 | 0.79 | 0.96 | 1.00 | 1.31 |

UE types

- Agreement: MU assessment for FR2-2 will focus on PC3 in Rel-17 timeline.  This, however, does not deprioritize the general work on other UE types in the WI (i.e., FWA and vehicular).

To conclude the FR2-2 test methods objective, a preliminary MU assessment is needed [2]. Per the agreement reached in RAN4 #101e [4], this assessment will focus on PC3 during Rel-17. Nonetheless, progress was also made in discussions of other power classes. Therefore, FWA (PC1) and vehicular UE (PC2) agreements [1] should also be documented. Additionally, RAN4 should discuss and confirm whether the permitted test methods can apply to PC1 and PC2 and consider communicating this outcome to RAN5.

Observation 1: While MU assessment will focus on PC3 in Rel-17, several agreements for PC1 and PC2 were reached. RAN4 should discuss whether the permitted test methods can apply to PC1 and PC2 and consider communicating the outcome to RAN5.

# 3	Conclusions

In this contribution we discussed potential updates for the extension of permitted UE RF test methods to FR2-2. The following proposals and observations were made:

Proposal 1: Add a column for 71 GHz to the minimum range length of DFF table and include in the TR.

Table 5.2.1.2-1: Minimum Range Length of DFF System for D = 5cm [3]

| f [GHz] QZ [cm] | 24.25 | 30 | 40 | 50 | 52.6 | 71 |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.45 | 0.55 | 0.72 | 0.88 | 0.93 | 1.23 |
| 30 | 0.53 | 0.63 | 0.79 | 0.96 | 1.00 | 1.31 |

Observation 1: While MU assessment will focus on PC3 in Rel-17, several agreements for PC1 and PC2 were reached. RAN4 should discuss whether the permitted test methods can apply to PC1 and PC2 and consider communicating the outcome to RAN5.

# 4	References

- RP-220924, “Status report for SI Study on enhanced test methods for FR2 NR UEs,” RAN #95e, Apple, March 2022

- R4-2207201, “WF on FR2-2 OTA test methods,” Intel, RAN4 #102e, March 2022

- 3GPP TR38.810, “Study on test methods,” version 16.6.1, November 2020

- 3GPP TR38.884, “Study on enhanced test methods for FR2,” version 18.0.0, April 2022

- R4-2120767, “WF on OTA test methods for 52.6~71GHz,” Intel, RAN4 #101e, November 2021