3GPP TSG-RAN WG4 Meeting #103-e	R4-2210633

# Electronic meeting, January 9th – 20th, 2022

Source:			CATT

Title:			WF on remaining issue for SAN RF requirements

Agenda item:		9.12.3

Document for:		Approval

SAN RF requirements

ΔfOBUE

The boundary between the out-of-band mask and spurious domain for SAN should be specified as 2*BWChannel from the channel edge based on ITU regulation.

ΔfOOB

Use ΔfOOB=20MHz as baseline for n256 and n255

Spurious emission

Agree the following spurious emission

| Spurious frequency range | Basic limit GEO/LEO class (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz –5th harmonic of the upper frequency edge of the DL operating band (NOTE 3, 4, 5, 6) | When Prated,c,sys ≤ 47dBm : -13 When Prated,c,sys > 47dBm : Prated,c,sys-60 (NOTE 1) | 4 kHz (NOTE 2) |

RAN4 can further revisit the low frequency boundary (30MHz) in maintenance phase if technical justification identified.

Principles to define OBUE requirements for SAN

Follow Annex 5 of ITU recommendation SM.1541-6 to define SAN OBUE requirements.

OBUE requirement

OBUE requirement is agreed to be specified as the following,

| 0 MHz f < 2* BWChannel | 0.002 MHz f_offset < 2* BWChannel + 0.002 MHz | maxSE limit, PSDchannel-[X] – 40*log10 f_offset-0.002BWChannel*2+1dBm | 4kHz |
| --- | --- | --- | --- |

- X = 3dB for LEO

- X= 0dB for GEO

RAN4 can further revisit the requirements including the value(X) during maintenance phase with technical input from companies.

Absolut ACLR requirement

Do not specify absolute ACLR requirement.

References

- R4-2208883, NTN - SAN TX radiated requirements: remaining issues, Ericsson

- R4-2209923, On SAN Spurious Emission requirements for Radiated  Characteristics, Nokia, Nokia Shanghai Bell

- R4-2208882, NTN - SAN TX conducted requirements: remaining issues, Ericsson

- R4-2209361, Discussion on SAN OoB mask and spurious emission requirements, Huawei, HiSilicon, Thales

- R4-2209592, Further discussion on conducted Tx requirements of satellite access node, ZTE

- R4-2209924, On SAN Spurious Emission requirements for Conducted Characteristics, Nokia, Nokia Shanghai Bell

- R4-2210082, Discussion on SAN OBUE, Thales

- R4-2210090, Discussion on SAN Spurious Emissions, Thales

- R4-2210110, On SAN OBUE definition using ITU-R recommendation, Thales