3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211521

# Electronic Meeting, August 15 – August 26, 2022

3GPP TSG-RAN WG2 Meeting #118-e		R2-2206418

Electronic, 9th May – 20th May 2022

Title:	Reply LS to RAN4 on RRM relaxation

Response to:	R2-2204487_R4-2207109

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	RAN2

To:	RAN4

Cc:

# Contact Person:

# Name:	                Chenli

E-mail Address:	 Chenli5g@vivo.com

# 1. Overall Description:

RAN2 would like to thank RAN4 for the LS on RRM relaxation. Based on further RAN4 progress, RAN2 discussed the coexistence of Rel-16 and Rel-17 RRM relaxation, and achieved the following agreements:

Regarding the coexistence cases of Rel-17 and Rel-17 RRM relaxation:

- Simultaneous configuration of R16 not-at-cell-edge criterion and R17 stationary criterion for idle/inactive mode is a valid configuration from the network’s point of view, where the network supports RRM relaxation for both R16 and R17 UEs in idle/inactive mode.

- From signalling’s point of view, any R16 RRM relaxation criterion and any R17 RRM relaxation criterion for idle/inactive mode can be configured in a same cell at a same time, as independent criteria (i.e., without requiring a UE to fulfil both the R16 and the R17 criteria in order to relax its RRM measurements).

- If combined with a not-at-cell-edge criterion (i.e. for Rel-17 stationary & Rel-17 not-at-cell-edge), the R17 stationary criterion can only be combined with the R17 not-at-cell-edge criterion, not with the R16 one.

In this way, RAN2 kindly request RAN4 to consider support cases#8 and case #9 in the previous LS [R2-2204487/R4-2207109]. But it is up to RAN4 to make the final decision on whether support case#8 and case#9, for example, considering other reasons from RAN4 (if any).

Besides, RAN2 also discussed the relaxation criteria, and achieved the following agreements:

Regarding the relaxation criteria:

- RAN2 assume to change the “Srxlev” for stationary criterion to “SS-RSRP” in RRC_CONNECTED, pending confirmation by RAN4. Please find the below TP (which has been agreed and will be implemented in TS 38.331) for your reference:

#### 5.7.4.4 Relaxed measurement criterion for a stationary UE The relaxed measurement criterion for a stationary UE is met when: - (SrxlevSS-RSRP RefStationaryConnected – SS-RSRPSrxlev) < SSearchDeltaP-StationaryConnected, Where: - Srxlev SS-RSRP = current Srxlev value of the PCell current L3 RSRP measurement of the PCell based on SSB (dB). - Srxlev SS-RSRP RefStationaryConnected = reference SS-RSRP Srxlev value of the PCell cell (dB), set as follows: - At the end of RRC reconfiguration procedure as specified in 5.3.5.3, when rrm-MeasRelaxationReportingConfig is included in the RRCReconfiguration message; or - If (Srxlev SS-RSRP–Srxlev SS-RSRP RefStationaryConnected) > 0; or - If the relaxed measurement criterion has not been met for TSearchDeltaP-StationaryConnected: - The UE shall set the value of Srxlev SS-RSRP RefStationaryConnected to the current Srxlev SS-RSRP value of the serving cell.

RAN2 would like to confirm with RAN4 whether it is reasonable, or whether there is any concern from RAN4.

# 2. Actions:

# To RAN WG4:

# RAN2 kindly request RAN4 to take the above information into account during the following work, and provide feedback, if any.

# 3. Date of Next RAN2 Meetings:

TSG-RAN WG2 #119 			August 15 – 26 2022					Electronic Meeting

TSG-RAN WG2 #119bis		October 10 – 19 2022				Electronic Meeting