3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211569

# Electronic Meeting, Aug 15 – Aug 26, 2022

Title: 	Discussion on A-MPR requirements for band n30

Agenda Item:	5.2.4.2

Source: 	MediaTek Inc.

Document for:	Approval

Introduction

In RAN-Plenary#96 meeting, new additional SEM requirements for “NS_21” was approved [1]. However, there is still open discussion about the A-MPR values associated with NS_21 requirements. The evaluation of A-MPR values for meeting NS_21 requirements is provided here.

Discussion

In RAN-Plenary#96 meeting, new additional SEM requirements for “NS_21” was approved for Canadian WCS according to the agreed WF R4-2119840 and according to the ISED-Canada clarification of the RSS-195 regulation in RP-221047.

The update of requirements associated with NS_21 is show below.

Table 6.5.2.3.y-1: Additional requirements for "NS_21" ΔfOOB MHz; Channel bandwidth (MHz) / Spectrum emission limit (dBm); Measurement bandwidth. 5; 10. 0-1; -13; -13; 1 MHz. 1-6; -13; -13; 1 MHz. 6-10; -25; -13; 1 MHz. 10-15; -25; 1 MHz. When "NS_21" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.y-1.

The A-MPR for NS_21 is listed below for further verification.

Table 6.2.3.14-2: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 0.36MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS | Region A1b RBstart ≤ 0.36MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A2 LCRB > 2.52MHz/12/SCS | Region A3b RBend ≥ 3.96MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A3a RBend ≥ 3.96MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 5 | DFT-s-OFDM | PI/2 BPSK | ≤ [4.0] | ≤ [2.0] | ≤ [1.5] | [2.0] | [4.0] |
|  |  | QPSK | ≤ [4.5] | ≤ [2.5] | ≤ [2.0] | ≤ [2.5] | ≤ [4.5] |
|  |  | 16 QAM | ≤ [4.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [4.5] |
|  |  | 64 QAM | ≤ [4.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [4.5] |
|  |  | 256 QAM | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] |
|  | CP-OFDM | QPSK | ≤ [4.5] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.5] |
|  |  | 16 QAM | ≤ [4.5] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.5] |
|  |  | 64 QAM | ≤ [4.5] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.5] |
|  |  | 256 QAM | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] |

Firstly, ACLR of -30dBc as the initial configuration is applied. Secondly, new A-MPR values for cases in Table 6.2.3.14-2 are further applied for checking whether new n30 SEM requirement in Table 6.5.2.3.y-1 is passed or failed. Based on the evaluation, the new n30 SEM requirement with new A-MPR values in Table 6.2.3.14-2 is met. Therefore, the square brackets in Table 6.2.3.14-2 can be removed.

Proposal 1: Based on the evaluation, the new n30 SEM requirement with applying new A-MPR values in Table 6.2.3.14-2 can be met. Therefore, the square brackets in Table 6.2.3.14-2 can be removed as follows.

Table 6.2.3.14-2: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 0.36MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS | Region A1b RBstart ≤ 0.36MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A2 LCRB > 2.52MHz/12/SCS | Region A3b RBend ≥ 3.96MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A3a RBend ≥ 3.96MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 5 | DFT-s-OFDM | PI/2 BPSK | ≤ 4.0 | ≤ 2.0 | ≤ 1.5 | 2.0 | 4.0 |
|  |  | QPSK | ≤ 4.5 | ≤ 2.5 | ≤ 2.0 | ≤ 2.5 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
|  | CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |

Conclusion

In the contribution, regarding new SEM for “NS_21” with associated new A-MPR values, the evaluation is provided.

Proposal 1: Based on the evaluation, the new n30 SEM requirement with applying new A-MPR values in Table 6.2.3.14-2 can be met. Therefore, the square brackets in Table 6.2.3.14-2 can be removed as follows.

Table 6.2.3.14-2: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 0.36MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS | Region A1b RBstart ≤ 0.36MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A2 LCRB > 2.52MHz/12/SCS | Region A3b RBend ≥ 3.96MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A3a RBend ≥ 3.96MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 5 | DFT-s-OFDM | PI/2 BPSK | ≤ 4.0 | ≤ 2.0 | ≤ 1.5 | 2.0 | 4.0 |
|  |  | QPSK | ≤ 4.5 | ≤ 2.5 | ≤ 2.0 | ≤ 2.5 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
|  | CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |

- References

[1] RP-221868, “CR to 38.101-1 on corrections of NS_21 requirements”, Apple, Telus, MediaTek Inc.