3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211599

# Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2420 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Power Saving Multiple RS Handling Clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc,. MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 08/10/2022 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The agreement from WF R4-2210613 #### Issue 1-1: Clarification on multiple RLM-RS/BFD-RS - For entering condition, - Option 1: good serving cell quality criterion is fulfilled when the radio link quality is better than the threshold (Qin + X dB) for at least one resource in the set of resources for RLM/BFD. - For exiting condition, - Option 1: good serving cell quality criterion is not fulfilled when the radio link quality is worse than the threshold (Qin + X dB) for all resource in the set of resources for RLM/BFD. Agreement: Option 1 - X with candidate values {2,4,6, 8} dB can be configured by NW with default value as 0 dB. The current spec is missing the “multiple RLM-RS/BFD-RS” part of the agreement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clairfy the captured agreement is applicable when multiple resources are configured in the set |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicable condition is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1, 8.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

#### 8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria

<Paragraphs without change are omitted>

The UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met:

-	The UE sends out-of sync indications to the higher layers,

-	The timer T310 is running.

-	No DRX is used

-	The good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] is not fulfilled for all resources in the set of resources for radio link monitoring when multiple resources are configured in the set.

<End of Change #1>

<Start of Change #2>

#### 8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria

<Paragraphs without change are omitted>

The UE is no longer allowed to relax BFD measurements and apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

-	The timer beamFailureDetectionTimer is running.

-	No DRX is used

-	The good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] is not fulfilled for all resources in the set of resources for beam failure detection when multiple resources are configured in the set.

<End of Change #2>