3GPP TSG-RAN WG4 Meeting #104-e	R4-2211611

# Electronic Meeting, 15th - 26th August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.133: Corrections to NR RSTD requirements and test cases (Rel 16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Change 1: The measurement accuracy requirements for RSTD in Table 10.1.23.2-1 contain square brackets and an undefined parameter ∆. This status has not changed for several meetings in RAN4 and is preventing the respective test cases to get completed in RAN5. Change 2: Test case A.6.6.12.1 and A.6.7.13.1 have a few typos. In addition SSB_RP in not defined, which is necessary for RAN5 to define and complete the test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: In Table 10.1.23.2-1 removed square brackets and the undefined parameter ∆ (Note 7 related to ∆ voided) Change 2: In test cases A.6.6.12, A.6.7.13.1, added SSB_RP values and corrected several typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RSTD test cases A.6.6.12, A.6.7.13.1 cannot be completed in RAN5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.23, A.6.6.12, A.6.7.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 10.1.23	RSTD Measurements

#### 10.1.23.1	Introduction

The requirements in Clause 10.1.23 shall apply, provided the UE has received nr-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report one or more DL RSTD measurements defined in TS 38.215 [4].

#### 10.1.23.2	Measurement Accuracy Requirements

The accuracy requirements for RSTD measurement shall be within ±(X+Y) Tc.

X is defined in Table 10.1.23.2-1 for AWGN channel and Table 10.1.23.2-3 for fading channel for FR1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

X is defined in Table 10.1.23.2-2 for AWGN channel and Table 10.1.23.2-4 for fading channel for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

Note: The requriements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

When UE measures RSTD on PRS resources belonging to different PFLs, then the RSTD accuracy is defined as the accuracy corresponding to the largest accuracy value among different PFLs.

When UE measures RSTD on PRS resources belonging to same PFL, Y=32 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 160 ms.

When UE measures RSTD on PRS resources belonging different PFLs, Y=[256] Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than [1280] ms.

[Editor notes: The margins for measurements on different PFLs shall be considered in the group delay margin]

Editor’s Note: FFS whether and how to form the accuracy numbers considering enhanced requirements in future releases, e.g. capturing margin values in separate tables.

Table 10.1.23.2-1: RSTD absolute accuracy in FR1 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| 252 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| 170 |  |  | ≥ 52 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 78 |  |  | ≥ 104 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 147 |  | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -117 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 | -50 |
| 84 |  |  | ≥ 48 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 40 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 86 |  | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -114 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -113 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 | -50 |
| 40 |  |  | ≥ 64 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 22 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Void |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### A.6.6.12	RSTD measurements

#### A. 6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA

##### A. 6.6.12.1.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in Table A.6.6.12.1.1-1.

Table A.6.6.12.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The general test parameters are listed in Table A.6.6.12.1.1-2, and cell specific test parameters are listed in Table A.6.6.12.1.1-3.

Table A.6.6.12.1.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355[34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and secondplaces in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.1 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 0 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.1.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 2 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 3 | dBm/ 38.16MHz | -63.20 | -63.96 | -63.96 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.1.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| SSB | Config 1~3 | dB | 10 | 3 | 3 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -69.59 | -69.93 | -69.93 |
|  | Config 2 | dBm/ 9.36MHz | -69.59 | -69.93 | -69.93 |
|  | Config 3 | dBm/ 38.16MHz | -63.72 | -63.89 | -63.89 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -95 | -95 |
|  | Config 2 | dBm/SCS | -88 | -95 | -95 |
|  | Config 3 | dBm/SCS | -85 | -92 | -92 |
| PRS |  | dB | -6 | 13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

##### A.6.6.12.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 9.9.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.2.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049

# < Unchanged sections omitted >

#### A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer

##### A.6.7.13.1.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.7.13.1.1-1.

Table A.6.7.13.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

Table A.6.7.13.1.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  |  | Cell 2 | Cell 1 |  | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 |  |  | Freq1 | freq1 |  | Freq1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  | 2 |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  | 3 |  | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode | 1 |  | FDD |  |  |  | FDD |  |  |
|  | 2 |  | TDD |  |  |  | TDD |  |  |
|  | 3 |  | TDD |  |  |  | TDD |  |  |
| TDD configuration | 1 |  | N/A |  |  |  | N/A |  |  |
|  | 2 |  | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  | 3 |  | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |  |  | - | SR.1.1 FDD |  | - |
|  | 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  | 3 |  | SR.2.1 FDD |  |  |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |  |  | - | CR.1.1 FDD |  | - |
|  | 2 |  | CR.1.1 TDD |  |  | - | CR.1.1 TDD |  | - |
|  | 3 |  | CR.2.1 FDD |  |  | - | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |  |  | - | CCR.1.1 FDD |  | - |
|  | 2 |  | CCR.1.1 TDD |  |  | - | CCR.1.1 TDD |  | - |
|  | 3 |  | CCR.2.1 TDD |  |  | - | CCR.2.1 TDD |  | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  | 2 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  | 3 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  |  |
|  | 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  | 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  |  | - | 3 |  |
|  | 2,3 |  | - | 3 |  |  | - | 3 |  |
| SMTC configuration | 1 |  | SMTC.2 |  |  |  | SMTC.2 |  |  |
|  | 2,3 |  | SMTC.1 |  |  |  | SMTC.1 |  |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  |  |  | PRS.1.2 FR1 |  |  |
|  | 2 |  | PRS.1.1 FR1 |  |  |  | PRS.1.2 FR1 |  |  |
|  | 3 |  | PRS.2.1 FR1 |  |  |  | PRS.2.2 FR1 |  |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ |  |  | ‘10’ | ‘01’ |  |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  |  | N/A | 3 |  |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  |  | N/A | 5 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  |  | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  |  | -98 |  |  |
|  | 3 |  | -95 |  |  |  | -95 |  |  |
| PRS | 1~3 | dB | -6 |  |  | -13 | -6 |  | -13 |
| PRPNote3 | 1,2 | dBm/SCS | -104 |  |  | -111 | -104 |  | -111 |
|  | 3 |  | -101 |  |  | -108 | -101 |  | -108 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -69.07 |  |  | -69.83 | -69.07 |  | -69.83 |
|  | 3 | dBm/ 38.16MHz | -62.98 |  |  | -63.74 | -62.98 |  | -63.74 |
| PRS | 1~3 | dB | -6 |  |  | -13 | -6 |  | -13 |
| SSB | 1~3 | dB | -6 |  |  | -13 | -6 |  | -13 |
| Propagation condition | 1~3 | - | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

##### A.6.7.13.1.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

< End of changes >