3GPP TSG-RAN WG4 Meeting # 104-e	R4- 2211620

# Electronic Meeting,  August 15th – August 26th, 2022

Agenda Item:	11.10.3

Source: 		Qualcomm Incorporated

Title: 			Initial view for RRM core requirements for measurements without gap

Document for:	Discussion

# 1.	Introduction

The RRM requirements for measurement without gaps is one of WI under R18 further enhanced measurement gap [1]. In this paper, we would like to share our initial view.

# 2. 	Discussion

  - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4]

    - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed

    - Define related requirements, such as CSSF, measurement period, scheduling restriction etc.

  - Inter-RAT measurements without gaps [RAN4]

    - Inter-RAT NR measurements

    - Inter-RAT LTE measurement

R17 introduced reporting capability for NR and EUTRA via NCSG which is nr-NeedForGapNCSG-reporting and eutra-NeedForGapNCSG-reporting. Since reporting IE NeedForGapNCSG-InfoEUTRA covers no-ncsg-no-gap support, R17 NCSG reporting framework already cover for inter-RAT NR to LTE measurement without gap.

In last RAN2 meeting [2], NCSG reporting capability for NR and LTE has been updated and NCSG reporting capability and supporting NCSG feature were decoupled.

Observation: R17 NCSG reporting capability and supporting NCSG feature are decoupled.

Observation: For the NR UE who can measure inter-RAT LTE without gap, the NR UE supporting eutra-NeedForGapNCSG reporting capability can signal no-ncsg-no-gap via NeedForGapNCSG-infoEUTRA.

Observation: R17 NCSG signalling covers R16 needforgap signalling and R17 NCSG framework has more complete signaling and requirements than that of R16 needforgap framework.

Proposal: No additional signalling for inter-RAT LTE measurement is required. R17 NCSG reporting framework is sufficient for inter-RAT NR to LTE measurement.  NeedforNCSGgap-infoNR is used instead of needforgap-infoNR.

Proposal 2: No additional interruption is required for SSB-based inter-f and intra-f measurements without gap. If interruption required, R17 NCSG is applicable.

# 3. Conclusions

Observation: R17 NCSG reporting capability and supporting NCSG feature are decoupled.

Observation: For the NR UE who can measure inter-RAT LTE without gap, the NR UE supporting eutra-NeedForGapNCSG reporting capability can signal no-ncsg-no-gap via NeedForGapNCSG-infoEUTRA.

Observation: R17 NCSG signalling covers R16 needforgap signalling and R17 NCSG framework has more complete signaling and requirements than that of R16 needforgap framework.

Proposal: No additional signalling for inter-RAT LTE measurement is required. R17 NCSG reporting framework is sufficient for inter-RAT NR to LTE measurement.  NeedforNCSGgap-infoNR is used instead of needforgap-infoNR.

Proposal 2: No additional interruption is required for SSB-based inter-f and intra-f measurements without gap. If interruption required, R17 NCSG is applicable.

# 4.    References

# [1] R4-221018, Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps

, MediaTek, Intel, RAN Plenary #95-e

[2] R2-2206642, UE capability bit to support 2 per-UE gap only for UE concurrent gap, Intel, RAN2 #118-e