3GPP TSG-RAN WG4 Meeting #104-e	R4-2211636

# Electronic Meeting, August 15 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TC2-2 for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-7-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test case should be introduced for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing following TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for PUCCH SCell activation and deactivation delay requirements will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections A.4.5.3.x2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

A.4.5.3.x2	PUCCH SCell Activation and deactivation of FR1 unknown SCell

A.4.5.3.x2.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation are done within the required time period defined in clause 8.3.12 for EN-DC mode, when PUCCH is configured for a being activated NR FR1 SCell. The SCell is unknown by the UE at the time of activation. Supported test configurations are shown in table A.4.5.3.x2.1-1.

The E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. The general test parameters and NR cell specific test parameters are given in Table A.4.5.3.x2.1-2 and A.4.5.3.x2.1-3 below. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is E-UTRAN PCell, Cell2 is an activated NR PSCell, and Cell3 is the PUCCH SCell being activated and deactivated. The test consists of three successive time periods with duration of T1, T2 and T3, respectively. The UE shall be continuously scheduled in Cell1 (PCell) and cell2 (PSCell) throughout the test.

Before the test starts, the UE is connected to the PCell (Cell 1) on E-UTRA radio channel 1 (PCC) and PSCell (Cell2) on NR radio channel 1 (SCC1), but is not aware of SCell (Cell3) on NR radio channel 2 (SCC2). The UE is only monitoring the PCC and SCC1.

The test case includes two sub test cases as defined in Table A.4.5.3.x2.1-2a. The Cell2 (PSCell) and Cell3 (PUCCH SCell) belong to same Timing Adcance Group (TAG) in sub1 test case, and belong to different TAG in sub2 test case. The UE has valid TA in sub1 test case and does not have valid TA in sub2 test case for the PUCCH SCell activation.

At the beginning of T1, the UE receives an RRC message by which the SCell (Cell3) gets configured with PUCCH on NR radio channel 2 (SCC2). The UE now starts monitoring the SCC2 also. During T1, test equipment sends a MAC message for activation of the SCell. The MAC message for the activation is received at the UE antenna connector at slot # denoted m, which is defined as the start of time period T2.

During T2, the test equipment should send a PDCCH order to the UE to initiate RA procedure on the PUCCH SCell at slot (m+ THARQ+Tactivation_timeNR slot length) in sub2 test case. The test equipment verifies the activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received from the PUCCH SCell.A MAC message for deactivation of the SCell is received at the UE antenna connector at slot # denoted n, which is defined as the start of time period T3.

During T3, the test equipment verifies the deactivation time by counting the slots from the time when the PUCCH SCell deactivation command is sent until CQI reporting from the PUCCH SCell is discontinued.

Time period T3 starts when a MAC message for deactivation of PUCCH SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the PUCCH SCell in a slot n+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PSCell during the activation and the deactivation of PUCCH SCell, respectively.

Table A.4.5.3.x2.1-1: PUCCH SCell Activation and deactivation test configurations in FR1

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, PSCell FDD duplex mode, PUCCH SCell TDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, PSCell TDD duplex mode, PUCCH SCell TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, PSCell FDD duplex mode, PUCCH SCell TDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, PSCell TDD duplex mode, PUCCH SCell TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.4.5.3.x2.1-2: General test parameters for PUCCH SCell activation and deactivation in EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRA RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRA RF channel number 1. |
| Active PSCell |  | Cell2 | Configured activated PSCell on NR RF channel number 1. |
| Configured PUCCH SCell |  | Cell3 | Configured PUCCH SCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to Cell1, Cell2 and Cell3. |
| DRX |  | OFF | Continuous monitoring of primary cell |
| TimeAlignmentTimer for sTAG | ms | 5120 | Assuring UE has not valid TA in sub2 test case. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| T1 | S | 7 | During this time the PUCCH SCell shall be configured and detected. |
| T2 | S | 1 | During this time the UE shall activate the PUCCH SCell. |
| T3 | S | 1 | During this time the UE shall deactivate the PUCCH SCell. |

Table A.4.5.3.x2.1-2a: Configuration for sub test case

| Parameter | Unit | Sub1 test case | Sub2 test case |
| --- | --- | --- | --- |
| Cell3 timing offset to Cell2 | µs | 0 | 3 |
| Timing advance group | - | Cell2 and Cell3 belong to same TAG | Cell2 and Cell3 belong to different TAG, Cell2 belongs to pTAG, and Cell3 belongs to sTAG. |

Table A.4.5.3.x2.1-3: NR Cell specific test parameters for PUCCH SCell activation and deactivation in EN-DC

| Parameter |  | Unit | Cell2 |  |  | Cell3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Frequency Range |  |  | FR1 |  |  | FR1 |  |  |
| Duplex mode | Config 1,4 |  | FDD |  |  | TDD |  |  |
|  | Config 2,3,5,6 |  | TDD |  |  | TDD |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  | TDDConfig.1.1 |  |  |
|  | Config 2,5 |  | TDDConf.1.1 |  |  | TDDConfig.1.1 |  |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |  | TDDConfig.2.1 |  |  |
| BWchannel | Config 1,2,4,5 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  | Config 3,6 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| DL Initial BWP configuration | Config 1-6 |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration | Config 1-6 |  | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
| UL Initial BWP configuration | Config 1-6 |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| UL dedicated BWP configuration | Config 1-6 |  | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |  | SR.1.1 TDD |  |  |
|  | Config 2,5 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  | Config 3,6 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |  | CR.1.1 TDD |  |  |
|  | Config 2,5 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  | Config 3,6 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |  |  | CCR.1.1 TDD |  |  |
|  | Config 2,5 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  | Config 3,6 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  |  | TRS.1.1 TDD |  |  |
|  | Config 2,5 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  | Config 3,6 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  | SMTC.1 |  |  |
| SSB configuration | Config 1,2,4,5 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  | Config 3,6 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5 | kHz | 15 kHz |  |  | 15 kHz |  |  |
|  | Config 3,6 |  | 30 kHz |  |  | 30 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4,5 | kHz | 15 kHz |  |  | 15 kHz |  |  |
|  | Config 3,6 |  | 30 kHz |  |  | 30 kHz |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104 |  |  | -104 |  |  |
| Note2 | Config 1,2,4,5 | dBm/SCS | -104 |  |  | -104 |  |  |
|  | Config 3,6 |  | -101 |  |  | -101 |  |  |
| SS-RSRP Note3 | Config 1,2,4,5 | dBm/SCS | -87 |  |  | -infinite | -87 |  |
|  | Config 3,6 |  | -84 |  |  | -infinite | -84 |  |
|  |  | dB | 17 |  |  | -infinite | 17 |  |
|  |  | dB | 17 |  |  | -infinite | 17 |  |
| Io Note3 | Config 1,2,4,5 | dBm/9.36MHz | -58.96 |  |  | N/A | -58.96 |  |
|  | Config 3,6 | dBm/38.16MHz | -52.86 |  |  | N/A | -52.86 |  |
| Time offset to Cell1 Note 4 |  | s | 3 |  |  | -3 |  |  |
| Propagation condition |  | - | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |  |

A.4.5.3.x2.2	Test Requirements

In sub1 test case, the UE shall send the first CSI report for SCell in the first available uplink resource after slot (m+k). UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.  Whether CSI report in slot (m+k) was interrupted is checked by monitoring ACK/NACK sent in PCell in slot (m+k).

The UE shall start sending CSI report for the SCell with non-zero CQI index on the PUCCH SCell no later than m+THARQ+Tactivation_time+Tmeas+TCSI_ReportingNR slot length from begin of T2. Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms = 85ms, and Tmeas = 5*Trs = 100ms, as defined in clause 8.3.

In sub2 test case, the UE shall start sending CSI report for the SCell with non-zero CQI index on the PUCCH SCell no later than m+ THARQ+Tdelay_PUCCH_SCellNR slot length from begin of T2.

Tdelay_PUCCH_SCell = Tactivation_time + max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3), Tmeas) + TCSI_reporting_after

Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + 2* Trs + 5ms = 85ms, and Tmeas = 5*Trs = 100ms, as defined in clause 8.3.

The UE shall stop sending CSI reports for SCell at latest in a slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T2, interruption of PSCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length to  m+1+THARQ+3ms+TXNR slot length+Ninterruption, and interruption of E-UTRA PCell during SCell activation shall not happen outside the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, as defined in clause 8.3.

During T3, interruption of PSCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3, and interruption of E-UTRA PCell during SCell deactivation shall not happen outside the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length.

The interruption of PSCell and PCell shall not be more than the values specified for EN-DC in Clause 8.2.1.2.4.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in  a slot as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.