3GPP TSG-RAN WG4 Meeting #104-e                                                                                      R4-2211646

Electronic Meeting, August 15 – August 26, 2022

Source: 	CATT

Title: 	Discussion on the BS transmitter conformance testing for 52.6-71GHz

Agenda item:	9.14.5.2

Document for:	Approval

- Introduction

In RAN4#103-e, a WF [1] on BS RF conformance testing for FR2-2 was approved. Some the remaining issues in the WF need to be further discussed

In this contribution, we provide our views on the remaining issues for transmitter test.

- Discussion

Frequency range for TX spurious emission

As per the WF [1], the remaining issues concerning frequency range for TX spurious emission are shown as below:

- Sub-topic#3-4 Measurement system frequency capabilities and OOB blocking related / spurious emissions related frequency parameters Agreement: Companies are encouraged to do further study for at least - Upper frequency limit, with and without mixer solutions - Capability to generate OOB blocking interferer signal power - Increasing lower limit of measurement above 30 MHz

For FR2-1, the spurious emission defined in table 6.7.5.2.5.2.3-1 and step frequencies defined in Table 6.7.5.2.5.2.3-2 in TS 38.141-2 are copied in the following.

Table 6.7.5.2.5.2.3-2: Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [26], annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in table 6.7.1-1. |  |  |  |  |  |  |

Table 6.7.5.2.5.2.3-1: BS radiated Tx spurious emission limits in FR2 (Category B)

| Frequency range (Note 4) | Test limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 18 GHz | -30 dBm | 1 MHz | Note 1 |
| 18 GHz Fstep,1 | -20 dBm | 10 MHz | Note 2 |
| Fstep,1 Fstep,2 | -15 dBm | 10 MHz | Note 2 |
| Fstep,2 Fstep,3 | -10 dBm | 10 MHz | Note 2 |
| Fstep,4 Fstep,5 | -10 dBm | 10 MHz | Note 2 |
| Fstep,5 Fstep,6 | -15 dBm | 10 MHz | Note 2 |
| Fstep,6 min(2nd harmonic of the upper frequency edge of the DL operating band in GHz; 60 GHz) | -20 dBm | 10 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [5], s4.1 NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [26], annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 4: The step frequencies Fstep,X are defined in table 6.7.5.2.5.2.3-2. |  |  |  |

As shown in above table, for FR2-1, 60GHz is defined as max limit frequency of frequency range (Fstep,6    min(2nd harmonic of the upper frequency edge of the DL operating band in GHz; 60 GHz) ).  60GHz is slightly larger than 59.1GHz maximum Fstep,6 of all FR2-1 band(n257, n258, n259).  At same time, 60GHz is feasible to be measured.

For FR2-2, the max limit frequency point should be slightly higher than maximum Fstep,6 of all FR2-2 band. At same time, max limit frequency point is also feasible to be measured. The Fstep,6 of n263 is 127GHz. So we can choose a frequency slightly larger than Fstep,6 (127GHz) as max limit frequency point, e.g., 130GHz. We think the 130GHz is feasible to be measured [2].

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |

Proposal 1: Choose a frequency slightly higher than Fstep,6 (127GHz) as max limit frequency point, e.g., 130GHz.

- Conclusion

This contribution provides analysis on BS transmitter conformance testing for 52.6-71GHz. The following proposals are concluded as follows:

Proposal 1: Choose a frequency slightly larger than Fstep,6 (127GHz) as max limit frequency point, e.g., 130GHz.

- References

- R4-2210638, WF on BS RF conformance testing for FR2-2, ZTE, RAN4#103-e

- https://fccid.io/ZNFV600VM/Test-Report/Part-30-mmWave-Test-Report-2-4640968.pdf