3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211751

# Electronic Meeting, Aug 15 – Aug 26, 2022

Title:	Discussion on UE RF requirements for IoT NTN

Agenda Item:	12.5.4

Source:	MediaTek Inc.

Document for:	Approval

- Introduction

This document analyses the potential UE Tx and Rx RF requirements for NB-IoT/eMTC via NTN.

- RF requirements impact compared to TN

2.1	General analysis and view for each requirement

The table below provides analysis on the UE RF requirements for IoT NTN compared to requirements applicable for existing NB-IoT and Cat-M1 in TS 36.101. For each requirement, we indicate whether it is band-specific and whether we propose to reuse the existing TS 36.101 requirements and provide comments where some changes may be required/considered compared to TS 36.101. Considering that existing IoT eco system with massive IoT devices, for the sake of accelerating the market deployment of IoT NTN, to reuse TN requirements with slight modification is feasible.

Table 2.1-1

| Requirement name | Band-specific? | Reuse 36.101 requirement? | Comments |
| --- | --- | --- | --- |
| Clause 6: Transmitter Characteristics |  |  |  |
| 6.1: General | No | Yes | Generic requirements from TN should apply. |
| 6.2.1: Tx power | No | Yes - subset | We propose to define 23dBm power class- for both categories. |
| 6.2.2: MPR | No | TBD | Depends on outcome of SEM/ACLR discussion |
| 6.2.3: A-MPR | Yes | TBD | Depends on outcome of SEM/ACLR discussion and spurious emission for UE coexistence. |
| 6.2.4: Configured Tx power | No | Yes | Expect the same as TN for both categories |
| 6.3: Output Power Dynamics | No | Yes – but see comment and section 2.2 | NTN Cat-M1: Expect the same minimum output power and TX off power as TN Clarification needed on whether sTTI requirements apply for cat-M1 UE. |
|  |  |  | NTN NB1/2: Expect the same minimum output power and TX off power as TN NB1/2 |
| 6.4: Transmit signal quality |  |  |  |
| 6.4.1: Frequency error | Band-range specific | See comments and section 2.2 | Pre-compensation should apply here in the same way as for following NR NTN definition for both categories, but with existing TN values. |
| 6.4.2: Transmit modulation quality | No | Yes for Cat-M1(see comments); Yes for NB | NTN Cat-M1: Clarification needed on modulation orders to support for Cat-M1 UE, but same EVM requirements to apply. |
|  |  |  | NTN NB1/2: Same modulation as TN (BPSK and QPSK). |
| 6.5: Output RF spectrum emissions |  |  |  |
| 6.5.1: Occupied bandwidth | No | Yes | NTN Cat-M1: Expect the same as TN |
|  |  |  | NTN NB1/2: Expect the same as TN NB1/2 |
| 6.5.2: Out of band emission |  |  |  |
| 6.5.2.1: SEM | No | Yes | NTN Cat-M1: Expect the same as TN. Reconfirm this after coexistence verification |
|  |  |  | NTN NB1/2: Expect the same as TN NB1/2 |
| 6.5.2.2: Additional SEM | Yes | N/A | Not defined for b24 or b65 today. Same applies to both categories. |
| 6.5.2.3: ACLR | No | TBD. See comment and section 2.2 | Depends on coexistence verification outcome |
| 6.5.3: Spurious emission |  |  |  |
| 6.5.3.1: Minimum requirements | No | Yes | Expect general requirements from TN to apply for both categories |
| 6.5.3.2: For UE co-existence | Yes | TBD | For both categories, it depends on considerations for defined bands and protected bands. We would assume similar approach as for NR NTN here and assume -50dBm/MHz requirement would be met. |
| 6.6: Transmit intermodulation | No | Yes | NTN Cat-M1: 1.4MHz bandwidth is not covered for TN. Missing requirements here? |
|  |  |  | NTN NB1/2: Follow 36.101 requirement |
| 7: Receiver characteristics |  |  |  |
| 7.1: General | No | Yes |  |
| 7.2: Diversity characteristics | No | Yes |  |
| 7.3: Reference sensitivity | Yes - See comment | Yes - but see comment | NTN Cat-M1 (1.4MHz): - b255: reuse b24 value of [-102.7] dBm (FDD), [-103.5] dBm (HD-FDD) - TBC - b256: –102.2 dBm (FDD) and -103 dBm (HD-FDD) - TBC |
|  |  |  | NTN NB1/2: Seems appropriate to re-use the value of all TN bands, -108.2dBm |
| 7.4: Maximum input level | No | No | NTN Cat-M1: To consider whether similar relaxation should apply as for NR |
|  |  |  | NTN NB1/2: To consider whether similar relaxation should apply as for NR |
| 7.5: ACS | No | TBD. See comment and section 2.2 | Depends on outcome of coexistence verification and maximum input level |
| 7.6: Blocking characteristics |  |  |  |
| 7.6.1: In-band | No | Yes | NTN Cat-M1: Expect the same as TN |
|  |  |  | NTN NB1/2: Expect the same as TN NB1/2 |
| 7.6.2: Out-of-band | No - but see comment | See comment and section 2.2 | For both categories: For b255, reuse 36.101 requirements. For b256, depends on the outcome of further confirmation for NR NTN in RAN4#104-e. We would expect a similar approach for IoT NTN. |
| 7.6.3: Narrow band | No | Yes | NTN Cat-M1: Expect the same as TN |
|  |  |  | NTN NB1/2: Not applicable (as for TN) |
| 7.7: Spurious response | No | Yes | NTN Cat-M1: Expect the same as TN |
|  |  |  | NTN NB1/2: Expect the same as TN NB1/2 |
| 7.8: Intermodulation | No | Yes | NTN Cat-M1: Expect the same as TN |
|  |  |  | NTN NB1/2: Expect the same as TN NB1/2 |
| 7.9: Spurious emissions | No | Yes | General requirements from TN should apply for both categories. |

2.2	Specific points that need further confirmation

Modulation schemes supported

For NB-IoT only BPSK and QPSK are supported in UL and DL for NB-IoT NTN, due to support only up to Rel-16.

For Cat-M1, higher order modulation up to 64QAM is supported in DL, and up to 16QAM in UL. However, we would like views from stakeholders on which modulation orders should be supported in DL and UL.

Frequency error

The frequency error requirements for category M1 and NB1/NB2 for >1GHz are 0.1ppm, which is the same as NR. Given that the same pre-compensation procedure used in NR has been adopted in IoT NTN, we believe that the existing requirements for NB-IoT and cat-M1 can be re-used, with the definition adapted in a similar manner as being discussed for NR NTN UE.

Short TTI applicability

Need to check this further for Cat-M1. Does not seem ruled out, but equally question usefulness due to e2e system latency.

b256 aspects

NB-IoT: For TN, the same Rx sensitivity is supported for all bands (-108.2dBm). It is therefore expected that the same would apply for NTN IoT.

Cat-M1: FD-FDD is supported, we assume that the flexible b256 filters selection should follow the n256 outcome.

ACLR/ACS and corresponding SEM

This will depend on the outcome of coexistence analysis. However, we would expect the same UE ACLR/SEM and ACS values to be reused from TN. Not possible to modify spectrum utilization if there was any issue, as protocol specs are frozen (so that should be confirmed easily).

- Conclusion

In the contribution, to propose using Table 2.1-1 as starting points for IoT NTN UE TX/RX requirements.

Observation 1: Regarding IoT NTN UE TX/RX requirements for TS 36.102, to leverage structure from TS 36.101 and TS 38.101-5 with modification in requirements can be feasible.

Observation 2:  Regarding specification drafting consideration in [1], several options about skeleton structures are proposed. In order to aid progress and gain common understanding, the option-2 example shown in Annex A is used, with titles and contents in square brackets, which provides flexibility to make any modification if necessary.

Proposal 1: To agree the proposal in the Table 2.1-1, or at least use them as the starting points for further discussions on UE TX/RX structures (e.g., contents in Annex A) and requirements for TS 36.102.

- References

[1] R4-2211798, “New specification drafting considerations for IoT NTN”, MediaTek Inc.

Annex <A> (Example of TX/RX requirements):

<Start of text>

6	Transmitter characteristics

6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

[Unless otherwise stated, NB1 and NB2 requirements specified for an E-UTRA band shall also apply for the re-farmed NR band (e.g. if NB1/NB2 requirements are specified for E-UTRA band 256, they shall also apply for NR band n256).]

6.2	Transmit power

6.2.1	 UE maximum output power

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

[6.2.1A	UE maximum output power for category M1]

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non CA configuration and UL-MIMO unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table [6.2.1A-1]: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 255 |  |  | 23 | ±2 | [TBD] | [TBD] |
| 256 |  |  | 23 | ±2 | [TBD] | [TBD] |

[6.2.1B	UE maximum output power for category NB1 and NB2]

Category NB1 and NB2 UE Power Classes are specified in Table [6.2.1B-1] and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table  [6.2.1B-1]: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 255 | 23 | ±2 | [TBD] | [TBD] |
| 256 | 23 | ±2 | [TBD] | [TBD] |

6.2.2		UE maximum output power reduction

<Text will be added>

[6.2.2A	UE maximum output power reduction for category M1]

[For category M1 UE, Power Class 3 and 5, the requirements in clause 6.2.3E of 36.101 shall apply.]

[6.2.2B	UE maximum output power reduction for category NB1 and NB2]

[For UE category NB1 and NB2 power class 3 and 5, the allowed Maximum Power Reduction (MPR) for the maximum output power is as specified in clause 6.2.3F of 36.101.]

For the UE maximum output power modified by MPR, the power limits specified in sub-clause [6.2.4B] apply.

6.2.3 	UE additional maximum output power reduction

<Text will be added>

[6.2.3A	UE additional maximum output power reduction for category M1 UE]

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table [6.2.3E-1 of 36.101]. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table [6.2.3A-1] along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table [6.2.3A-1] [and from 6.2.3-2 to 6.2.3-15] are in addition to the allowed MPR requirements specified in subclause [6.2.2A].

Table [6.2.3A-1]: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_01 | [TBD] | [255, 256] | [TBD] | N/A |

For subPRB allocation, the allowed A-MPR values specified below in Table [6.2.3A-x] for category M1 UE in addition to the allowed MPR requirements specified in subclause [6.2.2A].

Table [6.2.3A-x]: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | [TBD] | [255, 256] | N/A |

[6.2.3B	UE additional maximum output power reduction for category NB1 and NB2 UE]

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table [6.2.3B-1] along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table [6.2.3B-1] are in addition to the allowed MPR requirements specified in subclause [6.2.2B-1].

Table [6.2.3B-1]: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | [TBD] | [255, 256] | N/A |

6.2.4		Configured transmitted power

<Text will be added>

[6.2.4B	Configured transmitted Power for category NB1 and NB2]

[For power class 3 and 5, the requirements in clause 6.2.5F of 36.101 shall apply.]

6.3	Output power dynamics

6.3.1		Minimum output power

The minimum controlled output power of the UE is defined as the broadband transmit power of the UE, i.e. the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value.

The minimum output power is defined as the mean power in one sub-frame (1ms). [The minimum output power shall not exceed the values specified in Table 6.3.2.1-1 of 36.101.]

[6.3.1B	UE Minimum output power for category NB1 and NB2]

[For category NB1 and NB2 UE, the requirements in clause 6.3.2F of 36.101 shall apply.]

6.3.2		Transmit OFF power

Transmit OFF power is defined as the mean power when the transmitter is OFF. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the UE is not considered to be OFF.

[6.3.2B	Transmit OFF power for category NB1 and NB2]

[For category NB1 and NB2 UE, the transmit OFF power requirement in clause 6.3.3F of 36.101 shall apply.]

6.3.3		ON/OFF time mask

[The General ON/OFF time mask requirements with measurement period of one subframe for Frame Structure 1 defined in clause 6.3.4.1 of 36.101 shall apply.]

[6.3.3B	ON/OFF time mask for category NB1 and NB2]

[For category NB1 and NB2 UE, the requirements for ON/OFF time mask in clause 6.3.4F of 36.101 shall apply.]

6.3.4		Power Control

<Text will be added>

[6.3.4A	Power control for category M1]

[The power control requirements specified for category M1 UEs in clause 6.3.5E of 36.101 shall apply.]

[6.3.4B	Power Control for category NB1 and NB2]

[Power control requirements in clause 6.3.5F of 36.101 shall apply for category NB1 and NB2 UEs.]

6.4	Transmit signal quality

6.4.1		Frequency error

<Text will be added>

[6.4.1A	Frequency error for UE category M1]

[For E-UTRA satellite access, the UE basic measurement interval of modulated carrier frequency is 1 UL timeslot (0.5ms). The UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 clause 23.21.2.2).]

[For category M1 FD-FDD UEs and HD-FDD UEs and for continuous uplink transmissions of duration ≤ 64 ms, the mean value of basic measurements of UE pre-compensated modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the ideally pre-compensated reference uplink carrier frequency signalled to the UE by the SAN.]

[For category M1 HD-FDD UEs and for continuous uplink transmissions of duration > 64 ms, the UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4.1A-1 observed over a period of one time slot (0.5 ms) compared to the ideally pre-compensated reference uplink carrier frequency signalled to the UE by the SAN.]

Table [6.4.1A-1]: Frequency error requirement for HD-FDD UE category M1

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

[NOTE: For the test case, the exact location of the UE is explicitly provided to the UE from the test equipment. The signal received by the UE in DL direction should include the Doppler effect.]

[In all cases, the requirement will be verified for at least two cases of which one has zero Doppler conditions.]

[6.4.1B	Frequency error for UE category NB1 and NB2]

[For E-UTRA satellite access, the UE basic measurement interval of modulated carrier frequency is 1 UL timeslot (0.5ms). The UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 clause 23.21.2.2).]

[For UE category NB1 and NB2, the UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4.1B-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency signalled to the UE by the SAN.]

Table [6.4.1B-1]: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

[NOTE: For the test case, the exact location of the UE is explicitly provided to the UE from the test equipment. The signal received by the UE in DL direction should include the Doppler effect.]

[In all cases, the requirement will be verified for at least two cases of which one has zero Doppler conditions.]

6.4.2		Transmit modulation quality

<Text will be added>

[6.4.2A	Transmit modulation quality for category M1]

[Transmit modulation quality requirements for category M1 UEs are defined in clause 6.5.2E of 36.101.]

[6.4.2B	Transmit modulation quality for Category NB1 and NB2]

[Transmit modulation quality requirements for Category NB1 and NB2 UEs are specified in clause 6.5.2F of 36.101.]

6.5	Output RF spectrum emissions

The output UE transmitter spectrum consists of the three components; the emission within the occupied bandwidth (channel bandwidth), the Out Of Band (OOB) emissions and the far out spurious emission domain.

Figure 6.5-1: Transmitter RF spectrum

6.5.1		Occupied bandwidth

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the channel bandwidth specified in Table 6.5.1-1.

Table 6.5.1-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / Channel bandwidth |
| --- | --- |
|  | 1.4 MHz |
| Channel bandwidth (MHz) | 1.4 |

[6.5.1B	Occupied bandwidth for category NB1 and NB2]

The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at the transmit antenna connector. Occupied bandwidth shall be less than the channel bandwidth of category NB1 and NB2 specified in clause [5.3B].

6.5.2		Out of band emission

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an Adjacent Channel Leakage power Ratio. [The out-of-band emission requirements are defined in clause 6.6.2 of 36.101 for applicable channel bandwidths specified in clause 5.3.]

6.5.2.1	Spectrum emission mask

[The requirements for spectrum emission mask in clause 6.6.2.1 of 36.101 shall apply.]

6.5.2.2	Additional Spectrum Emission Mask

<[not needed unless clearly justified]>

6.5.2.3	Adjacent Channel Leakage Ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. ACLR requirements for one E-UTRA carrier are specified for two scenarios for an adjacent E-UTRA and /or UTRA channel as shown in Figure 6.5.2.3-1.

Figure 6.5.2.3-1: Adjacent Channel Leakage requirements for one E-UTRA carrier

6.5.2.3.1	Minimum requirement E-UTRA

<Text will be added>

6.5.2.3.2	Minimum requirement UTRA

<Text will be added>

[6.5.2B	Out of band emission for category NB1 and NB2]

[6.5.2B.1	Spectrum emission mask]

[The requirements for spectrum emission mask of category NB1 and NB2 UE in clause 6.6.2F.1 of 36.101 shall apply.]

[6.5.2B.2	Additional Spectrum Emission Mask for Category NB1 and NB2]

<[not needed unless clearly justified]>

[6.5.2B.3	Adjacent Channel Leakage Ratio for category NB1 and NB2]

Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The assigned category NB1or NB2 channel power and adjacent channel power are measured with filters and measurement bandwidths specified in Table [6.5.2B.3-1]. If the measured adjacent channel power is greater than [TBD] dBm then the category NB1 or NB2 UE ACLR shall be higher than the value specified in Table [6.6.2B.3-1]. GSMACLR requirement is intended for protection of GSM system. UTRAACLR requirement is intended for protection of UTRA and E-UTRA systems.

Table [6.5.2B.3-1]: category NB1 and NB2 UE ACLR requirements

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | [TBD] dB | [TBD] dB |
| Adjacent channel center frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |

6.5.3		Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements inline with SM.329 [2] and E-UTRA operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5.3.1	Minimum requirements

[The minimum requirements in clause 6.6.3.1 of 36.101 shall apply for the supported channel bandwidths in clause 5.3.]

6.5.3.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified E-UTRA band, for coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5.3.2-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 255 | E-UTRA Band [TBD] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range |  |  |  |  |  |  |
| 256 | E-UTRA Band [TBD] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range |  |  |  |  |  |  |

6.5.3.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

[Note: There are no minimum requirements based on network signalled value yet.]

[6.5.3B	Spurious emission for category NB1 and NB2]

[When UE is configured for category NB1 or NB2 uplink transmissions, the requirements in subclause 6.6.3F of 36.101 shall apply.]

6.6	Transmit intermodulation

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.6.1		Minimum requirement

[Reserved – Note: Cat-M1 bandwidth is not covered for TN.]

[6.6.1B	Minimum requirement for category NB1 and NB2]

[The UE category NB1 and NB2 transmitter intermodulation minimum requirements are specified in clause 6.7.1F of 36.101.]

7	Receiver characteristic

7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective sections below.

With the exception of subclause 7.3, the requirements shall be verified with the network signalling value NS_01 configured (Table [6.2.3-1]).

All the parameters in clause 7 are defined using the UL reference measurement channels specified in Annexes A.2.2 and A.2.3 of [36.101], the DL reference measurement channels specified in Annex A.3.2 of [36.101] and using the set-up specified in Annex C.3.1 of [36.101].

7.2	Diversity characteristics

For category M1, category NB1 and category NB2 UE, the requirements in Section 7 assume that the receiver is equipped with single Rx port.

7.3	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category M1, category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

7.3.1		Minimum requirements (QPSK)

<[Reserved or Text will be added]>

[7.3.1A	Minimum requirements (QPSK) for UE category M1]

For category M1 UE, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [36.101] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in [36.101] Annex A.5.1.1/A.5.2.1) with parameters specified in Table [7.3.1A-1] and Table [7.3.1A-2].

Table [7.3.1A-1]: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| E-UTRA Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 256 | [-102.2] | FDD |
| 255 | [-102.7] | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in [36.101] subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 of [36.101] NOTE 5: For cat M1 the same reference sensitivity requirement applies for all applicable channel bandwidths (Table [5.3.1-1]) NOTE 6: The reference receive sensitivity shall be met for an uplink transmission bandwidth less than or equal to 6 RB. NOTE 7: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |

Table [7.3.1A-2]: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| E-UTRA Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 256 | [-103] | HD-FDD |
| 255 | [-103.5] | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in [36.101] subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 of [36.101] NOTE 5: For cat M1 the same reference sensitivity requirement applies for all applicable channel bandwidths (Table [5.3.1-1]) |  |  |

The reference receiver sensitivity (REFSENS) requirement specified in Table [7.3.1A-1] and Table [7.3.1A-2] shall be met for an uplink transmission bandwidth less than or equal to that specified in Table [7.3.1A-3].

NOTE:	Table [7.3.1A-3] is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the [36.101] Annex G (informative).

Table [7.3.1A-3]: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 256 | 61 | FDD and HD-FDD |
| 255 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table [5.3-1]). |  |  |

[7.3.1B	Minimum requirements for UE category NB1 and NB2]

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in [36.101] Annex A.3.2 with received signal level as specified in Table [7.3.1B.1-1]. Requirement in Table [7.3.1B.1-1] applies for any uplink configuration.

Table [7.3.1B.1-1]: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause [5.2B] | [- 108.2] |

7.4	Maximum input level

7.4.1		Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [36.101] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in [36.101] Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4.1-1.

Table 7.4.1-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | [TBD2] |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in [36.101 Table 7.3.1-2] with PCMAX_L as defined in [36.101 subclause 6.2.5]. NOTE 2: Reference measurement channel is Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex [A.5.1.1/A.5.2.1 of 36.101]. |  |  |

[7.4.1B	Minimum requirements for category NB1 and NB2]

Category NB1 and NB2 UE maximum input level requirement is [TBD] dBm. For this input level the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2 [of 36.101].

7.5	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

7.5.1		Minimum requirements

The UE shall fulfil the minimum requirement specified in Table 7.5.1-1 for all values of an adjacent channel interferer up to [TBD] dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5.1-2 and Table 7.5.1-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [36.101] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in [36.101] Annex A.5.1.1/A.5.2.1). For operating bands with an unpaired DL part (as noted in Table [5.2-1]), the requirements only apply for carriers assigned in the paired part.

Table 7.5.1-1: Adjacent channel selectivity

|  |  | Channel bandwidth |
| --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz |
| ACS | dB | [TBD] |

Table 7.5.1-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + [14] dB |
| PInterferer | dBm | REFSENS +[TBD] dB |
| BWInterferer | MHz | 1.4 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |
| [NOTE x: TBD] |  |  |

Table 7.5.1-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | [TBD] |
| PInterferer | dBm | [TBD] |
| BWInterferer | MHz | 1.4 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |
| [NOTE x: TBD] |  |  |

[7.5.1B	Minimum requirements for category NB1 and NB2]

Category NB1 and NB2 UE shall fulfil the minimum requirement specified in Table [7.5.1B-1] for all values of an adjacent channel interferer up to [TBD] dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table [7.5.1B-1] where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2 of [36.101].

Table [7.5.1B-1]: Adjacent channel selectivity parameters for category NB1 and NB2

| ACS1 test Parameters |  |  |
| --- | --- | --- |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + [14] dB |  |
| Interferer signal power (PInterferer ) / dBm | REFSENS + [TBD] dB | REFSENS + [TBD] dB |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |
| ACS2 test Parameters |  |  |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | [TBD] dBm | [TBD] dBm |
| Interferer signal power (PInterferer ) / dBm | [TBD] dBm |  |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |

7.6	Blocking characteristics

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

7.6.1		In-band blocking

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

7.6.1.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [36.101] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in [36.101] Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.1.1-1 and 7.6.1.1-2. For operating bands with an unpaired DL part (as noted in Table [5.2-1]), the requirements only apply for carriers assigned in the paired part.

Table 7.6.1.1-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | [6] |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| [NOTE x: TBD] |  |  |

Table 7.6.1.1-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | [TBD] | [TBD] |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| [255, 256] | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| [NOTE x: TBD] |  |  |  |  |

[7.6.1.1B	Minimum requirements for category NB1 and NB2]

Category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in [36.101] Annex A.3.2 with parameters specified in Table [7.6.1.1B-1].

Table [7.6.1.1B-1]: In-band blocking parameters for category NB1 and NB2

| IBB1 test Parameters |  |
| --- | --- |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + [6] dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | [TBD] dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | +7.5 MHz + 0.005 MHz and -7.5 MHz - 0.005 MHz |
| IBB2 test Parameters |  |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + [6] dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | [TBD] dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset range from category NB1 or NB2 channel edge | From +12.5 MHz to FDL_high + 15 MHz and From -12.5 MHz to FDL_low - 15 MHz |

7.6.2		Out-of-band blocking

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied.

7.6.2.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [36.101] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in [36.101] Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.2.1-1 and 7.6.2.1-2. For operating bands with an unpaired DL part (as noted in Table [5.2-1]), the requirements only apply for carriers assigned in the paired part.

Table 7.6.2.1-1: Out-of-band blocking parameters

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | [6] |
| [NOTE x: TBD] |  |  |

Table 7.6.2.1-2: Out of band blocking

| E-UTRA band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | [TBD] | [TBD] | [TBD] |
| [255, 256] | FInterferer (CW) | MHz | FDL_low –[15] to FDL_low –[60] | FDL_low –[60] to FDL_low –[85] | FDL_low –[85] to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| [NOTE x: TBD] |  |  |  |  |  |

[7.6.2.1B	Minimum requirements for category NB1 and NB2]

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [36.101] Annexes A.3.2 with parameters specified in Table [7.6.2.1B-1].

For Table [7.6.2.1B-1] in frequency range 1, 2 and 3, up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause [7.7.1B] spurious response are applicable.

Table [7.6.2.1B-1]: Out-of-band blocking parameters for category NB1 and NB2 UE

| Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- |
|  |  | Range 1 | Range 2 | Range 3 |
| Pwanted | dBm | REFSENS + [6] dB |  |  |
| Pinterferer (CW) | dBm | [TBD] | [TBD] | [TBD] |
| Finterferer range | MHz | FDL_low –[15] to FDL_low – [60] | FDL_low –[60] to FDL_low – [85] | FDL_low –[85] to 1 MHz |
|  | MHz | FDL_high + 15 to FDL_high + 60 | FDL_high + 60 to FDL_high + 85 | FDL_high + 85 to 12750 MHz |
| [NOTE x: TBD] |  |  |  |  |

7.6.3		Narrow band blocking

This requirement is measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

7.6.3.1	Minimum requirements

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [36.101] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in [36.101] Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3.1-1. For operating bands with an unpaired DL part (as noted in Table [5.2-1]), the requirements only apply for carriers assigned in the paired part.

Table 7.6.3.1-1: Narrow-band blocking

| Parameter | Unit | Channel Bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Pw | dBm | PREFSENS + channel-bandwidth specific value below |
|  |  | [TBD] |
| Puw (CW) | dBm | [TBD] |
| Fuw (offset for Df = 15 kHz) | MHz | 0.9075 |
| Fuw (offset for Df = 7.5 kHz) | MHz |  |
| [NOTE x: TBD] |  |  |

7.7	Spurious response

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in subclause 7.6.2 is not met.

7.7.1		Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [36.101] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in [36.101] Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.7.1-1 and 7.7.1-2. For operating bands with an unpaired DL part (as noted in Table [5.2-1]), the requirements only apply for carriers assigned in the paired part.

Table 7.7.1-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | [6] |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table [7.3.1A-3]. N OTE 2: Reference measurement channel is specified in [36.101] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 of [36.101]. NOTE 3: The REFSENS power level is specified in Table [7.3.1A-1]. |  |  |

Table 7.7.1-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | [TBD] |
| FInterferer | MHz | Spurious response frequencies |

[7.7.1B	Minimum requirements for UE category NB1 and NB2]

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in [36.101] Annexe A.3.2 with parameters specified in Tables [7.7.1B-1].

Table [7.7.1B-1]: Spurious response parameters for UE category NB1 and NB2

| Parameter | Unit | Level |
| --- | --- | --- |
| Psignal | dBm | REFSENS+[6] |
| PInterferer (CW) | dBm | [TBD] |
| FInterferer | MHz | Spurious response frequencies |
| Number of spurious response frequencies |  | 24 (in OOB range 1, 2, 3) |
| NOTE 1: Reference measurement channel is specified in Annex A.3.2 of [36.101]. NOTE 2: The REFSENS power level is specified in [7.3.1B.1-1]. NOTE 3: OOB range 1, 2, 3 refers to Table [7.6.2.1B-1]. |  |  |

7.8	Intermodulation characteristics

Intermodulation response rejection is a measure of the capability of the receiver to receiver a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

7.8.1		Wide band intermodulation

The wide band intermodulation requirement is defined following the same principles using modulated E-UTRA carrier and CW signal as interferer.

7.8.1.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [36.101] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in [36.101] Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.8.1.1 for the specified wanted signal mean power in the presence of two interfering signals. For operating bands with an unpaired DL part (as noted in Table [5.2-1]), the requirements only apply for carriers assigned in the paired part.

Table 7.8.1.1-1: Wide band intermodulation

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | [12] |
| PInterferer 1 (CW) | dBm | [TBD] |
| PInterferer 2 (Modulated) | dBm | [TBD] |
| BWInterferer 2 |  | 1.4 |
| FInterferer 1 (Offset) | MHz | -BW/2 –2.1 / +BW/2+ 2.1 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| [NOTE x: TBD] |  |  |

[7.8.1B	Wideband intermodulation for category NB1 and NB2]

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in [36.101] Annex A.3.2 with parameters specified in Table [7.8.1B-1] for the specified wanted signal mean power in the presence of two interfering signals.

Table [7.8.1B-1]: Wide band intermodulation for category NB1 and NB2

| Parameters for wideband intermodulation |  |
| --- | --- |
| Category NB1 or NB2 signal power | REFSENS + [12] dB |
| CW interferer signal power | [TBD] dBm |
| 1.4 MHz E-UTRA interferer signal power | [TBD] dBm |
| CW interferer offset | ± 2.2 MHz |
| 1.4 MHz E-UTRA interferer offset | ± 4.4 MHz |

7.9	Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

7.9.1		Minimum requirements

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9.1-1

Table 7.9.1-1: General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | [TBD] dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | [TBD] dBm |  |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH_RA/RB as defined in [36.101] Annex C.3.1. |  |  |  |

<end of text>